3GPP TSG-RAN2 Meeting #103	R2-1813008

# Gothenburg, Sweden, 20th Aug 2018 - 24th Aug 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 0008 | rev | 4 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on total layer2 buffer size |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2018-08-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - For NR, PDCP layer is responsible for reordering and thus reordering packets in PDCP layer should be taken into account for all bearer types. - Current text states that total L2 buffer may consider the pre-processed RLC PDUs and this may lead to NW’s under-estimation of buffer usage in the UE. - The concrete equation of total L2 buffer size of MR-DC and NR-DC (taking into both MN terminated and SN terminated into account) is missing. - Values of RLC RTT per SCS are still TBD. - It is not clear which RLC RTT is used if the NR CA composes of CCs configured with different SCS. - The value of X2/Xn delay + Queuing in MN is still FFS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following changes are done: 1. It is clarified that the total layer2 buffer is used to store PDCP reordering data in receiving side also for non-split bearer. 2. The note is added that additional L2 buffer required for preprocessing of data is not taken into account in the formula. 3. Added the equation to calculate total L2 buffer size for MR-DC and NR-DC. 4. Removed TBDs and added the RLC RTT values in table 4.1.4-1 5. It is clarified that the RLC RTT for the smallest SCS is used to calcualte buffer size for the for given band combination of NR CG. 6. Capture 55ms as value of “X2/Xn delay + Queuing in MN/SN” for EUTRA CG and 25ms as value of “X2/Xn delay + Queuing in MN/SN” for NR CG. This CR is applicable for both EN-DC and SA. Impact analysis: Impacted functionality: UE total L2 buffer size calucation Inter-operability: If the proposed change is implemented by the UE and not by the gNB, gNB may transmit PDCP PDU larger than UE can store in the buffer. If the proposed change is implemented by the eNB and not by the UE, there is no interoperability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE and NW cannot be have the same understanding of the UE L2 buffer size and NW may over-schedule resulting in packet drop in the UE due to buffer over-flow. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of change

### 4.1.4	Total layer 2 buffer size

The total layer 2 buffer size is defined as the sum of the number of bytes that the UE is capable of storing in the RLC transmission windows and RLC reception and reordering windows and also in PDCP reordering windows for all radio bearers.

The required total layer 2 buffer size in MR-DC and NR-DC is the maximum value of the calculated values based on the following equations:

- MaxULDataRate_MN * RLCRTT_MN + MaxULDataRate_SN * RLCRTT_SN + MaxDLDataRate_SN * RLCRTT_SN + MaxDLDataRate_MN * (RLCRTT_SN + X2/Xn delay + Queuing in SN)

- MaxULDataRate_MN * RLCRTT_MN + MaxULDataRate_SN * RLCRTT_SN + MaxDLDataRate_MN * RLCRTT_MN + MaxDLDataRate_SN * (RLCRTT_MN + X2/Xn delay + Queuing in MN)

Otherwise it is calculated by MaxDLDataRate * RLC RTT + MaxULDataRate * RLC RTT.

NOTE: Additional L2 buffer required for preprocessing of data is not taken into account in above formula

The required total layer 2 buffer size is determined as the maximum total layer 2 buffer size of all the calculated ones for each band combination and the applicable Feature Set combination in the supported MR-DC or NR band combinations. The RLC RTT for NR cell group corresponds to the smallest SCS numerology supported in the band combination and the applicable Feature Set combination.

wherein

X2/Xn delay + Queuing in SN = 25ms if SCG is NR, and 55ms if SCG is EUTRA

X2/Xn delay + Queuing in MN = 25ms if MCG is NR, and 55ms if MCG is EUTRA

RLC RTT for EUTRA cell group = 75ms

RLC RTT for NR cell group is defined in Table 4.1.4-1

Table 4.1.4-1: RLC RTT for NR cell group per SCS.

| SCS (KHz) | RLC RTT (ms) |
| --- | --- |
| 15KHz | 50 |
| 30KHz | 40 |
| 60KHz | 30 |
| 120KHz | 20 |

End of change