3GPP TSG-RAN WG2 #103			    R2-1813037

Gothenburg, Sweden, 20th – 24th August, 2018

Agenda item:	   11.1.2 (NR_newRAT-Core)

Source:	LG Electronics Inc.

Title: 	E2E reliability in hop-by-hop RLC ARQ

Document for:	Discussion and Decision

1.	Introduction

This document includes a text proposal for TR 38.874 based on offline discussion.

2.	Text Proposal for TR 38.874

The following changes to TR 38.874 are proposed:

********* Start of Change **********

8.2.3 	Multi-hop RLC ARQ

----

The issue of end to end reliability in hop-by-hop RLC ARQ case could be addressed by specifying, e.g., the following mechanisms:

- Modification of PDCP protocol/procedures. This solution would not be applicable to Rel-15 UEs which means that Rel-15 UE performance may be impaired.

- Rerouting of PDCP PDUs buffered on intermediate IAB-nodes in response to a route update (FFS what information needs to be exchanged between IAB nodes).

- Introducing UL status delivery (from the Donor gNB to the IAB node), whereby the IAB node can delay the sending of RLC ACKs to the UE until a confirmation of reception at the Donor gNB.

Table 8.2.3 - 1: Comparison of mechanisms for lossless delivery of UL data

|  | Modification of PDCP protocol/procedures | Rerouting of PDCP PDUs buffered on intermediate IAB nodes | Introducing UL status delivery |
| --- | --- | --- | --- |
| Supported for Rel-15 UEs | No | Yes | Yes |
| Signaling overhead | No | No | Yes - IAB donor should send UL status delivery to UE’s access IAB node. |
| Support of lossless handover | Yes | Yes, but require the followings until all in-flight packets have been delivered: - Source IAB donor should NOT release PDCP/SDAP entities of all bearer(s); - Source IAB donor should NOT release BH RLC-channel(s); - Target network node (e.g., IAB donor or gNB) needs to perform reordering and discarding operations for all PDCP SDU(s) received from both UE and source IAB donor. | Yes |

********* End of Change **********