3GPP TSG-RAN2 Meeting #103	R2-1813080

# Gothenburg, Sweden, 20th – 24th August 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | CRNum | rev | - | Current version: | 15.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the configuration of POs within a PF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-04 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The current mechanism for configuration of POs and mapping of POs to PDCCH monitoring occasions does not take into account that (depending on the number of beams) there may be configured PDCCH monitoring occasions for which there are non matching SS Block transmissions (i.e. PDCCH monitoring occasions which are “superfluous”). This may cause extended POs, resulting in increased UE energy consumption, and/or undesirable impacts on the load on the PDCCH, PDSCH and PRACH resources. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of a new parameter, firstPDCCH-MonitoringOccasionOfPO, in the PCCH-Config IE signalled in SystemInformationBlock1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | POs may not be distributed in the PF with time gaps in between, which may lead to undesirably long POs resulting in increased UE energy consumption and/or may cause undesirable load peaks on the PDCCH signalling, the PDSCH transmission resources (potentially making it difficult to execute all the PDSCH paging transmissions before the next PO), PRACH resources and other access resources in the network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS/TR 38.304 CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### –	DownlinkConfigCommonSIB

The IE DownlinConfigCommonSIB provides common downlink parameters of a cell.

DownlinkConfigCommonSIB information element

-- ASN1START

-- TAG-DOWNLINK-CONFIG-COMMON-SIB-START

DownlinkConfigCommonSIB ::=		SEQUENCE {

frequencyInfoDL					FrequencyInfoDL-SIB,

initialDownlinkBWP				BWP-DownlinkCommon,

bcch-Config 						BCCH-Config,

pcch-Config 						PCCH-Config,

...

}

BCCH-Config ::=					SEQUENCE {	modificationPeriodCoeff			ENUMERATED {n2, n4, n8, n16},

...

}

PCCH-Config ::=				SEQUENCE {

defaultPagingCycle					PagingCycle,

nAndPagingFrameOffset				CHOICE {

oneT								NULL,

halfT								INTEGER (0..1),

quarterT							INTEGER (0..3),

oneEighthT							INTEGER (0..7),

oneSixteenthT						INTEGER (0..15)

},

ns									ENUMERATED {four, two, one},

firstPDCCH-MonitoringOccasionOfPO	CHOICE {

sCS15KHZoneT																SEQUENCE (SIZE (1..4)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT													SEQUENCE (SIZE (1..4)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT									SEQUENCE (SIZE (1..4)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT				SEQUENCE (SIZE (1..4)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT	SEQUENCE (SIZE (1..4)) OF INTEGER (0..2239),

sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT					SEQUENCE (SIZE (1..4)) OF INTEGER (0..4479),

sCS120KHZoneEighthT-SCS60KHZoneSixteenthT									SEQUENCE (SIZE (1..4)) OF INTEGER (0..8959),

sCS120KHZoneSixteenthT														SEQUENCE (SIZE (1..4)) OF INTEGER (0..17919)

}	OPTIONAL,			-- Need R

...

}

-- TAG-DOWNLINK-CONFIG-COMMON-SIB-STOP

-- ASN1STOP

DownlinkConfigCommonSIB field descriptions

frequencyInfoDL-SIB Basic parameters of a downlink carrier and transmission thereon

initialDownlinkBWP The initial downlink BWP configuration for a SpCell (PCell of MCG or SCG).

bcch-Config The modification period related configuration.

pcch-Config The paging related configuration.

BCCH-Config field descriptions

modificationPeriodCoeff Actual modification period, expressed in number of radio frames= modificationPeriodCoeff * defaultPagingCycle. n16 corresponds to value 16, n32 corresponds to value 32, and so on. The BCCH modification period should be larger or equal to 40.96s.

PCCH-Config field descriptions

defaultPagingCycle Default paging cycle, used to derive ‘T’ in TS 38.304 [20]. Value rf32 corresponds to 32 radio frames, rf64 corresponds to 64 radio frames and so on.

nAndPagingFrameOffset Used to derive the number of total paging frames in T (corresponding to parameter N in TS 38.304 [20]) and paging frame offset (corresponding to parameter PF_offset in TS 38.304 [20])

ns Number of paging occasions in paging frame

firstPDCCH-MonitoringOccasionOfPO Points out the first PDCCH monitoring occasion of each PO in the PF, see TS 38.304 [20].