3GPP TSG-RAN WG2 #103	R2-1813097

Gothenburg, Sweden, 20th – 24th Aug 2018

Agenda Item:	9.14.2

Source:	Ericsson

Title:	Intergrity protection failure while T300 is running for EDT

Document for:	Discussion, Decision

# Text proposal

Next change

#### 5.3.3.3y	Integrity check failure from lower layers while T300 is running

The UE shall:

1>	upon receiving Integrity check failure indication from lower layers concerning SRB1 or SRB2 while T300 is running:

2> discard the stored UE AS context and resumeIdentity;

2> perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other', upon which the procedure ends;

Next change

#### 5.3.7.2	Initiation

The UE shall only initiate the procedure either when AS security has been activated or for a NB-IoT UE supporting RRC connection re-establishment for the Control Plane CIoT EPS optimisation. The UE initiates the procedure when one of the following conditions is met:

1>	upon detecting radio link failure, in accordance with 5.3.11; or

1>	upon handover failure, in accordance with 5.3.5.6; or

1>	upon mobility from E-UTRA failure, in accordance with 5.4.3.5; or

1>	upon integrity check failure indication from lower layers concerning SRB1 or SRB2 when T300 is not running; or

1>	upon an RRC connection reconfiguration failure, in accordance with 5.3.5.5; or

1>	upon an RRC connection reconfiguration failure, in accordance with TS38.331 [82, 5.3.5.5].