3GPP TSG-RAN WG2 #103	R2-1813221

# Gothenburg, Sweden, 20 – 24 August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | 0047 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 37.340 on the Layer 2 handling for bearer type change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-08-09 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The description for the Layer 2 handling for bearer type change is not correct in Table A-1. Necessary changes were discussed during RAN2-AH1807 in Montreal, but could not be agreed and was postponed to RAN2#103. This CR is based on R2-1810953, with a few clarifications added to the text. Related Stage-3 changes are covered in RIL [E571]. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated table and notes for the Layer 2 handling for bearer type change. Note 1 captures RLC/MAC actions on the LTE side. Note 2 captures the agreement that on NR side, when MAC reset is avoided by LCH ID change, RLC bearer needs to be released and added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The informative behaviour for the bearer type change is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A (informative) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

Annex A (informative):
Layer 2 handling for bearer type change

This subclause provides for information an overview on L2 handling for bearer type change in EN-DC, with and without security key change (from KeNB to S-KgNB and from S-KgNB to KeNB), i.e. with and without a change of the termination point.

Table A-1: L2 handling for bearer type change with and without security key change

| Bearer type change from row to col | MCG |  | Split |  | SCG |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | no key change | with key change (KeNB <-> S-KgNB) | no key change | with key change (KeNB <-> S-KgNB) | no key change | with key change (KeNB <-> S-KgNB) |
| MCG | N/A | PDCP: Re-establish MCG RLC: See Note 1 MCG MAC: See Note 1 SCG RLC: No action SCG MAC: No action | PDCP: Reconfigure MCG RLC: No action MCG MAC: No action SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: See Note 1 MCG MAC: See Note 1 SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Recovery MCG RLC: Re-est+release MCG MAC: Reconfigure SCG RLC: Establish SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: Re-est+release MCG MAC: Reconfigure SCG RLC: Establish SCG MAC: Reconfigure |
| Split | PDCP: Recovery MCG RLC: No action MCG MAC: No action SCG RLC: Release SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: See Note 1 MCG MAC: See Note 1 SCG RLC: Release SCG MAC: Reconfigure | N/A | PDCP: Re-establish MCG RLC: See Note 1 MCG MAC: See Note 1 SCG RLC: See Note 2 SCG MAC: See Note 2 | PDCP: Recovery MCG RLC: Re-est+release MCG MAC: Reconfigure SCG RLC: No action SCG MAC: No action | PDCP: Re-establish MCG RLC: Re-est+release MCG MAC: Reconfigure SCG RLC: See Note 2 SCG MAC: See Note 2 |
| SCG | PDCP: Recovery MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: Release SCG MAC: Reconfigure | PDCP: Re-establish MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: Release SCG MAC: Reconfigure | PDCP: Reconfigure MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: No action SCG MAC: No action | PDCP: Re-establish MCG RLC: Establish MCG MAC: Reconfigure SCG RLC: See Note 2 SCG MAC: See Note 2 | N/A | PDCP: Re-establish MCG RLC: No action MCG MAC: No action SCG RLC: See Note 2 SCG MAC: See note 2 |

NOTE 1: 	For MCG, the MAC/RLC behaviour depends on the solution selected by the network. It can be handover, which triggers MAC reset and RLC re-establishment. Alternatively, the logical channel identity can be changed, either via RLC bearer release and add (including RLC re-establishment), or via reconfiguration of the RLC bearer with RLC-re-establishment.

NOTE 2:	For SCG, MAC/RLC behaviour depends on the solution selected by the network. It can be reconfiguration with sync, with MAC reset and RLC re-establishment. Alternatively, the logical channel identity can be changed via RLC bearer release and add.

END OF CHANGES