3GPP TSG-RAN2#103	R2-1813296

Gothenburg, Sweden, 20th – 24th August, 2018

Title:	[Draft] LS on OSI acquisition

# Release:	Rel-15

Work Item:	NR_newRAT

Attachment: 	None

Source:	RAN2

To:	RAN1

# Contact Person:

# Name:	Keiichi Kubota

E-mail Address:	keiichik(at)qti.qualcomm.com

# 1. Overall Description:

RAN2#103 has made the following agreements on the OSI acquisition.

Agreements

1: 	38.331 should state "PDCCH containing the scheduling RNTI" instead of "DL-SCH"

2	Same relationship between monitoring occasion and SSB is defined for SI as for paging.

3: 	NR supports re-transmission for SI message within an SI window (as in LTE)

UE monitors the PDCCH monitoring occasion(s) in SI-Window for receiving SI message. RAN2 assumes that in case of non-default association (i.e. when osi-searchSpace is configured), Kth PDCCH monitoring occasion in the SI-Window corresponds to Kth transmitted SSB. In case of default association (i.e. when osi-searchSpace is not configured), PDCCH monitoring occasions for SI message reception in SI-Window are same as PDCCH monitoring occasions for RMSI where the mapping between PDCCH monitoring occasions and SSBs is defined in section 10.3 of TS 38.213. RAN2 would also remind that UE can assume that the same SI message is repeated in all beams.

RAN2 respectfully requests RAN1 to take the above agreements into account and provide feedback if any.

# 2. Actions:

RAN2 respectfully requests RAN1 to take the above agreements into account and provide feedback if any.

# 3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #103bis	8th –12th Oct 2018		Chengdu, China.

TSG-RAN WG2 Meeting #104	12th – 16th Nov 2018		Spokane, USA.