3GPP TSG-RAN WG2 AH 1807	R2-1813333

# Gothenburg, Sweden, 20th – 24th August 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR | 1185 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 2 Changes based on CN type indication for Redirection from ng-eNB to E-UTRA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_5GCN_connect-Core |  |  |  |  |  |  | Date: |  | 2018-06-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN2#103 meeting, RAN2 made following agreements for eLTE - RRC Release with Redirection message can a include target CN type (no restriction on the use case when the CN type is included) - If the selected cell support both 5GS and EPC then the CN type received from RRCRelease message is provided to NAS to the used to determine the NAS procedure to be performed. (The indication does not modify the cell selection behaviour) This CR captures the stage 2 details in the spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added stage 2 description about expected UE behaviour when UE receives CN Type in RRC Connection Release message for rediecting UE from ng-eNB to E-UTRA feequency cell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eLTE UE behaviour will not be specified when the CN type indication in the RRC Connection Release message is received. It is not aligned with SA2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 24.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of first change

## 3.2	Abbreviations

<<unnecessary changes omitted>>

N2	Reference point between the NG-RAN and the AMF

next change

## 24.5	Mobility

The procedure for the new mobility scenario intra-EUTRA inter-systerm Handover (i.e., handover between E-UTRA connected to 5GC and E-UTRA connected to EPC) is described in section 10.2.2c.

The core network is changed during the procedure, as defined in 3GPP TS 23.502 [83].

Inter-RAT handover to/from GERAN/UTRAN/CDMA2000 and cell change order to GERAN with NACC are not supported, and CS fallback described in subclause 10.2.5 is not applied except for the functionality of release with redirection to GERAN/UTRAN.

The following mobility procedures are supported:

-	RRC Connection Release with Redirection to GERAN/UTRAN/CDMA2000/EUTRAN;

-	Cell Change Order to GERAN without NACC.

When the UE is connected to E-UTRA/5GC, inter system fallback towards E-UTRAN is performed when 5GC does not support emergency services, voice services, for load balancing etc. Depending on factors such as CN interface availability, network configuration and radio conditions, the fallback procedure results in either CONNECTED state mobility (handover procedure) or IDLE state mobility (redirection) - see 3GPP TS 23.501 [82] and 3GPP TS 36.331 [16].

In the N2 signalling procedure, the AMF based on support for emergency services, voice service, any other services or for load balancing etc, may indicate the target CN type as EPC or 5GC to the ng-eNB node. When the target CN type is received by ng-eNB, the target CN type is also conveyed to the UE in RRC Connection Release Message.

The mobility in RRC_INACTIVE is described in section 10.1.9.

For E-UTRA connected to 5GC, in RRC_IDLE the UE monitors the PCCH for CN-initiated paging information, in RRC_INACTIVE the UE monitors the PCCH for RAN-initiated and CN-initiated paging information. NG-RAN and 5GC paging occasions overlap and the same paging mechanism is used in NG-RAN and in 5GC. The extended DRX (eDRX) is not used for E-UTRA connected to 5GC.

End of changes