3GPP TSG-RAN2 Meeting #103bis	R2-1813579

# Chengdu, China, 8th Oct 2018 - 12th Oct 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.323 | CR | 0250 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PDCP for eV2X (option-2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eV2X-Core |  |  |  |  |  | Date: |  |  | 2018-09-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - The first arrived packet with SN in the half-SN-space below Last_Submitted_PDCP_RX_SN may be discarded by mistake. - The packet with SN=0 may be discarded by mistake. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The initialization of the variable of Last_Submitted_PDCP_RX_SN and Next_PDCP_RX_SN is left to UE implementation. - The packet with SN=0 would be delivered to upper layer in any case. Impact analysis Impacted functionality: PDCP behavior Inter-operability: If the UE is implemented according to this CR but the network is not, there is no inter-operability problem. If the network is implemented according to this CR but the UE is not, there is no inter-operability problem. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The first arrived packet with SN in the half-SN-space below Last_Submitted_PDCP_RX_SN may be discarded by mistake. - The packet with SN=0 may be discarded by mistake. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4, 5.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Since the final version is not available, this CR is based on the draft version. |  |  |  |  |  |  |  |  |

Start of changes

5.1.4	SL Data Reception Procedures

For Sidelink reception, the UE shall follow the procedures in subclause 5.1.2.1.3 with following modifications:

-	the requirements for maintaining Next_PDCP_RX_SN and RX_HFN are not applicable;

-	perform the deciphering (if configured) as specified in subclause 5.6.1 and 5.6.2;

-	perform the header decompression (if configured) if SDU Type is set to 000, i.e. IP SDUs.

For sidelink duplication reception capable UE, if it receives a PDCP SN which is not "0", the Sidelink reception of the UE shall follow the procedures in subclause 5.1.2.1.4.1 with following modifications compared to above Sidelink reception procedure:

-	the requirements for maintaining Next_PDCP_RX_SN and RX_HFN are applicable;

-	perform the re-ordering procedure as specified in subclause 5.1.2.1.4.1.

If the UE receives a PDCP SN which is “0”, the UE shall deliver it to upper layers regardless of the re-ordering procedure.

NOTE:	If ciphering is not configured, it is up to UE implementation how to handle the first received PDCP Data PDU with SN not set to “0”.

Next change

5.6.1	SL Ciphering and Deciphering for one-to-many communication

For SLRB used for one-to-many communication, the ciphering function includes both ciphering and deciphering and is performed in PDCP as defined in [13]. The data unit that is ciphered is the data part of the PDCP PDU (see subclause 6.3.3). The ciphering function as specified in [6] is applied with KEY (PEK), COUNT (derived from PTK Identity and PDCP SN as specified in [13]), BEARER and DIRECTION (set to 0) as input. The ciphering function is configured by ProSe Function.

If ciphering is configured, the ciphering algorithm and related parameters including PGK, PGK Identity, and Group Member Identity are configured to the UE by ProSe Key Management Function. The UE shall set PTK Identity based on PGK, PGK Identity, and PDCP SN as specified in [13]. The UE shall derive PTK from PGK using PTK Identity and Group Member Identity, and derive PEK from PTK using the ciphering algorithm. The PGK Index, PTK Identity, and PDCP SN are included in the PDCP PDU header.

If ciphering is not configured, PGK Index and PTK Identity shall be set to "0" in the PDCP PDU header.

If ciphering is not configured, and sidelink duplication transmission is disabled for the SLRB, PDCP SN shall be set to "0" in the PDCP PDU header.

If ciphering is not configured, and sidelink duplication transmission is enabled for the SLRB, PDCP SN shall not be set to "0" in the PDCP PDU header.

End of changes