3GPP TSG-RAN WG2 Meeting #103bis	R2-1813670

Chengdu, China, 8-12 October 2018

Agenda Item:	11.5.1

Source: 	MediaTek Inc.

Title:  	Solutions for handling of large capabilities

Document for:	Discussion and decision

# Introduction

This document discusses some potential solutions for the management of large UE capability messages. In [1] and [2], it is suggested that the NR UE capability could reach sizes of multiple tens of kilobytes.  As noted in [5], the NR UE capability size is dominated by the rf-Parameters and featureSetCombinations, and given the potential for these fields to grow to significant sizes, it seems clear that some measures to handle large capability messages are needed.

# Discussion

## Capability indexing

The concept of using a capability “index” has been suggested, but has the concern that there is not an obvious, reliable, and standardisable choice for the index value.  The IMEI-SV could be used as an indicator of UE model, but does not necessarily guarantee exactly the same capabilities in all cases (e.g. due to PLMN specific settings).  SA2 need to discuss the potential index values as part of their study effort.  In general, when a database of UE capability indices is populated, we should consider the possibility that a UE may deviate from it and need to indicate “I have the capabilities of index Y, but with the following changes…”

To achieve this, as suggested in [4] and as currently captured in [7] (section 6.6), a “delta” approach could be taken.  We anticipate that in many cases, a UE would be able to indicate its full capability with the index; where deviations exist, they should be in a few areas such as support of specific band combinations, for which delta signalling can be defined.

Proposal 1: Consider an “index+delta” approach to signalling UE capability, in which the UE provides an index along with a list of delta-signalled capabilities for certain fields.

In the NR capability structure, the fields for which delta signalling would most likely be needed are the same ones responsible for most of the size: the supportedBandCombinationList (inside RF-Parameters), featureSets and featureSetCombinations.  It could also be discussed if delta signalling is needed for other areas, e.g. the per-band parameters in RF-Parameters.  All these fields can be addressed by defining ToRemove and ToAddMod lists in the ASN.1.  Similar delta signalling support could be added to the MRDC capability.

Proposal 2: Discuss which fields of the UE-NR-Capability and UE-MRDC-Capability would require delta signalling.  The supportedBandCombinationList, featureSets, and featureSetCombinations are proposed as a baseline set.

## Fallback to full capability request

Even in the presence of a database of capability indices, there is some possibility that a network will not recognize the index of a particular UE.  For example, the database may not have been updated for a newer UE model, or in case of a standardised set of index values, the network may be operating on an older version of the standard.  As a fallback mechanism, it should be possible for the network to request the full capability in case it cannot infer the capability from the index.

Proposal 3: The network should be able to request the full capability of the UE.

Considering the potential size of the NR capability, Proposal 3 implies that means to signal very large capability messages need to be considered.  As discussed in [6], generic compression is a valid approach, but by itself it may not be sufficient (e.g., if the capability size is on the order of multiple tens of kilobytes—as the analysis of [5] suggests is reasonable—it seems unlikely that general-purpose compression would reliably bring it down to the size of a single PDCP PDU).  Other approaches such as segmentation need to be discussed as well.

Proposal 4: Consider approaches such as compression and segmentation for handling a large UE capability message in cases where the full capability needs to be sent.

## Applicability to LTE

The analysis of [5] is specific to the NR and MRDC capabilities (which use the same structures for the band combination and feature set combination lists).  However, capability size has already been a problem in LTE as well, and this situation will only get worse in the future as more band combinations are deployed.  We therefore suggest that the solution(s) for NR/MRDC should be applied to LTE capability as well.  For the air interface, there is no obvious dependency on the core network connectivity, so the developed solution(s) should apply to both EPC and 5GC connectivity cases.

Proposal 5: The solution(s) for UE capability size management should be considered for all of NR, MRDC, and LTE, connected to either EPC or 5GC.

# Conclusion

This document promulgated the following proposals:

Proposal 1: Consider an “index+delta” approach to signalling UE capability, in which the UE provides an index along with a list of delta-signalled capabilities for certain fields.

Proposal 2: Discuss which fields of the UE-NR-Capability and UE-MRDC-Capability would require delta signalling.  The supportedBandCombinationList, featureSets, and featureSetCombinations are proposed as a baseline set.

Proposal 3: The network should be able to request the full capability of the UE.

Proposal 4: Consider approaches such as compression and segmentation for handling a large UE capability message in cases where the full capability needs to be sent.

Proposal 5: The solution(s) for UE capability size management should be considered for all of NR, MRDC, and LTE, connected to either EPC or 5GC.

# References

- RP-181146, “UE Capability ID Motivation”, MediaTek Inc., RAN#80

- R2-1810966, “Reply LS on optimisation of UE capability signalling”, LS from RAN2 to SA2, RAN2 ad hoc #1807

- RP-181459, “Study on optimisations on UE radio capability signalling – NR/E-UTRA Aspects”, MediaTek Inc. et al., RAN#80

- R2-1807822, “UE capability compression through capability ID”, MediaTek Inc., RAN2#102

- R2-1813671, “Analysis of UE capability size”, MediaTek Inc., RAN2#103bis

- R2-18xxxxx, “UE capability size reduction through generic compression”, MediaTek Inc., RAN2#103bis

- TR 23.743, “Study on optimisations on UE radio capability signalling”, 2018-09