3GPP TSG-RAN WG2 #103bis	R2-1814111

Chengdu, China, 8th October – 12th October 2018

Agenda Item:	12.2.7

Source:	Ericsson

Title:	Support of quality report in Msg3 for non-anchor access in NB-IoT

Document for:	Discussion, Decision

# Introduction

In the work item description (WID) of Rel-16 enhancements for NB-IoT, one of the objectives is to improve the multi-carrier operation as follows [1].

- Specify support of Msg3 quality reporting for non-anchor access [RAN1, RAN2]

- Specify signalling to indicate on a non-anchor carrier for paging a set of subframes which will contain NRS even when no paging NPDCCH is transmitted [RAN1, RAN2, RAN4]

In this paper we discuss the support of Msg3 quality reporting for non-anchor access. There has been already an agreement for the measurement quality reporting for anchor carrier. The framework now needs to be extended to support DL measurement reporting for non-anchor carrier.

# Discussion

In order to support the quality report in Msg3 for non-anchor access, from RAN2 perspective, the important aspect is the number of bits that would be required.

Currently 17 spare bits are available in RRCConnectionRequest-NB [2].

-- ASN1START

RRCConnectionRequest-NB ::=		SEQUENCE {

criticalExtensions					CHOICE {

rrcConnectionRequest-r13			RRCConnectionRequest-NB-r13-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCConnectionRequest-NB-r13-IEs ::=		SEQUENCE {

ue-Identity-r13							InitialUE-Identity,

establishmentCause-r13					EstablishmentCause-NB-r13,

multiToneSupport-r13					ENUMERATED {true}				OPTIONAL,

multiCarrierSupport-r13					ENUMERATED {true}				OPTIONAL,

earlyContentionResolution-r14			BOOLEAN,

cqi-NPDCCH-r14							CQI-NPDCCH-NB-r14,

spare									BIT STRING (SIZE (17))

}

-- ASN1STOP

For, anchor carrier measurement reporting in Msg3, 4 bits have been used as below.

-- ASN1START

CQI-NPDCCH-NB-r14 ::=	ENUMERATED {

noMeasurements, candidateRep-A, candidateRep-B, candidateRep-C,

candidateRep-D, candidateRep-E, candidateRep-F, candidateRep-G,

candidateRep-H, candidateRep-I, candidateRep-J, candidateRep-K,

candidateRep-L}

-- ASN1STOP

Further, a short CQI reporting is also supported in RRCConnectionReestablishmentRequest-NB as below.

-- ASN1START

CQI-NPDCCH-Short-NB-r14 ::=	ENUMERATED {

noMeasurements, candidateRep-1, candidateRep-2, candidateRep-3}

-- ASN1STOP

There is only 1 spare bit left in the RRCConnectionReestablishmentRequest-NB.

It is expected that similar measurement reporting as above would be required for non-anchor carrier and the same structure could be re-used. However, it is up to RAN1 and RAN4 to decide.

For EDT Msg3, as such there is no size restriction as the legacy Msg3, so more flexibility could be allowed for EDT case.

For EDT, more granular CQI reporting may be supported.

Wait for RAN1 and RAN4 to progress on this and decide upon the exact number of bits needed to report the measurement quality for non-anchor carrier.

# Conclusion

In section 2 we made the following observations:

Observation 1	It is expected that similar measurement reporting as above would be required for non-anchor carrier and the same structure could be re-used. However, it is up to RAN1 and RAN4 to decide.

Observation 2	For EDT, more granular CQI reporting may be supported.

Based on the discussion in section 2 we propose the following:

Proposal 1	Wait for RAN1 and RAN4 to progress on this and decide upon the exact number of bits needed to report the measurement quality for non-anchor carrier.

# References

RP-181451, New WID on Rel-16 enhancements for NB-IoT, Source Ericsson, Huawei, 3GPP TSG RAN Meeting #80, La Jolla, USA, June 11-14, 2018.

36.331, RRCv15.3.0