3GPP TSG-RAN WG2 #103bis	R2-1814113

Chengdu, China, 8th October – 12th October 2018

Agenda Item:	12.1.10

Source:	Ericsson

Title:	RSS for measurement improvements in LTE-MTC

Document for:	Discussion, Decision

# Introduction

LTE-MTC Release 15 introduces the resynchronization signal (RSS) [1][2]. The WID of Rel-16 MTC enhancements for LTE states that [3]:

The objective is to specify the following set of improvements for machine-type communications for BL/CE UEs.

Mobility enhancement:

Consider improving the DL RSRP and, if needed, RSRQ measurement accuracy, through use of RSS [RAN1, RAN4, RAN2]

In RAN1 #94 the following agreement was made regarding using RSS for mobility enhancements

Agreement

From RAN1 perspective, it is feasible to use RSS for measuring RSRP for cells at least for IDLE mode mobility

- RAN1 to identify the related parameters in the next RAN1 meeting

The contribution discusses the possible RAN2 impacts.

# Discussion

The new resynchronization signal (RSS) is a signal that will assist UEs in poor coverage to achieve network synchronization. This is achieved by providing substantial synchronization energy over a short time interval. RSS is allocated to the 11 contiguous symbols in a subframe, spanning 2 PRBs in the frequency domain. Both the duration and the periodicity of RSS are configurable. RSS may take durations of 8 to 40 ms and periods of 160 to 1280 ms. Existing ASN.1 structure for RSS configuration from 36.331 is presented below.

-- ASN1START

RSS-Config-r15 ::=				SEQUENCE {

duration-r15					ENUMERATED {sf8, sf16, sf32, sf40},

freqLocation-r15				INTEGER (0..98),

periodicity-r15					ENUMERATED {ms160, ms320, ms640, ms1280},

powerBoost-r15					ENUMERATED {dB0, dB3, dB4dot8, dB6},

timeOffset-r15					INTEGER (0..31)

}

-- ASN1STOP

RSS-Config field descriptions

duration Duration of RSS in subframes. Value sf8 corresponds to 8 subframes, value sf16 corresponds to 16 subframes and so on.

freqLocation Frequency location (lowest PRB number) of RSS.

periodicity Periodicity of RSS. Value ms160 corresponds to 160 ms, value ms320 corresponds to 320 ms and so on.

powerBoost Power offset of RSS relative to CRS in dB. Value dB0 corresponds to 0 dB, value dB3 corresponds to 3 dB, value dB4dot8 corresponds to 4.8 dB and so on.

timeOffset Time offset of RSS in frames. The actual value of time offset is based on the value of duration, as follows: For duration 160 ms, only value range 0 to 15 are applicable. Actual value = timeOffset * 1 frames. For duration 320 ms, actual value = timeOffset * 1 frames. For duration 640 ms, actual value = timeOffset * 2 frames. For duration 1280 ms, actual value = timeOffset * 4 frames.

From RAN1 perspective, it is feasible to use RSS for measuring RSRP for cells at least for IDLE mode mobility. However, they would also like RAN4 to study this especially more for the connected mode. Also, for the idle mode, because of the long RSS periodicities, RSS may not be available for measurement when a UE wakes up, in particular with an extended sleep duration and a UE significantly out-of-sync. Here, a UE may instead opt to use CRS-based measurements. Consequently, the relation between CRS-based and RSS-based measurements must be carefully studied.

Wait for RAN1 and RAN4 to /study this and specify how RAN2 can support.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	Wait for RAN1 and RAN4 to /study this and specify how RAN2 can support.

# References

R1-1807935, Introduction of Even Further Enhanced MTC for LTE, Ericsson, Busan, Korea, May 2018.

TS 36.211 V15.2.0, E-UTRA Physical channels and modulation, 3GPP TSG-RAN, Release 15, June 2018.

RP-181450, Rel-16 MTC enhancements for LTE, Ericsson, RAN #80, La Jolla, USA, June 2018.