3GPP TSG-RAN WG2 Meeting #103bis	R2-1814127

Chengdu, China, 8-12 October 2018

Agenda Item:	11.2.2.1

Source:	Charter Communications Inc.

Title:	Considerations on NR-U Paging in Unlicensed Spectrum

Document for: Discussion and Agreement

# Introduction

This contribution aims to clarify two aspects related to NR paging mechanism which require extension due to operations in unlicensed spectrum, together with proposing a new scenario for NR-U paging warranting consideration.

# Discussion

## Issue 1 – Introduce NR-U Paging over Primary Unlicensed Carrier

Although implicitly assumed thus far, there isn’t an explicit agreement in RAN2 to perform paging operation over unlicensed carrier. This is required for NR-U DC (where NR-U is PCell) and SA operations. Do note that RAN1#94 has already agreed to the benefits of supporting Paging over unlicensed carriers for NR-U DRS.

- Inclusion of the CSI-RS and RMSI-CORESET(s)+PDSCH(s) (carrying RMSI) associated with SS/PBCH block(s) in addition to the SS/PBCH burst set in one contiguous burst (tentatively referred to as the NR-U DRS) can be beneficial for

  - Meeting OCB requirement, Compacting signals in time domain to limit the required number of channel access and for short channel occupancy

  - Support of stand-alone NR-U deployments

  - Support of automatic neighbour relations (ANR) functionality in an NR-U deployment , Resolution of PCI confusion in an NR-U deployment

- Note: The NR-U DRS (it can be called something else in the future) can include signals and channels that are required for cell acquisition etc. and is not limited only to reference signals

- The transmission of additional signals such as OSI and paging within the NR-U DRS is allowed and can be beneficial

- Note: not imply that RMSI-CORESET+PDSCH and CSI-RS can only be transmitted as part of the NR-U DRS, and does not imply that these are necessarily part of all NR-U DRS transmissions.

Proposal 1: NG-RAN should support paging over unlicensed carrier for NR-U SA for RRC_IDLE, RRC_INACTIVE, and RRC_CONNECTED mode UEs.

Proposal 2: NG-RAN should support paging over unlicensed carrier for NR-U DC (where primary carrier is in unlicensed spectrum) for RRC_INACTIVE and RRC_CONNECTED mode UEs.

## Issue 2 –NR-U Paging impact due to LBT

Today the CN-based paging operation uses Class 2 operation on NG-AP, and a timer-controller operation @ NAS-level. At a high-level:

- For RRC_IDLE UEs, AMF informs NG-RAN node (either gNB, or ng-eNB) to initiate paging for UE identified via 5G-S-TMSI in certain cells/TAs via NG-AP:PAGING message

- NG-RAN node pages target UEs via RRC:PAGING message

- If target UEs do not respond to the page in a certain time duration, then AMF can re-initiate paging. At this juncture, paging escalation strategies may be applied e.g. previous page msg was restricted to few cells in 1 TA, but re-paging may involve increasing the paging area to e.g. all cells in that TA, or all cells in multiple TAs etc – paging escalation strategies aren’t standardized in 3GPP.

The NG-AP paging (a class 2) procedure from TS 38.413 [1] clause 8.5.2 is shown below:

The NAS procedure for Paging from TS 24.501 [2] clause 5.6.2.2.1 is shown below:

When introducing paging over unlicensed carriers, access to the medium can’t be guaranteed. For such cases, when the AMF starts T3513 [2] it will not be able to distinguish between:

- T3513 expiring due to UE not responding v/s

- T3513 expiring due to NG-RAN not being able to send the paging message on physical carrier (due to LBT not clearing)

Case a) should result in paging escalation at 5GC.

Case b) should allow the possibility to re-transmit the same paging message at the same granularity at least once more prior to escalating. This is to avoid unnecessary load on paging channels across multiple gNBs.

When discussed in the context of NR-U SA, to give a flavor of what the possible solutions could look like:

- Introduce an NR-U specific paging message of type Class 1 e.g. NG-AP:PAGING_U message and  NG-AP:PAGING_U RESPONSE message

- Convert existing Class 2 NG-AP:PAGING message to Class 1 message; this is a non-backward compatible change

Note: Possible solutions are mentioned in order to help understand the nature of resulting impact. The authors are not suggesting adoption of one of the above.

Proposal 3: NG-RAN Paging mechanism (including NG-AP impacts) should account for LBT.

## Issue 3 – Simultaneous NR-U Paging on Macro cell and small cells operating in Unlicensed Carrier(s) under a Macro cell coverage area

The figure below depicts a possible network deployment for Cable operators:

Figure 1 Unlicensed cells used to fill Licensed Carrier coverage gaps

In the above scenario small cells operating in unlicensed carriers are deployed to fill coverage (using either FR1 or FR2) holes left by macro node (gNB/ng-eNB). This is expected to be common for “indoor hot-spot” type coverage where outdoor signal suffers high attenuation, and is therefore augmented by deploying small cells at strategic locations to offset the O2I penetration loss. In Fig 1, G1’s coverage area fully covers S1 and S2’s coverage area, and partially covers S3. G2’s coverage is non-overlapping with G1. There could be other macro and small cell nodes with full or partial coverage overlaps, but are omitted for brevity.

With respect to outdoor-to-indoor signal penetration our testing has shown:

- For 3.5GHz, O2I penetration loss for concrete blocks is 20-23 dB, poured concrete (w/o hollow cavities) is 40-45db, and a complete signal attenuation for modern-day low-E glass (e.g. the kind mandated for state of Colorado)

- For 28GHz, O2I penetration loss is around 25-28 dB for high-E glass, and complete signal attenuation for concrete (hollow or otherwise), and modern-day low-E glass

Given the penetration loss range, depending on the building type (e.g. double-walled residential, high-E glass, low-E glass, concrete etc) and frequency used, for a subscriber who is situated indoors, the outdoor signal (from the macro node G1) is unlikely to penetrate indoor. In such scenarios, UE may be out of coverage of macro node (G1 and G2) but in coverage of (either outdoor or indoor) small cells (S3 gNB in Fig 1). If the UE underwent RRC_CONNECTEDRRC_IDLE/INACTIVE transition in the small cell (as the best cell for this UE), then subsequent pages can be routed via last_known_NGCI (which will point to this small cell). However, if UE underwent RRC_CONNECTEDRRC_IDLE/INACTIVE transition under the macro cell, and subsequently moved indoors under the small cell coverage, initial paging via macro node (G1) will fail. At this point, a paging area escalation will occur wherein multiple macro nodes in TA/TA_list (spanning G1, G2 etc) will be paged. This may not include small cell(s) or if it does then the number of small cell(s) which get included could be rather large. Depending on the multiple factors, the number of cells under a TA can range from 500-6000 spanning 30-300+ base stations. This causes an increased PDCCH load with a possible net effect of being unable to reach subscriber anyways.

There’s one of two ways to tackle such scenarios:

- Let CN (5GC) decide when and how to include small cells (under a macro cell area) for paging operations

- Let NG-RAN decide when and how to include the small cells (under a macro cell area) for paging operations

Usually detailed radio network planning is not made visible to CN. Therefore, adopting a solution based on a) would be ill-advised.

For a solution based on b), a possible way to alleviate this issue could be to create the notion of “Cell grouping for a given UE when in IDLE/INACTIVE mode”. Via implementation specific means e.g. normal radio network planning data (seen in Fig 1) fed via OAM, ANR, SON etc an operator could provision in Macro nodes a list of possible small cell nodes under its hierarchy. The small cells would generally be expected to either fall under or augment the master node’s coverage area.

For RRC_IDLE mode or RRC_INACTIVE UEs, the “Cell grouping for a given UE when in IDLE/INACTIVE mode” list can be taken from either the provisioned/learned associations (per above) or from the last known cell(s) from which the UEs last underwent the transition.

Depending on multiple factors such as PDCCH channel loading due to paging etc, when paged by CN, macro node could request such (fixed association) neighbor secondary nodes (which could operate in unlicensed spectrum) to also page the UE.

Correspondingly, a mechanism in NG-RAN will also be needed to allow NG-RAN to control whether and when NG-AP:PAGING message is sent over macro cells only or small cells only or simultaneously in both.

Proposal 4: For paging message with cell- or node- granularity, NG-RAN should allow simultaneous paging via Macro node and small-cell nodes (with full or partial coverage under a Macro’s coverage).

Proposal 5: If Proposal 4 is acceptable, then NG-RAN should also support a mechanism to allow control of whether pages are sent via macro- only or small-cell only or on both.

# Conclusion

In this contribution, we put forth the following proposals:

Proposal 1: NG-RAN should support paging over unlicensed carrier for NR-U SA for RRC_IDLE, RRC_INACTIVE, and RRC_CONNECTED mode UEs.

Proposal 2: NG-RAN should support paging over unlicensed carrier for NR-U DC (where primary carrier is in unlicensed spectrum) for RRC_INACTIVE and RRC_CONNECTED mode UEs.

Proposal 3: NG-RAN Paging mechanism (including NG-AP impacts) should account for LBT.

Proposal 4: For paging message with cell- or node- granularity, NG-RAN should allow simultaneous paging via Macro node and small-cell nodes (with full or partial coverage under a Macro’s coverage).

Proposal 5: If Proposal 4 is acceptable, then NG-RAN should also support a mechanism to allow control of whether pages are sent via macro- only or small-cell only or on both.

# References

- TS 38.413 v15.1.0

- TS 24.501 v15.1.0

This applies also to case of when CU-DU and/or CU-CP/UP split is employed.

The general ball park used here is: For every Macro cell in lower FR1 range, there’s likely to be a minimum of 3-10 small cells. A given small cell could be under the coverage area of no more than 2-3 macro cells.