3GPP TSG-RAN WG2 Meeting #103bis	R2-1814220

# Chengdu, China, 08 - 12 October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0093 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RDI handling for data forwarding at handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Forwarded SDAP PDUs from the source gNB may alter the QoS flow to DRB mapping rules of the target gNB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A requirement is added to ensure that the source gNB sets the RDI bits of all forwarded SDAP PDUs to zero. Impact analysis Impacted functionality: mobility. Inter-operability: none (network change only). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | At handover, the RQoS decisions taken by the source gNB might propagate to the target gNB and invalidate the mapping rules of the target gNB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

##### 9.2.3.2.2	U-Plane Handling

The U-plane handling during the Intra-NR-Access mobility activity for UEs in RRC_CONNECTED takes the following principles into account to avoid data loss during HO:

-	During HO preparation U-plane tunnels can be established between the source gNB and the target gNB;

-	During HO execution, user data can be forwarded from the source gNB to the target gNB.

-	Forwarding should take place in order as long as packets are received at the source gNB from the UPF or the source gNB buffer has not been emptied.

-	During HO completion:

-	The target gNB sends a path switch request message to the AMF to inform that the UE has gained access and the AMF then triggers path switch related 5GC internal signalling and actual path switch of the source gNB to the target gNB in UPF;

-	The source gNB should continue forwarding data as long as packets are received at the source gNB from the UPF or the source gNB buffer has not been emptied.

-	When forwarding data, the source gNB shall guarantee that the Reflective QoS flow to DRB mapping Indication of forwarded packets is set to zero (see 3GPP TS 37.324 [9]).

For RLC-AM bearers:

-	For in-sequence delivery and duplication avoidance, PDCP SN is maintained on a per DRB basis and the source gNB informs the target gNB about the next DL PDCP SN to allocate to a packet which does not have a PDCP sequence number yet (either from source gNB or from the UPF).

-	For security synchronisation, HFN is also maintained and the source gNB provides to the target one reference HFN for the UL and one for the DL i.e. HFN and corresponding SN.

-	In both the UE and the target gNB, a window-based mechanism is used for duplication detection and reordering.

-	The occurrence of duplicates over the air interface in the target gNB is minimised by means of PDCP SN based reporting at the target gNB by the UE. In uplink, the reporting is optionally configured on a per DRB basis by the gNB and the UE should first start by transmitting those reports when granted resources are in the target gNB. In downlink, the gNB is free to decide when and for which bearers a report is sent and the UE does not wait for the report to resume uplink transmission.

-	The target gNB re-transmits and prioritizes all downlink data forwarded by the source gNB (i.e. the target gNB should first send all forwarded PDCP SDUs with PDCP SNs, then all forwarded downlink SDAP SDUs before sending new data from 5GC), excluding PDCP SDUs for which the reception was acknowledged through PDCP SN based reporting by the UE. The target gNB sends PDCP SDUs applying a QoS flow to DRB mapping the same as the mapping in the source gNB.

-	The UE re-transmits in the target gNB all uplink PDCP SDUs starting from the oldest PDCP SDU that has not been acknowledged at RLC in the source, excluding PDCP SDUs for which the reception was acknowledged through PDCP SN based reporting by the target. The UE applies the same QoS flow to DRB mapping as it does in the source gNB.

For RLC-UM bearers:

-	The PDCP SN and HFN are reset in the target gNB;

-	No PDCP SDUs are retransmitted in the target gNB;

-	The target gNB prioritises all downlink SDAP SDUs forwarded by the source gNB over the data from the core network;

-	The UE does not retransmit any PDCP SDU in the target cell for which transmission had been completed in the source cell.