3GPP TSG-RAN WG2 Meeting #103bis									  R2-1814268

Chengdu, China, 8th - 12th October 2018                   (Revision of R2-1811789)

Source: 	vivo

Title:  	Text proposals for the evaluation results of the RLM for NR-U

Agenda Item:	11.2.2.2

Document for:	Discussion and Decision

1	Introduction

This contribution is to provide the text proposals for TR 38.889, capturing the evaluation results from “R2-1814267 Evaluation of the RLM for NR-U”.

2	Text Proposal

---------------------start of change--------------------------------------------------------------------------

References

---omitted---

[xx] R2-1814267 Evaluation of the RLM for NR-U, vivo, RAN2#103bis, October 8th-12th 2018

---------------------next change--------------------------------------------------------------------------

2.3.1	RLM/RLF and mobility

    - 2.3.1.x	RLM evaluation result 1 [xx]

This simulation focuses on the dense-urban (macro-layer) deployment scenario, in which all macro cells are on the same frequency layer. The number of RLM-RS(s) using different set of resources per BWP is set to X. The number of RLF occurrences within a certain simulation period (i.e. 10s) is used for the evaluation of RLM. The LBT failure rate is the probability of the failed transmission after the channel access procedure.

Figure x. LBT failure rate vs. RLF occurrences

According to the figure illustrated above, the increase of LBT failure rate leads to a significant increase in the RLF number. For example, for the cell edge users with 2 RLM RS(s), the RLF number of the 30% LBT failure rate is almost 2 times of the RLF number of the 20% LBT failure rate. Furthermore the LBT failure could cause more impacts on the RLF for the cell center uses. For example, for the cell center users with 2 RLM RS(s), the RLF number of 30% LBT failure rate is about 17 times of the RLF number of the 20% LBT failure rate. By increasing the number of reference signals used for RLM, the RLF rate is reduced. However when the LBT falure rate increases, increasing the number of RLM reference signals may not bring sufficient reduction on the RLF. For example, for the cell edge users, the RLF number of 5 RLM RS(s) with the 60% LBT failure rate is about 16% more than the RLF number of 8 RLM RS(s) with the 60% LBT failure rate.

---------------------next change------------------------------------------------------------------------------

    - 2.3.1.y	Solution x: Increase the transmission opportunites for RLM-RSs [xx]

Figure x. Increasing the transmission opportunites for RLM-RS(s) in both time domain and frequency domain

In time domain, we can increase the number of configured RLM-RS(s) during the RLM indication period. In the frequency domain, the RLM reference signals in each indication period can be configured in different frequencies (e.g. different frequency channels within a BWP, different BWP(s), and different cells). By increasing the transmission opportunities of RLM RS(s), the RLF rate can be reduced. The drawbacks of increasing the number of configured RLM RS(s) are the increased resource cost, the increased cpmplexity, and the increased UE power consumption for RLM-RS monitoring.

---------------------next change------------------------------------------------------------------------------

    - 2.3.1.z	Solution y: UE detection of the missed RLM-RS due to LBT failure [xx]

Figure x. A criterion for indicating IS/OOS/LBT failure

As shown in Figure x, if the measurement result of the RLM-RS(s) is below a threshold (e.g. Threshold_1), the UE will consider that the RLM-RS is not transmitted by the network deu to LBT failure, and the physical layer in the UE will not indicate Out-Of-Sync to higher layer, instead indicating LBT failure. This solution can reduce the number of RLF rate due to the LBT failure of the RLM-RS(s). The drawback is that the UE detection of the missed RLM-RS due to LBT failure may not be very reliable, as the measurement result of the RLM-RS(s) could also be below the threshold when a temporay blockage occurs in the air-interface.

---------------------next change------------------------------------------------------------------------------

Annex A:
Evaluation methodology

---omitted---

A.x	Evaluation assumptions for RLM [xx]

The following assumptions are used for the evaluation of radio link monitoring:

- : The probability of indicating OOS to higher layers per indication period, which can calculated according to the following equation,

: The probability of not indicating any indication to higher layers per indication period, which can calculated according to the following equation,

- : The probability of indicating OOS to higher layers per indication period, which can calculated according to the following equation,

Table 1. Initial parameters for RLM

| Attributes | Values or assumptions |
| --- | --- |
| N310 | 4 |
| N311 | 4 |
| T310 | 500 ms |
| The number of RLM-RSs (X) | Up to 8 |
| RLM-RS resource | SSB |
| SSB periodicity | 10 ms |
| Indication period | 10 ms |
| Evaluation period | 5 s |
| The number of gNBs | 19 |
| The number of users in the network | 570 (19*3*10) |
| Qout mapping to SINR | -7.3 dB |
| Qin mapping to SINR | -4.3 dB |

Table 2. System parameters of system-level simulation

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 4 GHz |
| Mode | DL only |
| Bandwidth | 20 MHz |
| Subcarrier Spacing | 15 kHz |
| Channel Model | UMa in TR 38.900 |
| TXRU mapping to antenna elements | One TXRU per panel per polarization |
| TXRU mapping weights | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT. |
| Beam selection | Based on the criteria of maximizing receive power after beamforming. |
| ISD | 200 m |
| BS Tx power | 33 dBm |
| Minimum distance between gNB and UE | 35m |
| BS Antenna Configuration | (Mg, Ng, M, N, P) = (1,1,2,1,2) (dH, dV) = (0.5, 0.5)λ |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |
| UE Configuration | (Mg, Ng, M, N, P,) = (1, 1, 1, 1, 2); (dV,dH) = (0, 0)λ. The polarization angles are 0 and 90 |
| UE array orientation | ΩUT,uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree |
| BS antenna pattern | See Table A.2.1-6 in TR 38.802 |
| UE antenna pattern | Omni |
| BS antenna height | 25m |
| UE antenna height | Same as 3D-UMa in TR36.873 |
| UE receiver noise figure | 10dB |
| UE distribution | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 10 users per TRP |

Note that the distance between cell center user and gNB is uniform distribution between 35m and 0.9*2/3*ISD, while the distance between cell edge user and gNB is uniform distribution between 0.9*2/3*ISD and 2/3*ISD.

----------------------end of change--------------------------------------------------------------------------