3GPP TSG-RAN WG2 Meeting #103bis	R2-1814289

Chengdu, China, 08 - 12 October 2018	(Revision of R2-1812326)

Agenda item:	11.2.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	RRC connected mode mobility for NR-U

WID/SID:	FS_NR_unlic – Release 16

Document for:	Discussion and Decision

# 1	Introduction

Connected mode mobility was treated in the RAN2 email discussion 103#56 with the purpose of agreeing on what should be part of mobility solutions for NR-U in Release 16.

In this paper we present our views on the steps pertaining to mobility, such as measurement reporting configuring, measurement triggers, and optimizations of signalling related to HO.

# 2	Measurement Reporting configuration

In unlicensed spectrum, it is possible to have multiple PLMN operating on the same carrier frequency. For RRC connected mode mobility, this can eventually result in the UE measuring and reporting an unlicensed cell that that is not suitable for mobility. A similar problem existed in LAA for measurement and reporting of target SCells. To solve this issue, Rel-13 LAA specifications introduced the possibility to add a whitelist of cells in the measurement reporting configuration.

Blacklisting and whitelisting have been also introduced to NR measurement objects [1], bringing the same benefits to NR-U, as well.

Observation 1: Adding a blacklist or whitelist of cells in the measurement object is supported in NR, and therefore applicable also to NR-U. Those lists are sufficient to avoid measuring and reporting cells which are not suitable for mobility (e.g. belonging to a different PLMN).

# 3	Triggering Criteria

In the RAN2 email discussion 103#56, in Q2.2 it was discussed whether the unlicensed channel load justifies introducing a new trigger event for measurement reports.

In particular, it has been proposed that Channel Occupancy (CO) could be used as a trigger quantity for reporting. We recall that RSSI and CO reporting have been agreed for RAN2, based on the same measurements being available also in LAA. We further recall that those measured were justified in LAA primarily as aiding the UE in selecting a free cell.

Current trigger mechanisms comprise RSRQ, which is defined as the ratio of RSRP to RSSI. Thus, a UE observing higher CO will also see a decrease in its RSRQ and as such the RSRQ does already (at least partially) include the effect of load on a carrier frequency.

Unless clear benefits are identified as compared to existing triggering criteria, we propose not to adopt new triggering criteria which are based on Channel Occupancy and/or UE-reported RSSI.

As the channel quality is already captured with RSRQ, a CO- selective triggering would be meant to initiate a HO of a UE in the cell center, to a cell with less load. However, the CO in unlicensed is highly variable and the length of transmissions of other users unpredictable. Therefore, the benefit of triggering of reports in reaction to a situation as compared to periodic reports is not clear.

Proposal 2: NR-U adopts same triggering criteria as in baseline Rel-15. That is, no new triggering criteria based on Channel Occupancy and/or UE-reported RSSI measurements, only triggering criteria based on RSRP, RSRQ and SINR.

Proposal 3: In NR-U as in LAA, only periodic reporting of Channel Occupancy and RSSI is supported.

# 4	RRC Connected mode mobility: Handover

The impact of LBT on delay for control plane signaling was one of the topics of the aforementioned email discussion.

The general procedure for a HO is that the UE first sends a measurement report, based on the configured measurement events and triggering criteria. Then the source cell sends a handover command to the UE, after having contacted the target cell. Finally, the UE performs a random-access procedure towards the target cell and sends a handover complete message [2].

Reducing latency and increasing robustness: conditional HO command

The use of conditional HO has been still advocated in the email discussion. We note that a conditional HO serves to reduce access latency, and in the case of rapidly declining cell quality (e.g. because of high speed) to increase the robustness for a HO. In that a conditional HO is an optimization applicable to NR-U and NR. The only NR-U specific aspect would be the possible use of new measurement quantities, and the latency reduction would be helping to mitigate the effect of LBT. However, the purpose, the method, and the signalling flow of conditional HO are expected to be the same in NR as in NR-U.

For that reason the study the general aspects of conditional HO has been excluded from the NR-U SI in RAN plenary #101.

Observation 2: There is no need to study conditional HO within the NR-U SI unless NR-U specific aspects are identified.

Increasing robustness: random access

Once the UE has received the HO command, it will attempt to perform random access. It has been readily recognized that LBT failures present a challenge to this step and should be mitigated.  RAN2#103 therefore came to following agreement:

- R2 assumes that RACH may be enhanced by additional opportunities, e.g. in time or frequency domain, FFS which messages the additional opportunities apply to.

This has been also reflected RAN1#94 as:

- In some scenarios it is beneficial for the maximum RAR window size to be extended beyond 10 ms to increase robustness to DL LBT failure

  - FFS: Value of maximum RAR window size

Observation 3: An increased RAR window size mitigates the impact of LBT failures during an HO process.

In the email discussion in the answers to question 2.3 on the NR-U specific challenge of LBT on latency it was opined that commonalities for robustness procedures developed in NR-U with other SI/WIs are not an issue. It was further proposed in answers to question 3 that conditional HO is investigated for NR-U.

We think that the robustness is addressed with increased RAR window size. We also think that one should not develop separate procedures in separate SI/WIs for one common problem. The rational of designing a solution to one problem in one place is that parallel work is avoided, and cleaner specifications will be obtained. We therefore think that NR-U should wait for the completion of work on conditional HO for NR, before studying similar solutions.

Instead, we propose adopting in NR-U the handover and (P)SCell management procedures as agreed for NR Rel-15. Any mobility enhancements for operation in unlicensed spectrum may be considered in a second phase, only after such enhancements are introduced for licensed NR operation.

Proposal 4: LTE-like handover procedure agreed for NR SA in Rel-15 is considered as baseline handover procedure for NR-U SA. Enhancements of the mobility procedures should be considered after they are introduced in licensed NR SA.

# 5	Conclusion

In this paper we discussed open items on RRC connected mode mobility for NR-U, and made the following observations and proposals:

Observation 1: Adding a blacklist or whitelist of cells in the measurement object is supported in NR, and therefore applicable also to NR-U. Those lists are sufficient to avoid measuring and reporting cells which are not suitable for mobility (e.g. belonging to a different PLMN).

Proposal 2: NR-U adopts same triggering criteria as in baseline Rel-15. That is, no new triggering criteria based on Channel Occupancy and/or UE-reported RSSI measurements, only triggering criteria based on RSRP, RSRQ and SINR.

Proposal 3: In NR-U as in LAA, only periodic reporting of Channel Occupancy and RSSI is supported.

Observation 2: There is no need to study conditional HO within the NR-U SI unless NR-U specific aspects are identified.

Observation 3: An increased RAR window size mitigates the impact of LBT failures during an HO process.

Proposal 4: LTE-like handover procedure agreed for NR SA in Rel-15 is considered as baseline handover procedure for NR-U SA. Enhancements of the mobility procedures should be considered after they are introduced in licensed NR SA.

# References

- 3GPP TS 38.331

- 3GPP TS 38.300