3GPP TSG RAN WG2 #103bis 	 R2-1814324

Chengdu, China, 8 – 12 October, 2018

Agenda Item:	9.2 TEI15

Source: 	Ericsson

Title:  	Configured Scheduling during Random Access Procedure

Document for:	Discussion and Decision

# Introduction

It is possible for a UE to perform a contention-based Random Access procedure when it has a configured uplink grant, which may lead to failure in transmitting Msg3. It has been discussed in NR and briefly in LTE how to solve the problem. This contribution proposes the same solution for LTE as is proposed for NR.

# Discussion

During random access procedure, HARQ transmission is supported for Msg3. However, the random access procedure may be triggered for a UE when configured scheduling is activated for the UE. A HARQ process (e.g. HARQ process 0) which is configured for configured scheduling may also be used for Msg3 transmission. Msg3 in the HARQ buffer may be overwritten by a MAC PDU generated for the configured uplink grant, and the UE may fail to retransmit Msg3.

In [1] and [2] a detailed analysis of the problem is described and a solution is proposed for NR. It was decided in RAN2#103 that LTE will align with NR in solving the problem. Therefore, a corresponding solution for LTE is provided in [3] and it is proposed that RAN2 discusses and agrees on the solution for LTE.

Proposal 1: Discuss and agree on a solution for Msg3 transmission colliding with configured scheduling.

# Summary

RAN2 is kindly asked to discuss the following observations and proposals:

Proposal 1: Discuss and agree on a solution for Msg3 transmission colliding with configured scheduling.

# References

- R2-1814797, Configured Scheduling during Random Access Procedure, Ericsson

- R2-1814798, Correction on Configured Scheduling during Random Access Procedure, Ericsson

- R2-1814325, Correction on Configured Scheduling during Random Access Procedure, Ericsson