3GPP TSG-RAN WG2 Meeting #103bis	     R2-1814362

Chengdu, P.R. China, 8-12 October 2018

Agenda Item:	11.1.2

Source:	Ericsson, AT&T, KDDI, LG, Kyocera, ZTE, Vivo

Title:	Way Forward on RLC Termination

Document for:	Discussion, Decision

# 1	Introduction

The current TR 38.874 v0.5.0 contains multiple architecture variants for supporting IAB. Given that there are only two more meetings as part of the study phase of IAB and it is neither necessary nor feasible to have many options standardized, it would be highly beneficial to down select the number of architectures for further consideration as part of the work item phase.

It would be highly beneficial to down select the number of architectures for further consideration during the remaining phase of the study item.

One way of doing this could be to agree on some fundamental issues affecting the architecture options and which have now been sufficiently studied in the study phase. Failing to do so will risk that considerable work item time will be spent next year in doing this down selection, leaving us insufficient time to ensure the specifications of IAB are finalized in rel-16.

One such issue is the termination point for RLC. Fundamentally, two options have been considered so far:

- Hop-by-hop ARQ: terminating the UE RLC in the IAB node, and rely on hop by hop RLC for the wireless backhaul links

- End-to-end ARQ: terminating the parts of the UE RLC in the Donor DU (ARQ), and parts of the RLC (e.g. segmentation) in the IAB node and intermediate nodes

One open issue is the termination point in the network of the UE’s RLC ARQ. All architecture options but one terminate the RLC at the access IAB node (i.e. hop-by-hop RLC).

This contribution analyses this issue further by looking at both options from different angles.

# 2	Overview

Currently all architecture variants of architecture group 1a in TR 38.874 (except Option 1 UP alternative) terminate UE’s RLC ARQ in the access IAB node. Alternative 1, on the other hand, terminates parts of RLC (i.e. segmentation) in the access IAB node and intermediate IAB nodes, while RLC ARQ is terminated at the Donor DU (as shown in the figure below).

There are several issues with terminating RLC ARQ for the UE at the Donor DU:

- It introduces a new functional split in 3GPP where MAC/RLC Segmentation is terminated in IAB nodes, while RLC ARQ in donor DU. Introducing such a new functional split may lead to significant work in RAN2/3 to specify and handle inter-node (Donor DU and IAB node) and inter-layer (RLC and MAC) interactions.

  - Note: This new split corresponds to the Option 3 split in 38.801 which have already been studied and rejected in RAN3.

- More capability required in the Donor DU to terminate the end to end RLC ARQ for all bearers of all UEs connected to all the IAB nodes that are served by the Donor DU. This increases the risk that not all potential gNB-DUs on the market would be capable to act as Donor DUs, which in turn limits the viability of IAB node deployments.

- It is questionable if the solution can support IAB node topology changes, specifically those involving Donor DU change, since all RLC entities of all bearers of all the UEs under the relocating IAB node and all the UEs under the descendant IAB nodes of the relocating IAB node need to be reset and re-established.

  - This is not transparent to the UEs and may cause service interruption as RLC packets that are partially transmitted over some of the hops have to be retransmitted again.

  - An alternative to avoid this is to perform RLC context transfer for all the affected bearers between the two DUs. Such functionality is supported neither in NR nor LTE, and there is no interface between the DUs.

Terminating parts of RLC in the DU and other parts in the IAB node introduces a new functional split in 3GPP which will lead to significant work in RAN2/RAN3. RAN3 has already rejected such functionality split during NR CU-DU split discussions (See 38.801 Option 3).

Terminating the end to end RLC ARQ at the Donor DU increases the capability requirements of the Donor DU, increasing the risk that not all DU would support IAB nodes, which limits the viability of IAB node deployments.

Since RLC relocation is not supported in 3GPP networks and UEs, inter-donor DU IAB node change is likely to lead to service interruption for all UEs connected to the IAB node or descendant IAB nodes in case RLC is terminated at the donor DU.

# 3 	Feasibility and performance comparison

TR 38.874 contains a comparison table of hop by hop RLC and end to end RLC. To quickly summarize, there are some minor pros/cons with the two solutions.

End-to-end ARQ suffers from slower and less efficient re-transmission because transmission failure is not detected hop by hop but end to end. It could benefit from the possibility to forward segment (without performing reassembly), but not performing reassembly would increase overhead since the RLC segments may become smaller and smaller at each hop.

Both solutions may require some parameter tuning due to the support of multi-hop e.g. End-to-end need to tune RLC behaviour due to longer delay, while hop-by-hop configurations have to ensure that PDCP window is not exceeded.

End-to-end RLC may have some issues with RLC window managements, e.g. stalls, more lost RLC status reports, etc. As for the handling of RLF due to reaching the maximum number of RLC retransmissions, since end to end RLC retransmissions take longer time than hop-by-hop, detecting and triggering an RLF due to reaching the maximum number of RLC retransmission may be delayed significantly and that can eventually cause additional packet losses (e.g. due to PDCP discard timer expiry) or unnecessary retransmissions (e.g. due to TCP timeout).

Hop-by-hop RLC may not guarantee lossless delivery of UL packets during an RLF on a backhaul link. This issue has been extensively discussed in the email discussion on the reliability of hop by hop RLC [1], and there are several solutions available on handling it. In our view, an RLF between two network nodes is a serious issue that the network deployment should try to avoid rather than a normally occurring scenario. Having some packet loss in such scenarios can even be considered as acceptable, as it is very likely that the performance is going to be degraded anyways, even if we assure UL lossless delivery is guaranteed (either via end to end RLC or via hop by hop RLC with the proposed enhancements) due to delays related to establishing the new route.  Apart from the solutions discussed in [1], multi-connectivity solutions can also be used to actively change the route when a problem occurs on one route.

The following observations are made:

Both end to end and hop by hop RLC are technically feasible and can be supported with proper configuration.

End to end RLC leads to several issues while having no significant benefit as compared with hop by hop RLC, except for guaranteeing UL lossless packet delivery during an RLF on a backhaul link between two IAB (network) nodes.

RLF on a backhaul link between network nodes is not a frequently occurring scenario, and even if lossless delivery is guaranteed, performance is likely going to degrade anyways due to the long delays related to establishing a new route.

Given that both solutions are feasible and there is no clear benefit with end to end RLC, it is proposed to adopt a hop-by-hop RLC solution for multi-hop IAB networks

The rel-16 work item on IAB should continue with the assumption that hop by hop RLC will be used.

# 4 	Conclusion

The following observation is made:

Observation 1	It would be highly beneficial to down select the number of architectures for further consideration during the remaining phase of the study item.

Observation 2	One open issue is the termination point in the network of the UE’s RLC ARQ. All architecture options but one terminate the RLC at the access IAB node (i.e. hop-by-hop RLC).

Observation 3	Terminating parts of RLC in the DU and other parts in the IAB node introduces a new functional split in 3GPP which will lead to significant work in RAN2/RAN3. RAN3 has already rejected such functionality split during NR CU-DU split discussions (See 38.801 Option 3).

Observation 4	Terminating the end to end RLC ARQ at the Donor DU increases the capability requirements of the Donor DU, increasing the risk that not all DU would support IAB nodes, which limits the viability of IAB node deployments.

Observation 5	Since RLC relocation is not supported in 3GPP networks and UEs, inter-donor DU IAB node change is likely to lead to service interruption for all UEs connected to the IAB node or descendant IAB nodes in case RLC is terminated at the donor DU.

Observation 6	Both end to end and hop by hop RLC are technically feasible and can be supported with proper configuration.

Observation 7	End to end RLC leads to several issues while having no significant benefit as compared with hop by hop RLC, except for guaranteeing UL lossless packet delivery during an RLF on a backhaul link between two IAB (network) nodes.

Observation 8	RLF on a backhaul link between network nodes is not a frequently occurring scenario, and even if lossless delivery is guaranteed, performance is likely going to degrade anyways due to the long delays related to establishing a new route.

Leading to the following proposal:

Proposal 1	The rel-16 work item on IAB should continue with the assumption that hop by hop RLC will be used.

It is proposed to agree to text proposal in section 6 to TR 38.874

# 5	References

[1] Summary of [103#53] [IAB] E2E reliability in hop-by-hop RLC ARQ, LG (Rapporteur), RAN2 #103bis, China, October 2018.

# 6	Text proposal to TR 38.874

### 8.2.3 	Multi-hop RLC ARQ

For RLC AM, ARQ can be conducted hop-by-hop along access and backhaul links (Figure 8.2-1b, c and 8.2-2). It is also possible to support ARQ end-to-end between UE and IAB-donor (Figure 8.2-1a). Since RLC segmentation is a just-in-time process it is always conducted in a hop-by-hop manner. The figures show example protocol stacks and do not preclude other possibilities.

The study includes hop-by-hop and end-to-end RLC ARQ.

The type of multi-hop RLC ARQ and adaptation-layer placement have the following interdependence:

- End-to-end ARQ: Adaptation layer is integrated with MAC layer or placed above MAC layer

- Hop-by-hop ARQ:  No interdependence

End-to-end reliability requires further study.

Table 8.2.3-1: Observations for end-to-end and hop-by-hop ARQ

| Metric | Hop-by-hop RLC ARQ | End-to-end RLC ARQ |
| --- | --- | --- |
| Forwarding latency | Potentially higher as packets have to pass through RLC-state machine on each hop. | Potentially lower as packets do not go through the RLC state machine on intermediate IAB-nodes. |
| Latency due to retransmission | Independent of number of hops | Increases with number of hops |
| Capacity | Packet loss requires retransmission only on one link. Avoids redundant retransmission of packets over links where the packet has already been successfully transmitted. | Packet loss may imply retransmission on multiple links, including those where the packet was already successfully transmitted. |
| Hop count limitation due to RLC parameters | Hop count is not affected by max window size. | Hop count may be limited by the end-to-end RLC latency due to max window size. |
| Hop count limitation due to PCDP parameters | Hop count may be limited by increasing disorder of PDCP PDUs over sequential RLC ARQ hops. This may increase probability to exceed max PDCP window size. | Hop count does not impact disorder of PDCP PDUs due to RLC ARQ. |
| Processing and memory impact on intermediate IAB-nodes | Larger since processing and memory is required on intermediate IAB-nodes. | Smaller since intermediate path-nodes do not need ARQ state machine and flow window. |
| RLC specification impact | No stage-3 impact expected | It requires a change to the RLC-MAC layer interaction and a change to RLC implementation relative to NR Release 15. The adaptation layer would need to receive the allowed PDU size from the MAC layer and convey this to the RLC layer. |
| L2 overhead | Equal or less compared to end-to-end RLC ARQ. | Higher if there is RLC segmentation as the adaptation layer header is added to each RLC SDU segment. |
| Operational impact for IAB-node to IAB-donor upgrades | IAB-nodes and IAB-donors use the same hop-by-hop RLC ARQ. As a result, this functionality is completely unaffected by the upgrade of IAB-node to IAB-donor at availability of fiber, potentially reducing the effort required to confirm proper operation. | End-to-end RLC ARQ results in a greater architectural difference between IAB nodes vs. IAB donor nodes. As a result, additional effort may be required to complete an upgrade of an IAB node to an IAB donor upon availability of fiber. |
| Configuration complexity | RLC timers are not dependent on hop-count. | RLC timers become hop-count dependent. |
| Lossless delivery of UL data during topology change (e.g. failure of radio link between IAB nodes) | Current specification cannot ensure data lossless delivery when IAB topology changes are performed without additional enhancements (examples listed below). | Lossless delivery ensured due to end to end RLC feedback. |

The issue of end to end reliability in hop-by-hop RLC ARQ case could be addressed by specifying, e.g., the following mechanisms:

- Modification of PDCP protocol/procedures. This solution would not be applicable to Rel-15 UEs which means that Rel-15 UE performance may be impaired.

- Rerouting of PDCP PDUs buffered on intermediate IAB-nodes in response to a route update (FFS what information needs to be exchanged between IAB nodes).

- Introducing UL status delivery (from the Donor gNB to the IAB node), whereby the IAB node can delay the sending of RLC ACKs to the UE until a confirmation of reception at the Donor gNB

Considering the anticipated high standardization impacts of end to end RLC ARQ and possible performance degradation during normal operations as compared to hop-by-hop RLC ARQ, without any clear benefit except in scenarios where there is an RLF on the backhaul links, hop-by-hop RLC ARQ has been chosen for rel-16.