3GPP TSG-RAN WG2 Meeting #103bis	R2-1814462

Chengdu, China, 8 - 12 October 2018

Agenda item:	12.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Solution candidates for minimizing HO interruption time in LTE

WID/SID:	LTE_feMob-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

The WID on further LTE mobility enhancements RP-181544 contains the following objectives:

- Specify further enhancements to achieve following targets, [RAN2/3]

  - reduce user data interruption during handover, which targets as close as possible to 0ms, i.e. relaxed requirements could be considered.

  - improve the robustness during handover

The WID also states that there will be a study phase first to evaluate the proposed solutions: Hence, in this contribution, we focus on the potential solutions for the first objective, i.e. minimization of interruption time during HO.

# 2	Scope of the work on reduction in user data interruption time

## 2.1	Scope of the user interruption reductions

The WID itself states the following example solutions to be considered:

- simultaneous connectivity with both source and target eNB,

- conditional handover

- enhancements to make-before-break

- support of carrier aggregation in source and carrier aggregation in target eNB during handover

The scope is also supposed to contain inter/intra-frequency handovers, inter-/intra-eNB handover and synchronous/asynchronous deployments (see further details on how synchronous and asynchronous should be understood in the R2-1814460 [1]). While it’s not perhaps possible to focus only on intra- or inter-frequency handovers, we note that inter-eNB handovers in asynchronous networks tend to cover also the intra-eNB and synchronous HO cases. Therefore, we think it would be possible to focus on those cases in the beginning to have a unified solution catering for all cases.

Proposal 1: Focus the WID study phase on inter-eNB handovers in asynchronous networks (with both intra- and inter-frequency handovers).

Further, we would like to note that the “as close as possible to 0ms” seems a bit open wording: As discussed in R2-1814460  [1], current LTE standard already allows for ~5ms interruption time (with Rel-14 RACH-less, make-before-break HO), but with assumption of fully synchronous network and the same TA (or TA = 0) for both source and target cells. These limitations are likely not changed, unless something completely different is done, as they are based on real-life implementations in both UEs and networks. Therefore, it would be necessary that each solution proposal clearly characterizes the user interruption time that can be achieved with the solution to allow comparing different solutions. For example, if it would be possible to reach the ~5ms interruption time with asynchronous networks and without RACH-less HO, that would already be an improvement over current LTE.

Observation 1: Current LTE allows for ~5ms interruption in user data (with MBB and RACH-less HO).

Proposal 2: When evaluating a solution proposal, the interruption time allowed should be clearly indicated.

## 2.2	Definition of user data interruption during HO

The definition of user data interruption during handover should first be remembered: For a UE in handover, it is defined as the time between cessation of user data scheduling at source eNB to the resumption of user data scheduling at target eNB. Figure 1 shows the C-plane handling diagram for the HO (taken from TS36.300), from which we can see that the user interruption is the time between steps 7 and 11 (i.e. the “handover execution” phase), consisting of following steps:

- UE acquiring synchronization to target eNB.

- UE completing random access procedure towards the target eNB.

- UE sending RRCConnectionReconfigurationComplete to target eNB.

- Data forwarding and SN status transfer from source eNB to target eNB.

Out of these, the step 4 is different than others since it’s done fully inside the network and does not have a direct UE signalling component. Thus, it can be potentially done “out of order” from other steps, even though the step itself is required to finish the handover, which also terminates  the user data interruption. Therefore, improvements related to that, while mostly in the scope of RAN3, could still be discussed and the candidate solutions should also indicate whether there are some differences in the handling of that step.

Observation 2: Improvements to the data forwarding and SN status transfer are RAN3 responsibility, and any proposals causing changes there need to be consulted with RAN3.

Finally, we would note that while user interruption and handover robustness are related, they are not the same thing: Robust handover procedure may even involve larger user interruption time (e.g. due to data repetition when sending the HO command), and smaller interruption time may also result in less robust handovers (e.g. due to larger chance of doing handover quickly, resulting in increased probability for too early handovers). More on how the robustness shall be understood in terms of handover can be found in our associated TDoc R2-1814463 [2].

Proposal 3: The following definition is used for user data interruption during handover: User data interruption is the time between UE executing the handover command (i.e. stopping listening to source cell scheduling) until the UE is scheduled with UP data in the target cell.

Figure 1. C-plane handling for LTE handovers (from TS36.300, subclause 10.1.2.1.1)

# 3	Potential solutions for minimizing HO interruption time

## 3.1	Basic LTE handover process

For comparison purposes, we show the basic LTE handover process in Figure 2 below. This is then used in the subsequent subsections for comparison purposes, to illustrate the differences to the existing HO procedure.

Figure 2: Basic handover process

## 3.2	DC-based handover

The DC-based handover relies on establishing a connection between the source and target eNBs before the handover occurs, as described in R2-1708588. Thus, the source eNB would become the MeNB and the target eNB would become the SeNB, and the roles would be swapped during the “handover” procedure. Finally, after the role swap, the SeNB (which is the source eNB now) could be released to fallback to the regular LTE transmissions. This is depicted in simplified terms in the Figure 3 below.

Figure 3. Illustration of DC-based handover procedure

Hence, we can quantify the steps involved in the DC-procedure as follows:

- Setup LTE DC (with target cell added as SeNB), changing bearers to split bearers.

- Swap the roles between PCell and PSCell (which means the PDCP anchoring point is switched from source to target eNB).

- Release the SeNB (which retains the target eNB).

With this procedure, there is no service interruption since UE is continuously connected to either source or target eNB, so the interruption time is equal to zero ms. The specification effort for doing this lies in the step 2, which has currently not been standardized. While this procedure requires more detailed discussion, we illustrate one possible message sequence chart for this below in Figure 4.

Figure 4: Example of DC role swap signalling flow

Observation 3: DC-based handover can provide zero ms interruption time in HO.

We would also note that the DC-based HO obviously requires some RAN3 work as it requires X2 signalling modifications: the exact impacts would require RAN3 work as well, but are not expected to be large.

Observation 4: The DC-based handover requires some RAN3 work.

Proposal 4: Consider DC-based handover as a candidate solution for reducing user data interruption.

## 3.3	Conditional handover

Conditional handover (CHO) has been proposed as a remedy for “faster” handovers, i.e. to initiate the handover earlier. Figure 5 below shows the basic procedural steps for this: UE receiving a conditional HO command, triggering the HO event and executing the handover procedure (which completes the procedure).

Figure 5: Conditional handover procedural steps

To illustrate the signalling procedure in details, we provide an example signalling chart in Figure 6 below.

Figure 6: Conditional handover signalling flow

However, what seems evident from the description is that the actual HO execution procedure is still the same. Therefore, the user interruption time is exactly the same as before – only the HO may be triggered earlier since the UE measurement reporting procedure is omitted, which may contribute somewhat to the HO robustness. We analyse the HO robustness more in R2-1814463 [2], but at least we observe and propose the following:

Observation 4: Conditional handover does not improve user data interruption time, even if it may improve the HO robustness

Proposal 5: Conditional handover is not considered as a candidate solution for minimizing user data interruption time.

# 4	Conclusions

We have discussed the candidate schemes

Observation 1: Current LTE allows for ~5ms interruption in user data (with MBB and RACHless HO).

Observation 2: Improvements to the data forwarding and SN status transfer are RAN3 responsibility, and any proposals causing changes there need to be consulted with RAN3.

Observation 3: DC-based handover can provide zero ms interruption time in HO.

Observation 4: Conditional handover does not improve user data interruption time, even if it may improve the HO robustness

Based on these and discussion in the contribution, we propose the following:

Proposal 1: Focus the WID study phase on inter-eNB handovers in asynchronous networks (wth both intra- and inter-frequency handovers).

Proposal 2: When evaluating a solution proposal, the interruption time allowed should be clearly indicated.

Proposal 3: The following definition is used for user data interruption during handover: User data interruption is the time between UE executing the handover command (i.e. stopping listening to source cell scheduling) from UE being scheduled with UP data in the target cell.

Proposal 4: Consider DC-based handover as a candidate solution solution for reducing user data interruption.

Proposal 5: Conditional handover is not considered as a candidate solution for minimizing user data interruption time.

# References

- R2-1814460 The analysis of LTE mobility interruption and possible enhancement directions, 3GPP TSG-RAN WG2 Meeting #103bis Chengdu, China, 8 - 12 October 2018.

- R2-1814463 Potential Improvements of Mobility Robustness in LTE, 3GPP TSG-RAN WG2 Meeting #103bis Chengdu, China, 8 - 12 October 2018.