3GPP TSG-RAN2 Meeting #103bis 	R2-1814553

Chengdu, China, 8th – 12th, October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | CRNum | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SCG failure handling in case of NE-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (draft CR to be merged into rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-27 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SCGFailureInformation message is missing in the ASN.1 for the NE-DC option. Also, the procedural text needs to be updated since it covers only EN-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SCGFailureInformation message added in the ASN.1 for NE-DC and the procedural text has been updated accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not implemented the UE can not perform the SCG failure procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.13, 6.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TR 38.331 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 5.6.13	SCG failure information

#### 5.6.13.1	General

Figure 5.6.13.1-1: SCG failure information

The purpose of this procedure is to inform E-UTRAN about an SCG failure the UE has experienced i.e. SCG radio link failure, SCG change failure.

#### 5.6.13.2	Initiation

A UE initiates the procedure to report SCG failures when SCG transmission is not suspended and when one of the following conditions is met:

1>	upon detecting radio link failure for the SCG, in accordance with 5.3.11; or

1>	upon SCG change failure, in accordance with 5.3.5.7a; or

1>	upon stopping uplink transmission towards the PSCell due to exceeding the maximum uplink transmission timing difference when powerControlMode is configured to 1, in accordance with subclause 7.17.2 of TS 36.133 [29].

In case of DC, upon initiating the procedure, the UE shall:

1>	suspend all SCG DRBs and suspend SCG transmission for split DRBs;

1>	reset SCG-MAC;

1>	stop T307;

1>	if the UE is operating in NE-DC:

2>	initiate transmission of the SCGFailureInformationEUTRA message as specified in TS 38.331 [X, 5.7.xa];

1>	else

2> initiate transmission of the SCGFailureInformation message in accordance with 5.6.13.3;

END OF CHANGES

START OF CHANGES

#### 5.6.13.x	Failure type determination in NE-DC

The UE shall set the SCG failure type as follows:

1>	if the UE initiates transmission of the SCGFailureInformationEUTRA message due to T307 expiry:

2>	set the failureType as t307-Expiry;

1>	else if the UE initiates transmission of the SCGFailureInformationEUTRA message to provide reconfiguration with sync failure information for an SCG:

2>	set the failureType as scg-ChangeFailure;

1>	else if the UE initiates transmission of the SCGFailureInformationEUTRA message to provide random access problem indication from SCG MAC:

2>	set the failureType as randomAccessProblem;

1>	else if the UE initiates transmission of the SCGFailureInformationEUTRA message to provide indication from SCG RLC that the maximum number of retransmissions has been reached:

2>	set the failureType as rlc-MaxNumRetx;

#### 5.6.13.x	Setting the contents of MeasResultSCG-FailureNR

The UE shall set the contents of the MeasResultSCG-FailureNR as follows:

1>	for each MeasOjectEUTRA for which a measId is configured and measurement results are available;

2>	include rlf-ReportEUTRA in measResultsPerMOList;

END OF CHANGES

START OF CHANGES

### 6.3.5	Measurement information elements

#### –	MeasResultSCG-FailureNR

The IE MeasResultSCG-Failure is used to provide information regarding failures detected by the UE in case of NE-DC.

MeasResultSCG-FailureNR information element

-- ASN1START

-- TAG-MEAS-RESULT-SCG-FAILURE-NR-START

MeasResultSCG-Failure ::=           SEQUENCE {

measResultPerMOList                     MeasResultList2EUTRA,

...

}

MeasResultList2EUTRA ::=               SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2EUTRA

MeasResult2EUTRA ::=  SEQUENCE {

rlf-ReportEUTRA				RLF-ReportEUTRA  	OPTIONAL,

...

}

-- TAG-MEAS-RESULT-SCG-FAILURE-NR-STOP

-- ASN1STOP

END OF CHANGES