3GPP TSG-RAN WG2#103	R2-1814561

Chengdu, China, 2018-10-08 to 2018-10-12

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | CRNum | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MCG failure handling in case of NE-DC and NR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (draft CR to be merged into rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT_core |  |  |  |  |  |  | Date: |  | 2018-09-27 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In case of NE-DC and NR-DC, there is need for further improvements compared to EN-DC in minimizing the service interruption time and maximizing the availability for URLLC services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE suspends MCG transmissions and sends MCG Failure Information if MCG RLF is detected when NE-DC or NR-DC is active and split SRB is configured. In addition, UE falls back to the RRC connection re-establishment procedure and does not try to report MCG Failure Information to SCG in case of simultaneous or consecutive RLF detections on MCG and SCG. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The consequence would be that UE has to do re-establishment and experience service interruption. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.x MCG failure information |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 5.7.x	MCG failure information

#### 5.7.x.x	 General

Figure 5.7.x.x-x: MCG failure information

The purpose of this procedure is to inform NR MN about an MCG failure the UE has experienced i.e. MCG radio link failure.

#### 5.7.x.x Initiation

If MCG transmissions are not already suspended, a UE initiates the procedure to report MCG failures when SCG transmissions are not suspended and when one of the following conditions is met:

1>  upon detecting radio link failure of the MCG, in accordance with 5.3.10; or

1>  upon re-configuration with sync failure of the MCG, in accordance with sub-clause 5.3.5.8.3; or

1>  upon mobility from NR failure, in accordance with sub-clause 5.4.3.5; or

1>  upon integrity check failure indication from lower layers concerning SRB1 or SRB2.

Upon initiating the procedure, the UE shall:

1>	suspend MCG transmission for all SRBs and DRBs;

1>	reset MCG-MAC;

1>	stop T304, if running;

#### 5.7.x.x Actions related to transmission of MCGFailureInformation message

The UE shall set the contents of the MCGFailureInformation message as follows:

1>	if the UE initiates transmission of the MCGFailureInformation message to provide MCG radio link failure information:

2>	include failureType and set it to the trigger for detecting MCG radio link failure;

1>	set the measResultListNR to include for each MCG cell that is configured, if any, within measResultNR the quantities of the concerned PCell, if available according to performance requirements in [x];

1>	for each MCG serving frequency included in measResultListNR, include within measResultNR the physCellId and the quantities of the best non-serving cell, based on RSRP, on the concerned serving frequency;

1>	set the measResultNeighCells to include the best measured cells on non-serving NR frequencies, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows;

2>	if the UE was configured to perform measurements for one or more non-serving NR frequencies and measurement results are available, include the measResultListNR;

2>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

1>	if SRB1 is configured as Split SRB;

2>	if the primaryPath is associated with the MCG;

3> the UE shall change the primaryPath to SCG;

The UE shall submit the MCGFailureInformation message to lower layers for transmission.

END OF CHANGES