3GPP TSG-RAN2 Meeting #103bis 	R2-1814565

Chengdu, China, 2018-10-08 to 2018-10-12

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | CRNum | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR RRC details for NE-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (draft CR to be merged into rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-27 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | NR ASN.1 is currently not supporting NE-DC, therefore new fields needs to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Separate bearer configuration fields (similar to nr-RadioBearerConfigX in LTE RRC) are introduced per termination point in NR RRC. LTE lower layer configuration generated by LTE RRC is encapsulated into the NR RRC message as an LTE RRC message. In addition, MR-DC release field (see R2-1814668 and R2-1814667 for the procedural text) is added to enable release and add. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NE-DC is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.3 Reception of an RRCReconfiguration by the UE 6.2.2 Message definitions |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

#### 5.3.5.3	Reception of an RRCReconfiguration by the UE

The UE shall perform the following actions upon reception of the RRCReconfiguration:

1>	if the RRCReconfiguration includes the fullConfig:

2>	perform the radio configuration procedure as specified in 5.3.5.11;

1>	if the RRCReconfiguration includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

1> if the RRCReconfiguration includes the masterKeyUpdate:

2>	perform security key update procedure as specified in 5.3.5.7;

1>	if the RRCReconfiguration message includes the mrdc-Release:

2>	release the MR-DC configurations as specified in section 5.3.5.10a;

Editor’s note: The procedures for MR-DC release are FFS.

1>	if the RRCReconfiguration includes the secondaryCellGroup:

2>	perform the cell group configuration for the SCG according to 5.3.5.5;

1>	if the RRCReconfiguration message contains the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

1>	if the RRCReconfiguration message includes the radioBearerConfig2:

2>	perform the radio bearer configuration according to 5.3.5.6

1>	if the RRCReconfiguration message includes the eutra-secondaryCellGroup:

2>	perform the cell group configuration for the SCG according to TS 36.331 [10], section 5.3.5.4;

1>	if the RRCReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	if the RRCReconfiguration message includes the dedicatedSIB1-Delivery:

2> perform the action upon reception of SIB1 as specified in 5.2.2.4.2;

1>	if the RRCReconfiguration message includes the dedicatedSystemInformationDelivery:

2> perform the action upon reception of System Information as specified in 5.2.2.4;

1>	set the content of RRCReconfigurationComplete message as follows:

2>	if the RRCReconfiguration includes the masterCellGroup containing the reportUplinkTxDirectCurrent, or;

2>	if the RRCReconfiguration includes the secondaryCellGroup containing the reportUplinkTxDirectCurrent:

3> include the uplinkTxDirectCurrentList;

1>  if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (MCG is E-UTRA):

2> if RRCReconfiguration was received via SRB1:

3> submit the RRCReconfigurationComplete via the EUTRA MCG embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10];

3> if reconfigurationWithSync was included in spCellConfig of an SCG:

4> initiate the random access procedure on the SpCell, as specified in TS 38.321 [3];

3> else:

4>  the procedure ends;

NOTE:	The order the UE sends the RRCConnectionReconfigurationComplete message and performs the Random Access procedure towards the SCG is left to UE implementation.

2> else (RRCReconfiguration was received via SRB3):

3> submit the RRCReconfigurationComplete message via SRB3 to lower layers for transmission using the new configuration;

NOTE:	For EN-DC, in the case of SRB1, the random access is triggered by RRC layer itself as there is not necessarily other UL transmission. In the case of SRB3, the random access is triggered by the MAC layer due to arrival of RRCReconfigurationComplete.

1> else:

2> submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

1>  if reconfigurationWithSync was included in spCellConfig of an MCG or SCG, and when MAC of an NR cell group successfully completes a random access procedure triggered above;

2>  stop timer T304 for that cell group;

2>  apply the parts of the CQI reporting configuration, the scheduling request configuration and the sounding RS configuration that do not require the UE to know the SFN of the respective target SpCell, if any;

2>  apply the parts of the measurement and the radio resource configuration that require the UE to know the SFN of the respective target SpCell (e.g. measurement gaps, periodic CQI reporting, scheduling request configuration, sounding RS configuration), if any, upon acquiring the SFN of that target SpCell;

2>  if the reconfigurationWithSync was included in spCellConfig of an MCG:

3> if the active downlink BWP, which is indicated by the firstActiveDownlinkBWP-Id for the target SpCell of the MCG, has a common search space configured:

4> acquire the SIB1 of the target SpCell of the MCG, as specified in 5.2.2.3.1;

2>  the procedure ends.

NOTE:	The UE is only required to acquire broadcasted SIB1 if the UE can acquire it without disrupting unicast data reception, i.e. the broadcast and unicast beams are quasi co-located.

END OF CHANGES

START OF CHANGES

#### –	RRCReconfiguration

The RRCReconfiguration message is the command to modify an RRC connection. It may convey information for measurement configuration, mobility control, radio resource configuration (including RBs, MAC main configuration and physical channel configuration) including and security configuration.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCReconfiguration message

-- ASN1START

-- TAG-RRCRECONFIGURATION-START

RRCReconfiguration ::= 				SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

rrcReconfiguration					RRCReconfiguration-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

RRCReconfiguration-IEs ::= 			SEQUENCE {

radioBearerConfig						RadioBearerConfig 														OPTIONAL, -- Need M

secondaryCellGroup						OCTET STRING (CONTAINING CellGroupConfig)								OPTIONAL, -- Need M

measConfig								MeasConfig																OPTIONAL, -- Need M

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					RRCReconfiguration-v1530-IEs											OPTIONAL

}

RRCReconfiguration-v1530-IEs ::= 	SEQUENCE {

masterCellGroup							OCTET STRING (CONTAINING CellGroupConfig)								OPTIONAL, -- Need M

fullConfig								ENUMERATED {true} 														OPTIONAL, -- Need N

dedicatedNAS-MessageList				SEQUENCE (SIZE(1..maxDRB)) OF DedicatedInfoNAS							OPTIONAL, -- Cond nonHO

masterKeyUpdate							MasterKeyUpdate														OPTIONAL, -- Cond MasterKeyChange

dedicatedSIB1-Delivery					OCTET STRING (CONTAINING SIB1)											OPTIONAL,

dedicatedSystemInformationDelivery		OCTET STRING (CONTAINING SystemInformation)								OPTIONAL,

nonCriticalExtension					RRCReconfiguration-v15xx-IEs																OPTIONAL

}

RRCReconfiguration-v15xx-IEs ::= 	SEQUENCE {

radioBearerConfig2-r15x             OCTET STRING (CONTAINING RadioBearerConfig)									OPTIONAL, -- Need M

mrdc-Release-r15x                   ENUMERATED {true}															OPTIONAL, -- Need N

eutra-SecondaryCellGroup-r15x       OCTET STRING (CONTAINING RRCConnectionReconfiguration)						OPTIONAL, -- Need M

nonCriticalExtension					SEQUENCE {}																OPTIONAL

}

MasterKeyUpdate ::=					SEQUENCE {

keySetChangeIndicator					BOOLEAN,

nextHopChainingCount					NextHopChainingCount,

nas-Container							OCTET STRING													OPTIONAL,	-- Cond securityNASC

...

}

-- TAG-RRCRECONFIGURATION-STOP

-- ASN1STOP

RRCReconfiguration-IEs field descriptions

dedicatedNAS-MessageList This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for each PDU in the list.

fullConfig Indicates that the full configuration option is applicable for the RRCReconfiguration message.

keySetChangeIndicator True is used in an intra-cell handover when a KgNB key is derived from a KAMF key taken into use through the latest successful NAS SMC procedure, or N2 handover procedure with KAMF change, as described in TS 33.501 [11] for KgNB re-keying. False is used in an intra-NR handover when the new KgNB key is obtained from the current KgNB key or from the NH as described in TS 33.501 [11].

masterCellGroup Configuration of master cell group.

nas-Container This field is used to transfer UE specific NAS layer information between the network and the UE. The RRC layer is transparent for this field, although it affects activation of AS- security after inter-system handover to NR. The content is defined in TS 24.501.

mrdc-Release A one-shot field indicating whether the UE releases the stored SCG related configuration (e.g. secondaryCellGroup, eutra-SecondaryCellGroup, SRB3 and SCG measConfig). New SCG configurations can then be configured.

nextHopChainingCount Parameter NCC: See TS 33.501 [11]

radioBearerConfig, radioBearerConfig2 Configuration of Radio Bearers (DRBs, SRBs) including SDAP/PDCP. In EN-DC this field may only be present if the RRCReconfiguration is transmitted over SRB3.

secondaryCellGroup Configuration of NR secondary cell group.

eutra-secondaryCellGroup Includes the LTE RRCConnectionReconfiguration message as specified in TS 36.331 [10]. In this version of the specification, the LTE RRC message only includes fields radioResourceConfigDedicated (with no PDCP configuration present) and/or mobilityControlInfo and/ or measConfig. If eutra-secondaryCellGroup is configured, the network always includes this field upon MN handover to initiate an LTE SCG reconfiguration with sync and key change.

| Conditional Presence | Explanation |
| --- | --- |
| nonHO | The field is not present in case of reconfiguration with sync within NR or to NR; otherwise it is optionally present, need N. |
| securityNASC | This field is mandatory present in case of inter system handover. Otherwise the field is optionally present, need N. |
| MasterKeyChange | This field is mandatory present in case the security algorithms are modified (as indicated in SecurityAlgorithmConfig in SecurityConfig, included in the received RadioBearerConfig). Else if ReconfigurationWithSync is included, this field is optionally present, need N, otherwise the field is absent. |
| FullConfig | It is optionally present, Need N, during reconfiguration with sync and also in first reconfiguration after reestablishment. It is not present otherwise. |

END OF CHANGES