3GPP TSG-RAN WG2 #103bis	R2-1814665

Chengdu, China, 2018-10-08 to 2018-10-12(Revision of R2-1810882)

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0097 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Defining inter-system and intra-system handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT_core |  |  |  |  |  |  | Date: |  | 2018-09-27 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The terms inter-system and intra-system handover are already used in several stage 2/3 documents (e.g. 38.331, 23.501) without being defined properly elsewhere. An attempt was made in RAN2 #AH1801 to define this terms, but this were not agreed upon. This is a revised resubmission of that contribution. In E-UTRA, inter-RAT HO implied intra-system HO and inter-RAT HO implied also inter-system HO (i.e. RAT change implied CN change as well). However, with the introduction of NR and the support of E-UTRA connected to 5GC, several handover types arise, namely: - Intra-RAT, intra-system: HO from E-UTRA connected to EPC to E-UTRA connected to EPC, HO from E-UTRA connected to 5GC to E-UTRA connected to 5GC, HO from NR to NR. - Intra-RAT, inter-system: HO from/to E-UTRA connected to 5GC to/from E-UTRA connected to EPC. - Inter-RAT, intra-system: HO from/to NR to/from E-UTRA connected to 5GC - Inter-RAT, inter-system: HO from/to E-UTRA connected to 5GC to/from NR Impact analysis Impacted functionality: Functionalities that deal with handover Inter-operability: If the network and UE have different understanding of inter-system, intra-system, inter-RAT, intra-RAT handovers, the UE may apply a different behavior than what is expected from the network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definition and description of different types of handovers introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some field descriptions not clear/ambigious, lengthy/repetitive definitions/descriptions will required whenenver there is a need to distinguish the different types of handover if no central/common defintion is available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2 Defintions 9.1 Overview (Mobility and state transitions) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF THE CHANGE

## 3.2	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1], in 3GPP TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1] and 3GPP TS 36.300 [2].

Cell-Defining SSB: an SSB with an RMSI associated.

gNB: node providing NR user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

Intra-system Handover: Handover that does not involve a CN change (i.e. EPC to EPC or 5GC to 5GC, regardless of the source and target RAT)

Inter-system Handover: Handover that involves a CN change (i.e. from EPC to 5GC or vice versa, regardless of the source and target RAT).

MSG1: preamble transmission of the random access procedure.

MSG3: first scheduled transmission of the random access procedure.

ng-eNB: node providing E-UTRA user plane and control plane protocol terminations towards the UE, and connected via the NG interface to the 5GC.

NG-C: control plane interface between NG-RAN and 5GC.

NG-U: user plane interface between NG-RAN and 5GC.

NG-RAN node: either a gNB or an ng-eNB.

Numerology: corresponds to one subcarrier spacing in the frequency domain. By scaling a reference subcarrier spacing by an integer N, different numerologies can be defined.

Xn: network interface between NG-RAN nodes.

END OF THE CHANGE

START OF THE CHANGE

## 9.1	Overview

Load balancing is achieved in NR with handover, redirection mechanisms upon RRC release and through the usage of inter-frequency and inter-RAT absolute priorities and inter-frequency Qoffset parameters.

Measurements to be performed by a UE for connected mode mobility are classified in at least three measurement types:

-	Intra-frequency NR measurements;

-	Inter-frequency NR measurements;

-	Inter-RAT measurements for E-UTRA.

For each measurement type one or several measurement objects can be defined (a measurement object defines e.g. the carrier frequency to be monitored).

For each measurement object one or several reporting configurations can be defined (a reporting configuration defines the reporting criteria). Three reporting criteria are used: event triggered reporting, periodic reporting and event triggered periodic reporting.

The association between a measurement object and a reporting configuration is created by a measurement identity (a measurement identity links together one measurement object and one reporting configuration of the same RAT). By using several measurement identities (one for each measurement object, reporting configuration pair) it is then possible to:

-	associate several reporting configurations to one measurement object and;

-	associate one reporting configuration to several measurement objects.

The measurements identity is as well used when reporting results of the measurements.

Measurement quantities are considered separately for each RAT.

Measurement commands are used by NG-RAN to order the UE to start, modify or stop measurements.

From the CN perspective, a handover can be either intra-system (not involving CN type change) or inter-system (involving CN type change). Both intra-RAT and inter-RAT handovers can be either intra-sytem or inter-system.

The table below summarizes the different types of handover.

Table 9.1-1 Different types of handover.

|  |  | Target |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | E-UTRA connected to EPC | E-UTRA connected to 5GC | NR |  |
| Source | E-UTRA connected to EPC | Intra-system, Intra-RAT | Inter-system, Intra-RAT | Inter-system, Inter-RAT |  |
|  | E-UTRA connected to 5GC | Inter-system, Intra-RAT | Intra-system, Intra-RAT | Intra-system, Inter-RAT |  |
|  | NR | Inter-system, Inter-RAT | Intra-system, Inter-RAT | Intra-system, Intra-RAT |  |

END OF THE CHANGE