3GPP TSG-RAN WG2 #103bis	Tdoc R2-1814772

Chengdu, China, 8th – 12th October 2018

Agenda Item:	10.3.3

Source:	Ericsson

Title:	PDCP handling of RRC suspend

Document for:	Discussion, Decision

# Introduction

In previous meetings, PDCP handling at COUNT reset, i.e. at RRC resume or suspend had been discussed.

The following agreements were made in RAN2#102:

Confirm to support ROHC continue for Handover: both RLC UM bearer and RLC AM bearer and for Resumption: both RLC UM bearer and RLC AM bearer

drb-ROHCContinue should be indicated per PDCP entity.

When count is reset, in transmitter PDCP PDUs are discarded but PDCP SDUs are kept, in receiver stored PDCP PDUs are delivered to higher layer.

When stored UE AS is used at PDCP re-establishment (RRCResume): set state variables to their initial values, and reset t-reordering.

The following agreements were made in RAN2#103:

UP working assumption: RRC provides an indication to PDCP also at RRC suspend (to RRC_Inactive) (Name TBD). When this indication is triggered/received PDCP discards TX side AM PDCP PDUs and SDUs, resets State variables (including COUNT), and deliver RX side stored PDU to upper layer. At PDCP reestablishment (which is assumed to happen at RRC resume) ROHC continue is handled in PDCP. This agreement replaces earlier possibly conflicting agreements

The working assumption taken in RAN2#103 takes a different direction for the PDCP handling, i.e. is triggered based on RRC suspend indication, instead of handling at RRC resume. We analyse this new way of handling PDCP in this contribution.

# Discussion

Compared to previous discussions, with the new UP working assumption, it is envisaged that PDCP handling of the RRC transition to and from inactive is handled in two steps, compared to triggering PDCP behaviour only once. I.e. with the working assumption, it is assumed that:

- PDCP function is triggered at RRC suspend (transition to RRC_Inactive)  New function PDCP suspend

- PDCP function is triggered at RRC resume (transition back to RRC_Connnected)  PDCP re-establishment

Confirm PDCP suspend procedure is triggered at RRC suspend.

Confirm PDCP re-establishment procedure is triggered at RRC resume.

The working assumption furthermore outlines methods to be triggered within the procedures, which we will discuss below. We believe that during RRC suspend, the following general targets should be achieved:

- Count reset, i.e. reset of PDCP state variables

- PDCP entity continuation, i.e. no PDCP release/add i.e. therefore allowing RoHC continuation (if configured)

When PDCP suspend is triggered, no further transmissions shall be done until PDCP is re-establishment, therefore, already at this step, PDCP state variables can be reset, i.e. the count reset. Due to the state variable reset, reordering of potentially retransmitted data at PDCP re-establishment is not possible anymore, therefore also the reordering timer can be reset; all data in the reception buffer still waiting for reordering can thus also be delivered already out of order at this point in time.

Confirm that state variable reset (“count reset”) in transmitting and receiving PDCP entity is triggered at PDCP suspend.

Confirm that t-reordering timer reset and delivery of all stored PDCP SDUs in the receiver is triggered when state variables are reset, i.e. at PDCP suspend.

What remains to be discussed is whether PDCP PDUs and SDUs for AM and UM mode should be discarded at PDCP suspend or PDCP re-establishment.

At PDCP re-establishment, retransmission or transmission is procedure is performed for both AM and UM on PDCP SDU-level, not PDU-level. Therefore, it does not matter for this procedure, whether PDCP PDUs are discarded. Therefore, we propose:

PDCP PDU discard is not triggered at PDCP suspend nor PDCP re-establishment for AM and UM, since has not impact to specification.

PDCP SDUs are buffered IP-packets on PDCP layer. It is unclear whether those packets should be affected by an RRC state transition at all. For UM, PDCP SDUs are transmitted after PDCP re-establishment if not transmitted before; for AM, PDPC SDUs are transmitted/retransmitted if not successfully delivered before. Due to TX_NEXT reset and out of order delivery, AM loses its capability for in order delivery after re-establishment at suspend. However, if SDUs are not discarded they might still be retransmitted (potentially redundantly) at re-establishment, otherwise the PDCP re-establishment for AM becomes lossy. Therefore, we prefer to not discard SDUs during PDCP suspend (nor at PDCP re-establishment). Also, for UM, not discarding PDCP SDUs is beneficial, since it reduces packet loss at PDCP re-establishment.

PDCP SDUs are not discarded at PDCP suspend or PDCP-reestablishment for AM and UM.

It is noteworthy that if PDCP SDUs were discarded, it becomes questionable whey the PDCP suspend procedure is introduced at all, since PDCP release/add would lead to same behaviour – except that RoHC context could be maintained during PDCP suspend, however, with unclear gain.

We provide a PDCP CR [1] and RRC CR [2] implementing those changes.

# Conclusion

We propose the following:

Proposal 1	Confirm PDCP suspend procedure is triggered at RRC suspend.

Proposal 2	Confirm PDCP re-establishment procedure is triggered at RRC resume.

Proposal 3	Confirm that state variable reset (“count reset”) in transmitting and receiving PDCP entity is triggered at PDCP suspend.

Proposal 4	Confirm that t-reordering timer reset and delivery of all stored PDCP SDUs in the receiver is triggered when state variables are reset, i.e. at PDCP suspend.

Proposal 5	PDCP PDU discard is not triggered at PDCP suspend nor PDCP re-establishment for AM and UM, since has not impact to specification.

Proposal 6	PDCP SDUs are not discarded at PDCP suspend or PDCP-reestablishment for AM and UM.

# References

R2-1814771 - Introduction of PDCP suspend to PDCP, Ericsson

R2-1814774 - Introduction of PDCP suspend to RRC, Ericsson