3GPP TSG-RAN WG2#103bis	Tdoc R2-1814783

# Chengdu, PR China, 8th – 12th October 2018                              Revision of R2-1810086

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0238 | rev | 3 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Rebuilding MAC PDU for different grant size in RAR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-26 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The possibility to fall back from CBRA to CFRA may lead to mismatch of the original CBRA grant and the new CFRA grant. This may be handled by the UE by rebuilding the MAC PDU by adding BSR, PHR and/or padding according to the LCP procedure to fit the available grant. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The UL grant size is compared to the MAC PDU size. If the grant size is different than the MAC PDU, the UE rebuilds the MAC PDU to fit the size of the available grant. Impact analysis: Impacted functionality: Random access Interoperability analysis: If the network implements the CR and the UE does not the network will expect the UE to reply to a different grant size which may cause unforeseen behaviour in the UE. - If the UE implements the CR and the network does not no interoperability problem is foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE action when there is mismatch of MAC PDU size and grant size is unclear. It may cause the transmission to fail. It would in this case be handled by RLC retransmission. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 5.4.2.1	HARQ Entity

The MAC entity includes a HARQ entity for each Serving Cell with configured uplink (including the case when it is configured with supplementaryUplink), which maintains a number of parallel HARQ processes.

The number of parallel UL HARQ processes per HARQ entity is specified in TS 38.214 [7].

Each HARQ process supports one TB.

Each HARQ process is associated with a HARQ process identifier. For UL transmission with UL grant in RA Response, HARQ process identifier 0 is used.

When the MAC entity is configured with pusch-AggregationFactor > 1, the parameter pusch-AggregationFactor provides the number of transmissions of a TB within a bundle of the dynamic grant. After the initial transmission, pusch-AggregationFactor – 1 HARQ retransmissions follow within a bundle. When the MAC entity is configured with repK > 1, the parameter repK provides the number of transmissions of a TB within a bundle of the configured uplink grant. After the initial transmission, HARQ retransmissions follow within a bundle. For both dynamic grant and configured uplink grant, bundling operation relies on the HARQ entity for invoking the same HARQ process for each transmission that is part of the same bundle. Within a bundle, HARQ retransmissions are triggered without waiting for feedback from previous transmission according to pusch-AggregationFactor for a dynamic grant and repK for a configured uplink grant, respectively. Each transmission within a bundle is a separate uplink grant after the initial uplink grant within a bundle is delivered to the HARQ entity.

For each transmission within a bundle of the dynamic grant, the sequence of redundancy versions is determined according to subclause 6.1.4 of TS 38.214 [7]. For each transmission within a bundle of the configured uplink grant, the sequence of redundancy versions is determined according to subclause 6.1.2.3 of TS 38.214 [7].

For each uplink grant, the HARQ entity shall:

1>	identify the HARQ process associated with this grant, and for each identified HARQ process:

2>	if the received grant was not addressed to a Temporary C-RNTI on PDCCH, and the NDI provided in the associated HARQ information has been toggled compared to the value in the previous transmission of this TB of this HARQ process; or

2>	if the uplink grant was received on PDCCH for the C-RNTI and the HARQ buffer of the identified process is empty; or

2>	if the uplink grant was received in a Random Access Response; or

2>	if the uplink grant is part of a bundle of the configured uplink grant, and may be used for initial transmission according to subclause 6.1.2.3 of TS 38.214 [7], and if no MAC PDU has been obtained for this bundle:

3>	if there is a MAC PDU in the Msg3 buffer and the uplink grant was received in a Random Access Response:

4>	obtain the MAC PDU to transmit from the Msg3 buffer.

4>  if the uplink grant size does not match with size of the obtained MAC PDU:

5> 	indicate to the Multiplexing and assembly entity to include MAC subPDUs from the obtained MAC PDU in the subsequent uplink transmission(s);

5> 	obtain the MAC PDU to transmit from the Multiplexing and assembly entity.

3>	else:

4>	obtain the MAC PDU to transmit from the Multiplexing and assembly entity, if any;

3>	if a MAC PDU to transmit has been obtained:

4>	deliver the MAC PDU and the uplink grant and the HARQ information of the TB to the identified HARQ process;

4>	instruct the identified HARQ process to trigger a new transmission;

4>	if the uplink grant is addressed to CS-RNTI; or

4>	if the uplink grant is a configured uplink grant; or

4>	if the uplink grant is addressed to C-RNTI, and the identified HARQ process is configured for a configured uplink grant:

5>	start or restart the configuredGrantTimer, if configured, for the corresponding HARQ process when the transmission is performed.

3>	else:

4>	flush the HARQ buffer of the identified HARQ process.

2>	else (i.e. retransmission):

3>	if the uplink grant received on PDCCH was addressed to CS-RNTI and if the HARQ buffer of the identified process is empty; or

3>	if the uplink grant is part of a bundle and if no MAC PDU has been obtained for this bundle; or

3>	if the uplink grant is part of a bundle of the configured uplink grant, and the PUSCH of the uplink grant overlaps with a PUSCH of another uplink grant received on the PDCCH for this Serving Cell:

4>	ignore the uplink grant.

3>	else:

4>	deliver the uplink grant and the HARQ information (redundancy version) of the TB to the identified HARQ process;

4>	instruct the identified HARQ process to trigger a retransmission;

4>	if the uplink grant is addressed to CS-RNTI; or

4>	if the uplink grant is addressed to C-RNTI, and the identified HARQ process is configured for a configured uplink grant:

5>	start or restart the configuredGrantTimer, if configured, for the corresponding HARQ process when the transmission is performed.

When determining if NDI has been toggled compared to the value in the previous transmission the MAC entity shall ignore NDI received in all uplink grants on PDCCH for its Temporary C-RNTI.