3GPP TSG-RAN WG2 #103bis	R2-1814835

Chengdu, China, 8th – 12th October, 2018

Agenda Item: 	10.2.1

Source:	Ericsson

Title:	Overview description of SSB transmission in time domain in TS 38.300

Document for:	Discussion, Decision

# Introduction

The current version of TS 38.300 does not describe how SSBs are transmitted in time domain.

This contribution proposes to include a description in the specification in Stage 2.

# Discussion

In the current version of TS 38.300, the description of transmission of multiple SSBs in time domain is

“The periodicity of the SSB can be configured by the network and the time locations where SSB can be sent are determined by sub-carrier spacing”.

This does not give an overview understanding of how SSBs are transmitted in time domain. In 38.211 and 38.213 an SS-Burst is defined. In 36.331, we have parameters related to SS-Burst and overall in relation to how SSBs are transmitted in time domain such as SMTC configuration or ssb-periodicityServingCell. Without an overview description about the time domain aspect, it would be difficult to have a good understanding on the SSB periodicity related parameters and SS/PBCH block measurement in other RAN2 specifications.

Therefore, we propose the following text description which is included a CR [1].

SSBs transmitted in a half frame [TS 38.211] is defined as SS-Burst. The periodicity of the SS-Burst can be configured by the network and the time locations where SSB can be sent are determined by sub-carrier spacing.

The maximum number of SSBs (L) in a SS-Burst is dependent on frequency location of the SSB burst [TS 38.213]. Table 5.2.4.1 provides a summary.

Table 1: Number of SSBs in a SS-Burst

| Carrier Frequency (GHz) | L |
| --- | --- |
| <3 GHz | 4 |
| 3 - 6 GHz | 8 |
| >6 GHz | 64 |

The above seems simple addition to 38.300 in order to address the time domain aspect of SSB transmission. We propose RAN2 to discuss the necessity of the time domain description as well as how to capture the it in 38.300. We also propose to adopt CR0096 as baseline.

RAN2 to discuss the necessity of the time domain description as well as how to capture the it in 38.300.

Adopt CR0096 as baseline for the discussion.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	RAN2 to discuss the necessity of the time domain description as well as how to capture the it in 38.300.

Proposal 2	Adopt CR0096 as baseline for the discussion.

# References

R2-1814540, Relation between SSB and SS-Burst, Ericsson, RAN2#103bis