3GPP TSG-RAN WG2 Meeting #103bis 	R2-1814850

# Chengdu, China, 08th Oct 2018 – 12th Oct 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0390 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction in 38331 on InitialDownlinkBWP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-10-08 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the last RAN 1 meeting, the following agreement has been made: Agreements: - If a UE is not provided higher layer parameter searchSpaceOtherSystemInformation for Type0A-PDCCH common search space set, the UE determines the Type0A-PDCCH common search space set as the same as Type0-PDCCH common search space set (i.e., default Type0A-PDCCH common search space set). - If a UE is not provided higher layer parameter pagingSearchSpace for Type2-PDCCH common search space set, the UE determines the Type2-PDCCH common search space set as the same as Type0-PDCCH common search space set (i.e., default Type2-PDCCH common search space set). - Separate default Type0A-PDCCH common search space set is not needed. - Separate default Type2-PDCCH common search space set is not needed.. Hence, for initialDownlinkBWP, if search space for paging and osi is not configured, the UE shall use the search space configured for SIB1, i.e., PDCCH-ConfigSIB1 configred in MIB. Based on this, field descriptions for pagingSearchSpace and searchSpaceOtherSystemInformation in PDCCH-ConfigCommon should be corrected. In addition, in the LS R1-1810002, the following agreement has been achieved in the last RAN 1 meeting: Agreements: - Modify the agreements in RAN1#92bis as follows. - When monitoring for DCI in a BWP, the size of DCI format 0-0/1-0 is given by - For format 0-0/1-0 (regardless of RNTI) in CSS, the size is given by the initial DL BWPCORESET#0 - For format 0-0/1-0 in USS, the size is given by the active BWP as long as the DCI size budget is fulfilled - Otherwise, for format 0-0/1-0, the size is given by the initial DL BWPCORESET#0 - For DCI format 1-0 in CSS with P-RNTI, SI-RNTI, RA-RNTI, C-RNTI, CS-RNTI, or TC-RNTI: - the RB numbering for the scheduled PDSCH starts from the lowest RB in the CORESET the DCI is received in - the maximum number of RBs possible to indicate in the DCI is given by the size of the initial DL BWPCORESET#0. - Payload sizes for 2-2 and 2-3 are padded (if needed) to match the size of formats 0-0/1-0 as defined by the initial BWP in CSS - Modify the agreements in RAN1#91 as follows if Option #2 is supported in PCell - CORESET configured by RMSI is confined within the initial DL BWPthe bandwidth of CORESET#0 - For PCell, the initial DL BWP can be configured in SIB1 to be the same as or different with the initial DL BWP as initially defined by CORESET#0 - The initial DL BWP configured in SIB1 includes the bandwidth of CORESET#0 - If the initial DL BWP configured by SIB1 is different with the initial DL BWP as initially defined by CORESET#0, the configuration of the initial DL BWP configured by SIB1 is applicable after the initial access - For PSCell and SCell, the initial DL BWP of a cell can be configured to be the same as or different with the initial DL BWP as defined by CORESET#0 of the cell - The initial DL BWP of a cell includes the bandwidth of CORESET#0 of the cell Note: RAN1 assumes that the above agreements related to PCell, PSCell and SCell have no RAN1 specification impact and will be captured in RAN2 specs. Based on the part highlighted in green, we can make two observations: - The bandwidthof the initial BWP configured by SIB1 should be larger or equal than the bandwidth of CORESET#0 - If the bandwidth of the intial DL BWP configured by SIB 1 is different from that of the initial DL BWP initially defined by CORESET#0, the configuration of the initial DL BWP configured by SIB1 is applicable after the initial access Hence, changes need to be made for the bandwidth configuraiton of BWP. |
| Summary of change: |  | 1/ correction for PDCCH-ConfigCommon for paging and osi search space 2/ Bandwidth for CORESET if servingCellConfigCommon and SIB1 has different configurations. This correction is applicable for both SA and NSA Impact analysis Impacted functionality: PDCCH-ConfigCommon, BWP Inter-operability: - If the network is implemented according to the CR and the UE is not, there will not be issue for the UE and there will not be inter-operability problem. - If the UE is implemented according to the CR and the network is not, the re will not be issue for the UE and there will not be inter-operability problems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Various aspects of the IE configuredGrantConfig will not be correct |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

================================================START OF THE 1ST CHANGE=======================================================

–	PDCCH-ConfigCommon

The IE PDCCH-ConfigCommon is used to configure cell specific PDCCH parameters provided in SIB as well as during handover and PSCell/SCell addition.

PDCCH-ConfigCommon information element

-- ASN1START

-- TAG-PDCCH-CONFIGCOMMON-START

PDCCH-ConfigCommon ::=				SEQUENCE {

controlResourceSetZero				ControlResourceSetZero												OPTIONAL, 	-- Cond InitialBWP-Only

commonControlResourceSet			ControlResourceSet										OPTIONAL, 	-- Need R

searchSpaceZero						SearchSpaceZero											OPTIONAL, 	-- Cond InitialBWP-Only

commonSearchSpaceList				SEQUENCE (SIZE(1..4)) OF SearchSpace								OPTIONAL,	-- Need R

searchSpaceSIB1						SearchSpaceId														OPTIONAL,	-- Need S

searchSpaceOtherSystemInformation	SearchSpaceId														OPTIONAL,	-- Need S

pagingSearchSpace					SearchSpaceId														OPTIONAL, 	-- Need S

ra-SearchSpace						SearchSpaceId														OPTIONAL, 	-- Need S

...

}

-- TAG-PDCCH-CONFIGCOMMON-STOP

-- ASN1STOP

PDCCH-ConfigCommon field descriptions

commonControlResourceSet An additional common control resource setwhich may be configured and used for RAR/paging/system information. If the network configures this field, it uses a ControlResourceSetId other than 0 for this ControlResourceSet.

commonSearchSpaceList A list of additional common search spaces. If the network configures this field, it uses the SearchSpaceIds other than 0.

controlResourceSetZero Parameters of the common CORESET#0. The values are interpreted like the corresponding bits in MIB pdcch-ConfigSIB1. Even though this field is only configured in the initial BWP (BWP#0) controlResourceSetZero can be used in search spaces configured in other DL BWP(s) than the initial DL BWP if the conditions defined in 38.213, section 10 are satisfied.

pagingSearchSpace ID of the Search space for paging. Corresponds to L1 parameter 'paging-SearchSpace' (see 38.213, section 10). If the field is absent, the UE does not receive paging in this BWP(see 38.213, section 10). If the IE is configured for initial DL BWP and the field is absent, the UE shall use the search space for RMSI configured under PDCCH-ConfigSIB1 in MIB for paging reception.

ra-SearchSpace ID of the Search space for random access procedure. Corresponds to L1 parameter 'ra-SearchSpace' (see 38.214?, section FFS_Section) If the field is absent, the UE does not receive RAR in this BWP. This field is mandatory present in the DL BWP(s) if the conditions described in TS 38.321 [3], subclause 5.15 are met.

searchSpaceOtherSystemInformation ID of the Search space for other system information, i.e., SIB2 and beyond. Corresponds to L1 parameter 'osi-SearchSpace' (see 38.213, section 10) If the field is absent, the UE does not receive other system information in this BWP. If the IE is configured for initial DL BWP and the field is absent, the UE shall use the search space for RMSI configured under PDCCH-ConfigSIB1 in MIB for Other System Information reception.

searchSpaceSIB1 ID of the search space for SIB1 message. If the field is absent, the UE does not receive SIB1 in this BWP. (see 38.213, section 10)

searchSpaceZero Parameters of the common SearchSpace#0. The values are interpreted like the corresponding bits in MIB pdcch-ConfigSIB1. Even though this field is only configured in the initial BWP (BWP#0) searchSpaceZero can be used in search spaces configured in other DL BWP(s) than the initial DL BWP if the conditions described in Spec38.213 [13], section 10 are satisfied.

| Conditional Presence | Explanation |
| --- | --- |
| InitialBWP-Only | If SIB1 is broadcast the field is mandatory present in the PDCCH-ConfigCommon of the initial BWP (BWP#0) in dedicated signalling. It is absent in other BWPs and when sent in system information. In other cases, the field is optionally present. |

================================================START OF THE 2ND  CHANGE======================================================

–	BWP

The BWP IE is used to configure a bandwidth part as defined in 38.211, section 4.2.2.

For each serving cell the network configures at least an initial bandwidth part comprising of at least a downlink bandwidth part and one (if the serving cell is configured with an uplink) or two (if using supplementary uplink (SUL)) uplink bandwidth parts. Furthermore, the network may configure additional uplink and downlink bandwidth parts for a serving cell.

The bandwidth part configuration is split into uplink and downlink parameters and into common and dedicated parameters. Common parameters (in BWP-UplinkCommon and BWP-DownlinkCommon) are ”cell specific” and the network ensures the necessary alignment with corresponding parameters of other UEs. The common parameters of the initial bandwidth part of the PCell are also provided via system information. For all other serving cells, the network provides the common parameters via dedicated signalling.

BWP information element

-- ASN1START

-- TAG-BANDWIDTH-PART-START

BWP ::= 							SEQUENCE {

locationAndBandwidth				INTEGER (0..37949),

subcarrierSpacing					SubcarrierSpacing,

cyclicPrefix						ENUMERATED { extended }													OPTIONAL	-- Need R

}

-- TAG-BANDWIDTH-PART-STOP

-- ASN1STOP

BWP field descriptions

cyclicPrefix Indicates whether to use the extended cyclic prefix for this bandwidth part. If not set, the UE uses the normal cyclic prefix. Normal CP is supported for all numerologies and slot formats. Extended CP is supported only for 60 kHz subcarrier spacing. (see 38.211, section 4.2.2)

locationAndBandwidth Frequency domain location and bandwidth of this bandwidth part. The value of the field shall be interpreted as resource indicator value (RIV) as defined TS 38.214 with assumptions as described in TS 38.213, section 12, i.e. setting =275. The first PRB is a PRB determined by subcarrierSpacing of this BWP and offsetToCarrier (configured in SCS-SpecificCarrier contained within FrequencyInfoDL / FrequencyInfoUL) corresponding to this subcarrier spacing. In case of TDD, a BWP-pair (UL BWP and DL BWP with the same bwp-Id) must have the same center frequency (see 38.213, section 12). On PCell, if configured the bandwidth of the initial BWP configured by SIB1 should be equal or larger than the bandwidth of CORESET#0. If the initial DL BWP configured by SIB1 is different with the initial DL BWP as initially defined by CORESET#0, the configuration of the initial DL BWP configured by SIB1 is applicable after the initial access.

subcarrierSpacing Subcarrier spacing to be used in this BWP for all channels and reference signals unless explicitly configured elsewhere. Corresponds to subcarrier spacing according to 38.211, Table 4.2-1. The value kHz15 corresponds to µ=0, kHz30 to µ=1, and so on. Only the values 15, 30, or 60 kHz (<6GHz), and 60 or 120 kHz (>6GHz) are applicable. For the initial DL BWP this field has the same value as the field subCarrierSpacingCommon in MIB of the same serving cell.

================================================END OF CHANGES===============================================================