3GPP TSG-RAN2 Meeting #103bis 	R2-18149112

Chengdu, China, 2018-10-08 to 2018-10-12

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0397 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [E572] Definition of UE AS context Alt1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09-25 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In RAN2 adhoc in June, it was agreed: -> Add a UE variable for the UE AS context that includes the current RRC configuration, the current security context, the PDCP state including ROHC state,C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell (further details of the UE variable can be worked offline) -> How to introduce the UE context variable into the spec can be subject to contribution in the next meeting. In this contribution we attempt to introduce such UE variable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE variable Var-UE-AS-Context introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.3, 5.3.8.3, 5.3.13,7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 5.3.3.4	Reception of the RRCSetup by the UE

The UE shall perform the following actions upon reception of the RRCSetup:

1>	if the RRCSetup is received in response to an RRCReestablishmentRequest; or

1>	if the RRCSetup is received in response to an RRCResumeRequest or RRCResumeRequest1:

2> discard the stored UE AS context in Var-UE-AS-Context, fullI-RNTI and shortI-RNTI;

2>	indicate to upper layers fallback of the RRC connection;

1> perform the cell group configuration procedure in accordance with the received masterCellGroup and as specified in 5.3.5.5;

1>	perform the radio bearer configuration procedure in accordance with the received radioBearerConfig and as specified in 5.3.5.6;

1>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

1>	stop timer T300, T301 or T319 if running;

Editor’s Note: FFS Whether there is a need to define UE actions related to access control timers (equivalent to T302, T303, T305, T306, T308 in LTE). For example, informing upper layers if a given timer is not running.

1>	stop timer T320, if running;

1>	if the RRCSetup is received in response to an RRCResumeRequest or RRCSetupRequest:

2>	enter RRC_CONNECTED;

2>	stop the cell re-selection procedure;

1>	consider the current cell to be the PCell;

1>	set the content of RRCSetupComplete message as follows:

2> if upper layers provide an 5G-S-TMSI:

3> if the RRCSetup is received in response to an RRCSetupRequest:

4>  set the ng-5G-S-TMSI-Value to ng-5G-S-TMSI-Part2;

3> else:

4>  set the ng-5G-S-TMSI-Value to ng-5G-S-TMSI;

2>	set the selectedPLMN-Identity to the PLMN selected by upper layers (TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1;

2>	if the masterCellGroup contains the reportUplinkTxDirectCurrent:

3> include the uplinkTxDirectCurrentList;

2>	if upper layers provide the 'Registered AMF':

3> include and set the registeredAMF as follows:

4>	if the PLMN identity of the 'Registered AMF' is different from the PLMN selected by the upper layers:

5>	include the plmnIdentity in the registeredAMF and set it to the value of the PLMN identity in the 'Registered AMF' received from upper layers;

4>	set the amf-Identifier to the value received from upper layers;

3>	include and set the guami-Type to the value provided by the upper layers;

Editor’s Note: FFS Confirm whether the guami-Type is included and set in the abovementioned condition.

2>	if upper layers provide one or more S-NSSAI (see TS 23.003 [20]):

3>	include the s-nssai-List and set the content to the values provided by the upper layers;

2>	set the dedicatedNAS-Message to include the information received from upper layers;

1>	submit the RRCSetupComplete message to lower layers for transmission, upon which the procedure ends

#### 5.3.8.3	Reception of the RRCRelease by the UE

The UE shall:

1>	delay the following actions defined in this sub-clause 60ms from the moment the RRCRelease message was received or optionally when lower layers indicate that the receipt of the RRCRelease message has been successfully acknowledged, whichever is earlier;

1>	stop timer T320, if running;

1>	if the RRCRelease message includes redirectedCarrierInfo indicating redirection to eutra:

2>	if cnType is included:

3>	 the received cnType is provided to upper layers;

NOTE:	Handling the case if the E-UTRA cell selected after the redirection does not support the core network type specified by the cnType, is up to UE implementation.

1>	if the RRCRelease message includes the cellReselectionPriorities:

2>	store the cell reselection priority information provided by the cellReselectionPriorities;

2>	if the t320 is included:

3>	start timer T320, with the timer value set according to the value of t320;

1>	else:

2>	apply the cell reselection priority information broadcast in the system information;

1>	if deprioritisationReq is included:

2>	start or restart timer T325 with the timer value set to the deprioritisationTimer signalled;

2>	store the deprioritisationReq until T325 expiry;

1>	if the RRCRelease includes suspendConfig:

2> apply the received suspendConfig;

2> store fullI-RNTI, shortI-RNTI, nextHopChainingCount, t380 and ran-PagingCycle provided in suspendConfig;

2>	reset MAC;

2>	re-establish RLC entities for SRB1;

2>	if the RRCRelease message with suspendConfig was received in response to an RRCResumeRequest or an RRCResumeRequest1:

3> stop the timer T319 if running;

3> replace any previously stored security context with newly received security context in the suspendConfig;

3> replace the previously stored C-RNTI with the temporary C-RNTI in the cell the UE has received the RRCRelease message;

3> replace the previously stored cellIdentity with the cellIdentity of the cell the UE has received the RRCRelease message;

3> replace the previously stored physical cell identity with the physical cell identity of the cell the UE has received the RRCRelease message;

2>	else:

3> store the UE AS Context within Var-UE-AS-Context including the current RRC configuration, the current security context, the PDCP state including ROHC state, SDAP configuration, C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell;

2>	suspend all SRB(s) and DRB(s), except SRB0;

2>	start timer T380, with the timer value set to t380;

2>	indicate the suspension of the RRC connection to upper layers;

2> enter RRC_INACTIVE and perform procedures as specified in TS 38.304 [21]

1>	else

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with the release cause ‘other’.

### 5.3.13	RRC connection resume

#### 5.3.13.2	Initiation

The UE initiates the procedure when upper layers or AS (when responding to NG-RAN paging or upon triggering RNA updates while the UE is in RRC_INACTIVE) requests the resume of a suspended RRC connection.

Upon initiation of the procedure, the UE shall:

Editor’s Note: FFS Whether SCG configuration should be released or whether that should be treated as any other configuration (i.e. with delta signalling).

1>	if the upper layers provide an Access Category and one or more Access Identities upon requesting the resumption of an RRC connection:

2>	perform the unified access control procedure as specified in 5.3.14 using the Access Category and Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	if the resumption of the RRC connection is triggered due to an RNA update:

2>	if an emergency service is ongoing:

NOTE: 	How the RRC layer in the UE is aware of an ongoing emergency service is up to UE implementation.

3>	select '2' as the Access Category;

2>	else:

3>	select [the standardised RAN specific access category] as the Access Category;

Editor’s note: Which value to use for the standardised RAN specific access category needs to be confirmed by SA1.

2>	perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities provided by upper layers;

3>	if the access attempt is barred:

4>	set the variable pendingRnaUpdate to 'TRUE';

4>	the procedure ends;

1>	if the resumption of the RRC connection is triggered by response to NG-RAN paging:

2>	select '0' as the Access Category;

2>	perform the unified access control procedure as specified in 5.3.14 using the selected Access Category and one or more Access Identities provided by upper layers;

3>	if the access attempt is barred, the procedure ends;

1>	release the current dedicated Serving Cell configuration and apply the specified values in corresponding specification except for the parameters for which values are provided in SIB1;

1>	apply the default MAC Cell Group configuration as specified in 9.2.x1;

1>	release delayBudgetReportingConfig, if configured and stop timer T3xx, if running;

1>	apply the CCCH configuration as specified in 9.1.1.x2;

1>	apply the timeAlignmentTimerCommon included in SIB1;

1>	start timer T319;

1>	stop timer T380, if running;

1>	initiate transmission of the RRCResumeRequest message or RRCResumeRequest1 in accordance with 5.3.13.3.

Editor’s Note: FFS Requirements on up to date system information acquisition before connection resumption.

#### 5.3.13.3	Actions related to transmission of RRCResumeRequest or RRCResumeRequest1 message

The UE shall set the contents of RRCResumeRequest or RRCResumeRequest1 message as follows:

1>	if field useFullResumeID is signalled in SIB1:

2>	select RRCResumeRequest1 as the message to use;

2>	set the resumeIdentity to the stored fullI-RNTI value;

1>	else:

2>	select RRCResumeRequest as the message to use;

2>	set the shortResumeIdentity to the stored shortI-RNTI value;

1>	set the resumeCause in accordance with the information received from upper layers or from AS layer;

1>	set the resumeMAC-I to the 16 least significant bits of the MAC-I calculated:

2>	over the ASN.1 encoded as per section 8 (i.e., a multiple of 8 bits) VarResumeMAC-Input;

2>	with the KRRCint key and the previously configured integrity protection algorithm; and

2>	with all input bits for COUNT, BEARER and DIRECTION set to binary ones;

Editor’s Note: FFS Additional input to VarResumeMAC-Input (replay attacks mitigation).

1>	restore the RRC configuration and security context from the Var-UE-AS-Context except the cellGroupConfig;

1>	update the KgNB key based on the current KgNB or the NH, using the stored nextHopChainingCount value, as specified in TS 33.501 [11];

1>	derive the KRRCenc key, the KRRCint, the KUPint key and the KUPenc key;

1>	configure lower layers to apply integrity protection for all radio bearers except SRB0 using the previously configured algorithm and the KRRCint key and KUPint key immediately, i.e., integrity protection shall be applied to all subsequent messages received and sent by the UE;

NOTE 1:	Only DRBs with previously configured UP integrity protection shall resume integrity protection.

1>	configure lower layers to apply ciphering for all radio bearers except SRB0 and to apply the previously configured ciphering algorithm, the KRRCenc key and the KUPenc key, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE;

1>	restore the PDCP state and re-establish PDCP entities for SRB1;

1>	resume SRB1;

1> submit the selected message RRCResumeRequest or RRCResumeRequest1 for transmission to lower layers.

NOTE 2:	Only DRBs with previously configured UP ciphering shall resume ciphering.

If lower layers indicate an integrity check failure while T319 is running, perform actions specified in 5.3.13.5.

The UE shall continue cell re-selection related measurements as well as cell re-selection evaluation. If the conditions for cell re-selection are fulfilled, the UE shall perform cell re-selection as specified in 5.3.13.6.

#### 5.3.13.4	Reception of the RRCResume by the UE

The UE shall:

1>	stop timer T319;

1>	if the RRCResume includes the fullConfig:

2>	perform the full configuration procedure as specified in 5.3.5.11;

1>	else:

2>	restore the PDCP state and reset COUNT value for SRB2 and all DRBs;

2>	restore the cellGroupConfig from Var-UE-AS-Context;

2>	indicate to lower layers that stored UE AS context is used;

1>	discard the fullI-RNTI, shortI-RNTI and the Var-UE-AS-Context, except ran-NotificationAreaInfo;

1>	if the RRCResume includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

Editor’s Note: FFS Whether it is supported to configure secondaryCellGroup at Resume.

1>	if the RRCResume includes the radioBearerConfig:

2>	perform the radio bearer configuration according to 5.3.5.6;

Editor’s Note: FFS Whether there needs to be a second radioBearerConfig.

1>	resume SRB2 and all DRBs;

1>	if stored, discard the cell reselection priority information provided by the cellReselectionPriorities or inherited from another RAT;

1> stop timer T320, if running;

1>	if the RRCResume message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

1>	resume measurements if suspended;

Editor’s Note: FFS Whether there is a need to define UE actions related to access control timers (equivalent to T302, T303, T305, T306, T308 in LTE). For example, informing upper layers if a given timer is not running.

1>	enter RRC_CONNECTED;

1>	indicate to upper layers that the suspended RRC connection has been resumed;

1>	stop the cell re-selection procedure;

1>	consider the current cell to be the PCell;

1>	set the content of the of RRCResumeComplete message as follows:

2>  if the upper layer provides NAS PDU, set the dedicatedNAS-Message to include the information received from upper layers;

2> if the upper layer provides a PLMN, set the selectedPLMN-Identity to PLMN selected by upper layers (TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1;

2>	if the masterCellGroup contains the reportUplinkTxDirectCurrent:

3> include the uplinkTxDirectCurrentList;

1>	submit the RRCResumeComplete message to lower layers for transmission;

1>	the procedure ends.

## 7.4	UE variables

NOTE: 	To facilitate the specification of the UE behavioural requirements, UE variables are represented using ASN.1. Unless explicitly specified otherwise, it is however up to UE implementation how to store the variables. The optionality of the IEs in ASN.1 is used only to indicate that the values may not always be available.

#### –	Var-UE-AS-Context

The UE variable Var-UE-AS-Context includes current RRC configuration, the current security context, the PDCP state including ROHC state, C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell.

Var-UE-AS-Context UE variable

-- ASN1START

-- TAG-VAR-MEAS-CONFIG-START

Var-UE-AS-Context ::=					SEQUENCE {

-- RRC configuration

rrcConfig							RRCReconfiguration					OPTIONAL,

-- security context including current KgNB

securityContext						OCTETSTRING							OPTIONAL,

-- ROCH context and state. Up to UE implementation how to present ROCH context and state.

rohcState 							OCTETSTRING							OPTIONAL,

-- Cell identity of the source

cellIdentitySource					CellIdentity						OPTIONAL,

-- Physiscal cell identity of the source

pciSource							physCellId							OPTIONAL,

} 																		OPTIONAL

}

-- TAG-VAR-MEAS-CONFIG-STOP

-- ASN1STOP