3GPP TSG-RAN WG2 Meeting #103bis	     R2-1814923

Chengdu, PRC, 8-12 October 2018

Agenda Item:	10.4.1.3.7

Source:	Ericsson

Title:	RRC Connection release triggered by upper layers [H299]

Document for: Decision

# 1	Discussion

In R2-1812335 (H299) it is proposed to add another example of cases when the RRC connection could be released by upper layers e.g. The UE shall trigger a transition from 5GMM-CONNECTED mode with RRC inactive indication to 5GMM-IDLE mode upon selection of a PLMN that is not an equivalent PLMN to the registered PLMN.

In our view it does not make sense to try to list all cases this could happen in 24.501. e.g. first it might be difficult to find all cases today, and then cases can also be added in the future. If it a problem to have only one example as now, it would be better to remove that example and only refer to 24.501.

Keep text as is in section 5.3.9.1 (do not add text from H299).

If proposal 1 is not agreed, remove all examples from the note in 5.3.9.1 and only refer to 24.501.

# Proposed change in R2-1812335 (H299)

### 5.3.9 RRC connection release requested by upper layers

#### 5.3.9.1  General

The purpose of this procedure is to release the RRC connection. Access to the current PCell may be barred as a result of this procedure.

NOTE:      Upper layers invoke the procedure, e.g. upon determining that the network has failed an authentication check, or trigger a transition from 5GMM-CONNECTED mode with RRC inactive indication to 5GMM-IDLE mode upon selection of a PLMN that is not an equivalent PLMN to the registered PLMN see TS 24.501 [23].

#### 5.3.9.2  Initiation

The UE initiates the procedure when upper layers request the release of the RRC connection. The UE shall not initiate the procedure for power saving purposes.

The UE shall:

1>  if the upper layers indicate barring of the PCell:

2>  treat the PCell used prior to entering RRC_IDLE as barred according to TS 38.304 [20];

1>  perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other';