3GPP TSG-RAN WG2 Meeting #103bis	R2-1814935

# Chengdu, China, 08 - 12 October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0103 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to beam failure detection in Stage-2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rapporteur (Nokia), Qualcomm, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2018-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For beam failure detection, a beamFailureDetectionTimer is configured to manage the period based on which beam failure is declared. Current TS 38.300 defines that as a configured period but rather the period is floating based on the the frequency of beam failure instance indications from L1 and the configured timer value. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is explained that the UE declares beam failure when the number of beam failure instance indications from the physical layer reaches a configured threshold before a configured timer expiry. Impact analysis Impacted functionality: Beam Failure Detection. Inter-operability: - If the network is implemented according to the CR and the UE is not, no interoperability issues are foreseen. - If the UE is implemented according to the CR and the network is not, no interoperability issues are foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The beam failure detection procedure may not be correctly captured in the specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

### 9.2.8	Beam failure detection and recovery

For beam failure detection, the gNB configures the UE with beam failure detection reference signals and the UE declares beam failure when the number of beam failure instance indications from the physical layer reaches a configured threshold before a configured timer expiry.

After beam failure is detected, the UE:

-	triggers beam failure recovery by initiating a Random Access procedure on the PCell;

-	selects a suitable beam to perform beam failure recovery (if the gNB has provided dedicated Random Access resources for certain beams, those will be prioritized by the UE).

Upon completion of the Random Access procedure, beam failure recovery is considered complete.

End of Modified Subclause