3GPP TSG-RAN WG2 Meeting #103bis	R2-1814986

# Chengdu, China, 08 - 12 October 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0425 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TCI states configuration for CORESET#0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-09 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The usage of tci-StatesPDCCH-ToAddList in Control Resource Set configuration is now unclear as the MAC CE for TCI state indication for PDCCH is referring to the TCI states, which are defined in tci-StatesToAddModList in PDSCH configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify that tci-StatesPDCCH-ToAddList is only used to configure a single TCI state to be used for PDCCH. In case no TCI state is provided within this field, UE follows an indication of MAC CE. Impact analysis Impacted functionality: TCI states confguration for PDCCH. Inter-operability: - If the network is implemented according to the CR and the UE is not there is interoperability issue (UE may not be able to follow the TCI state indicated with MAC CE). - If the UE is implemented according to the CR and the network is not there is interoperability issue (the network may attempt to configure the UE with more than one entries in tci-StatesPDCCH-ToAddList, which hwould be deemed error configuration by the UE). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how the TCI states for PDCCH can be indicated to the UE, including indication for CORESET#0. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS 38.321 CR 0489 |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

### 6.3.2	Radio resource control information elements

<UNCHANGED TEXT OMITTED>

#### –	ControlResourceSet

The IE ControlResourceSet is used to configure a time/frequency control resource set (CORESET) in which to search for downlink control information (see 38.213, section FFS_Section).

ControlResourceSet information element

-- ASN1START

-- TAG-CONTROLRESOURCESET-START

ControlResourceSet ::=                  SEQUENCE {

controlResourceSetId                    ControlResourceSetId,

frequencyDomainResources                BIT STRING (SIZE (45)),

duration                                INTEGER (1..maxCoReSetDuration),

cce-REG-MappingType                     CHOICE {

interleaved                             SEQUENCE {

reg-BundleSize                          ENUMERATED {n2, n3, n6},

interleaverSize                         ENUMERATED {n2, n3, n6},

shiftIndex                              INTEGER(0..maxNrofPhysicalResourceBlocks-1)                      OPTIONAL   -- Need S

},

nonInterleaved                          NULL

},

precoderGranularity                     ENUMERATED {sameAsREG-bundle, allContiguousRBs},

tci-StatesPDCCH-ToAddList                   SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId               OPTIONAL,   -- Need N

tci-StatesPDCCH-ToReleaseList           SEQUENCE(SIZE (1..maxNrofTCI-StatesPDCCH)) OF TCI-StateId               OPTIONAL,   -- Need N

tci-PresentInDCI                        ENUMERATED {enabled}                                                    OPTIONAL,   -- Need S

pdcch-DMRS-ScramblingID                 INTEGER (0..65535)                                                  OPTIONAL,   -- Need S

...

}

-- TAG-CONTROLRESOURCESET-STOP

-- ASN1STOP

ControlResourceSet field descriptions

cce-REG-MappingType Mapping of Control Channel Elements (CCE) to Resource Element Groups (REG). Corresponds to L1 parameter 'CORESET-CCE-REG-mapping-type' (see 38.211Section sections 7.3.2.2 and 7.4.1.3.2).

controlResourceSetId Corresponds to L1 parameter 'CORESET-ID'. Value 0 identifies the common CORESET configured in MIB and in ServingCellConfigCommon (controlResourceSetZero) and is hence not used here in the ControlResourceSet IE. Values 1..maxNrofControlResourceSets-1 identify CORESETs configured by dedicated signalling or in SIB1. The controlResourceSetId is unique among the BWPs of a ServingCell.

duration Contiguous time duration of the CORESET in number of symbols. Corresponds to L1 parameter 'CORESET-time-duration' (see 38.211, section 7.3.2.2FFS_Section)

frequencyDomainResources Frequency domain resources for the CORESET. Each bit corresponds a group of 6 RBs, with grouping starting from the first RB group (see 38.213, section 10.1) in the BWP. The first (left-most / most significant) bit corresponds to the first RB group in the BWP, and so on. A bit that is set to 1 indicates that this RB group belongs to the frequency domain resource of this CORESET. Bits corresponding to a group of RBs not fully contained in the bandwidth part within which the CORESET is configured are set to zero. Corresponds to L1 parameter 'CORESET-freq-dom' (see 38.211, section 7.3.2.2).

interleaverSize Corresponds to L1 parameter 'CORESET-interleaver-size' (see 38.211, 38.213, section FFS_Section).

pdcch-DMRS-ScramblingID PDCCH DMRS scrambling initialization. Corresponds to L1 parameter 'PDCCH-DMRS-Scrambling-ID' (see 38.211, section 7.4.1). When the field is absent the UE applies the value of the physCellId configured for this serving cell.

precoderGranularity Precoder granularity in frequency domain. Corresponds to L1 parameter 'CORESET-precoder-granuality' (see 38.211, sections 7.3.2.2 and 7.4.1.3.2).

reg-BundleSize Resource Element Groups (REGs) can be bundled to create REG bundles. This parameter defines the size of such bundles. Corresponds to L1 parameter 'CORESET-REG-bundle-size' (see 38.211, section FFS_Section).

shiftIndex Corresponds to L1 parameter 'CORESET-shift-index'. When the field is absent the UE applies the value of the physCellIdconfigured for this serving cell (see 38.211, section 7.3.2.2).

tci-PresentInDCI If at least spatial QCL is configured/indicated, this field indicates if TCI field is present or not present in DL-related DCI. When the field is absent the UE considers the TCI to be absent/disabled. Corresponds to L1 parameter 'TCI-PresentInDCI' (see 38.214, section 5.1.5).

tci-StatesPDCCH-ToAddList, tci-StatesPDCCH-ToReleaseList This field is used for providing QCL relationship between the DL RS(s) in one RS Set (TCI-State) and the PDCCH DMRS ports. The list contains a maximum of 1 entry and in case no entries are provided, UE derives QCL relationship based on TCI State Indication for UE-specific PDCCH MAC CE (see 38.321, section 6.1.3.15). A TCI state indicated with this feld is one of of the TCI states defined in pdsch-Config. Corresponds to L1 parameter 'TCI-StatesPDCCH' (see 38.213, section10.).

End of changes