3GPP TSG-RAN WG2 Meeting #103bis	R2-1815002

Chengdu, China, 8th – 12th October 2018

Agenda Item: 	10.3.1.8

Source:	Apple, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated

Title:	Further clarification on LCH-to-cell restriction

Document for:		Discussion and Decision

# Introduction

In RAN2#103 meeting, LCH-to-cell restriction was further clarified for the non-duplicated bearer, EN-DC DC-duplicated bearer and CA-duplicated bearer, and following agreements were achieved.

The LCH-to-cell restriction can apply on the LCH in the following cases: Non-duplicated bearer; NR leg of EN-DC split bearer; FFS if applicable to NR leg of EN-DC DC-duplicated bearer, for both duplication activated and deactivated state. Restriction applies for CA duplication activated state and when CA duplication is not configured. Restriction does not apply for CA duplication deactivated state.

This contribution gives further clarification on DC-duplicated bearer and CA-duplicated bearer.

# Discussion

- Motivation to introduced the LCH-to-cell restriction

There are two motivations:

- Motivation 1: for CA duplication case

  - When CA duplication is activated, applying the restriction is to avoid the duplicated packets transmitting on the same serving cell;

  - When CA duplication is deactivated, lifting the restriction is to maximum the resource ulitization for the primary leg’s data transmission.

- Motivation 2: for non-duplication case

  - Since different serving cells can provide different QoS transmission garantuees, applying the restriction can link the service to the serving cells that can meet its QoS requirement.

  - For some special service (e.g. MDBV), NW would like to only map this service to a cell, as a way to isolate it.

Figure-1. LCH-to-cell restriction in CA-duplication and non-duplication

- Case 1: EN-DC DC-duplicated bearer

For EN-DC DC duplication, the serving cells for two links are always isolated. Hence, the motivation to configure the restriction for DC-duplicated bearer is different from that in CA duplication case (motivation#1), but same as that in non-duplication case (motivation#2).

As shown in Figure-2, if the restriction is lifted in NR-leg when duplication is deactivated, the DC-duplicated bearer in NR-leg will be mapped on Cell#3 which cannot garantuee its QoS requirement. Hence, for DC-duplicated bearer, the restriction should be applied on the LCH regardless of the duplication activation or not.

Proposal 1: For EN-DC DC-duplicated bearer, LCH-to-cell restriction should be applied on the LCH regardless of the duplication activated or not.

Figure-2. LCH-to-cell restriction in DC-duplication

- Case 2: CA-duplicated bearer

For CA duplication, it is possible that UE is configured with different serving cells with different QoS garantuees. In the example shown in Figure-3, if the restriction is lifted when CA duplication is deactivated, the DRB#1 with QoS#1 requirement will be transmitted on all three cells, including the cell#3 which cannot meet the QoS#1 requirement.

In addition, if NW would like to only map MDBV service (DRB#2) on cell#3, if we allow the DRB#1 mapping on cell#3 when CA duplicaton is deactivated, it will impact the NW scheduling on MDBV service.

According to the two motivations, the DRB#1 should only mapped on cell#1 and cell#2.

Proposal 2: For CA-duplicated bearer, when CA dupliation is deactivated, the logical channel restriction of the logical channels should be the combination of the restrictions configured for both the primary and secondary logical channels.

Figure-3. LCH-to-cell restriction in CA-duplication

# Conclusion

Based on the discussion in section 2, we propose

Proposal 1: For EN-DC DC-duplicated bearer, LCH-to-cell restriction should be applied on the LCH regardless of the duplication activated or not.

Proposal 2: For CA-duplicated bearer, when CA dupliation is deactivated, the logical channel restriction of the logical channels should be the combination of the restrictions configured for both the primary and secondary logical channels.

The corresponding 38.300 CR and 38.321 CR are provided in [1][2].

# Reference

R2-1815003	Clarification on LCH-to-cell restriction (38.321 CR)	Apple, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated

R2-1815004	Clarification on LCH-to-cell restriction (38.300 CR) 	Apple, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated