3GPP TSG-RAN WG2 #103bis	R2-1815130

# Chengdu, China, October 8 – October 12, 2018

Agenda Item: 10.3.3

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.323 | CR | 0021 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing PDCP suspend procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-10-08 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - RAN2#103 made following agreements. The new “PDCP suspend” procedure should be included in the PDCP specification. UP working assumption: RRC provides an indication to PDCP also at RRC suspend (to RRC_Inactive) (Name TBD). When this indication is triggered/received PDCP discards TX side AM PDCP PDUs and SDUs, resets State variables (including COUNT), and deliver RX side stored PDU to upper layer. At PDCP reestablishment (which is assumed to happen at RRC resume) ROHC continue is handled in PDCP. This agreement replaces earlier possibly conflicting agreements. - For UM DRBs and AM DRBs, the stored PDCP PDUs should be discarded at PDCP re-establishment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - A new section is introduced to describe PDCP behavior at RRC Suspend. - It is specified for UM DRBs and AM DRBs that the PDCP entity discard PDCP PDUs at PDCP re-establishment. Impact analysis Impacted functionality: PDCP suspend Inter-operability: If the network is implemented according to the CR while the UE is not, the UE PDCP entity is not suspended. If the UE is implemented according to the CR while the network is not, no interoperability problems are foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE PDCP entity is not suspended at RRC Suspend, which results in delayed delivery of stored PDCP SDUs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2, 5.1.4(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS38.331 |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of changes

5.1.2	PDCP entity re-establishment

When upper layers request a PDCP entity re-establishment, the UE shall additionally perform once the procedures described in this section. After performing the procedures in this section, the UE shall follow the procedures in subclause 5.2.

When upper layers request a PDCP entity re-establishment, the transmitting PDCP entity shall:

-	for UM DRBs and AM DRBs, reset the header compression protocol for uplink and start with an IR state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and SRBs, set TX_NEXT to the initial value;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	for UM DRBs and AM DRBs, discard all stored PDCP PDUs;

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	for UM DRBs, for each PDCP SDU already associated with a PDCP SN but for which a corresponding PDU has not previously been submitted to lower layers:

-	consider the PDCP SDUs as received from upper layer;

-	perform transmission of the PDCP SDUs in ascending order of the COUNT value associated to the PDCP SDU prior to the PDCP re-establishment without restarting the discardTimer, as specified in subclause 5.2.1;

-	for AM DRBs, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below:

-	perform header compression of the PDCP SDU as specified in the subclause 5.7.4;

-	perform integrity protection and ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the subclause 5.9 and 5.8;

-	submit the resulting PDCP Data PDU to lower layer, as specified in subclause 5.2.1.

When upper layers request a PDCP entity re-establishment, the receiving PDCP entity shall:

-	process the PDCP Data PDUs that are received from lower layers due to the re-establishment of the lower layers, as specified in the subclause 5.2.2.1;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	for SRBs and UM DRBs, if t-Reordering is running:

-	stop and reset t-Reordering;

-	for UM DRBs, deliver all stored PDCP SDUs to the upper layers in ascending order of associated COUNT values after performing header decompression;

-	for AM DRBs, perform header decompression for all stored PDCP SDUs if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and AM DRBs, reset the header compression protocol for downlink and start with NC state in U-mode (as defined in RFC 3095 [8] and RFC 4815 [9]) if drb-ContinueROHC is not configured in TS 38.331 [3];

-	for UM DRBs and SRBs, set RX_NEXT and RX_DELIV to the initial value;

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure.

Start of next changes

5.1.4	PDCP entity suspend

This procedure is applicable to AM DRBs.

When upper layers request a PDCP entity suspend, the transmitting PDCP entity shall:

-	set TX_NEXT to the initial value;

-	discard all stored PDCP SDUs and PDUs;

When upper layers request a PDCP entity suspend, the receiving PDCP entity shall:

-	if t-Reordering is running:

-	stop and reset t-Reordering;

-	deliver all stored PDCP SDUs to the upper layers in ascending order of associated COUNT values after performing header decompression;

-	set RX_NEXT and RX_DELIV to the initial value.

End of changes