3GPP TSG-RAN WG2 Meeting#103bis	R2-1815609

# Chengdu, China, October 8-12, 2018											  Revision of R2-1813801

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 3571 | rev | 1 | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for SPS configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eV2X-Core |  |  |  |  |  |  | Date: |  | 2018-07-09 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the current version of TS 36.331, - In sps-Config(-r12) and sps-Config-v1530, both SPS C-RNTI based DL and UL SPS (single or multiple) configuration are included; - In sps-Config-v1430, both SPS V-RNTI based SL and UL SPS (multiple) configuration are included; The condition of SPS/SPS2/SPS3 / SPS4 restrict the co-existence of sps-Config-v1430 and sps-Config(-r12)/sps-Config-v1530, i.e., either sps-Config-v1430 or sps-Config(-r12)/sps-Config-v1530 can be configured, but not simultaneously two of them. Considering DL SPS configuration only exist in sps-Config(-r12)/sps-Config-v1530 but not in sps-Config-v1430, the condition (SPS/SPS2/SPS3 / SPS4) causes that one cannot configure - SPS C-RNTI based DL SPS and SPS V-RNTI based UL SPS at the same time. - SPS C-RNTI based DL SPS and SPS V-RNTI based SL SPS at the same time. This is not reasonable, since simultaneous operation of SPS C-RNTI based DL SPS scheduling and SPS V-RNTI based UL and SL SPS scheduling is allowed by TS 36.302 (Combination 2.7a/2.7b in Table 8.2-2). |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR clarify in SPS/SPS2 that sps-Config-v1430 can be present together with sps-Config(-r12)/sps-Config-v1530 as long as UL SPS is configured either in sps-Config(-r12)/sps-Config-v1530 or in sps-Config-v1430, but not both. (It is found that in the final version of TS 36.331 V15.3.0, the condition of SPS3/SPS4 is removed from ASN.1 definition but still kept in in condition description. Therefore, there would be discussion in HRLLC WI on how to handle the SPS3/SPS4 condition, and thus it would be helpful if the CR in eV2X WI is separated from the SPS3/SPS4 issue, to avoid cross-WI collision, as recommended by HRLLC WI rapporteur. It is the reason why the change in r1 version is only in SPS/SPS2 condition but not in SPS3/SPS4 condition.) Impact analysis Impacted functionality: SPS configuration Inter-operability: If the UE is implemented according to this CR but the network is not, there is no inter-operability problem. If the network is implemented according to this CR but the UE is not, there is inter-operability problem. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | One cannot configure - SPS C-RNTI based DL SPS and SPS V-RNTI based UL SPS at the same time. - SPS C-RNTI based DL SPS and SPS V-RNTI based SL SPS at the same time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

| Conditional presence | Explanation |
| --- | --- |
| UL-LWA | The field is optionally present, need ON if ul-LWA-Config-r14 is present. Otherwise the field is not present. |
| CRSIM | The field is optionally present, need ON, if neighCellsCRS-Info-r11 is not present; otherwise it is not present. |
| DRB-Setup | The field is mandatory present if the corresponding DRB is being set up and the UE is connected to EPC; otherwise it is not present. |
| DRB-SetupM | The field is mandatory present upon setup of MCG or split DRB; The field is optionally present, Need ON, upon change from SCG to MCG DRB, for EN-DC upon bearer type change of MCG RLC bearer with key change (i.e. bearer type change from MCG/split to MCG/split bearer with key change); otherwise it is not present. |
| DRB-SetupS | The field is mandatory present upon setup of SCG or split DRB, or upon change from MCG to split DRB; The field is optionally present, Need ON, upon change from MCG to SCG DRB; otherwise it is not present. |
| HO-Conn | The field is mandatory present in case of handover to E-UTRA or when the fullConfig is included in the RRCConnectionReconfiguration message or in case of RRC connection establishment (excluding RRConnectionResume); otherwise the field is optionally present, need ON. Upon connection establishment/ re-establishment only SRB1 is applicable (excluding RRConnectionResume). |
| HO-toEUTRA | The field is mandatory present in case of handover to E-UTRA or when the fullConfig is included in the RRCConnectionReconfiguration message; In case of RRC connection establishment (excluding RRConnectionResume); and RRC connection re-establishment the field is not present; otherwise the field is optionally present, need ON. |
| HO-toEUTRA2 | The field is mandatory present in case of handover to E-UTRA or when the fullConfig is included in the RRCConnectionReconfiguration message; otherwise the field is optionally present, need ON. |
| LWIP | The field is optionally present, Need ON, if drbTypeLWIP-r13 is not set to eutran; otherwise it is not present and the UE shall delete any existing value for this field. |
| NR-PDCP | The field is optional present, Need ON, when the SRB is configured with NR-PDCP prior to reception of this reconfiguration message. Otherwise it is not present. |
| PDCP | For the bearers configured with E-UTRA PDCP, the field is mandatory present if the corresponding DRB is being setup; the field is optionally present, need ON, upon reconfiguration of the corresponding split DRB or LWA DRB, upon the corresponding DRB type change from split to MCG bearer, upon the corresponding DRB type change from MCG to split bearer or LWA bearer, upon the corresponding DRB type change from LWA to LTE only bearer, upon handover within E-UTRA and upon the first reconfiguration after re-establishment but in all these cases only when fullConfig is not included in the RRCConnectionReconfiguration message; otherwise it is not present. |
| PDCP-S | The field is mandatory present if the corresponding DRB is being setup; the field is optionally present, need ON, upon SCG change; otherwise it is not present. |
| RLC-Setup | This field is optionally present if the corresponding DRB is being setup, need ON; otherwise it is not present. |
| SCellAdd | The field is optionally present, need ON, upon SCell addition; otherwise it is not present. |
| Setup | The field is mandatory present if the corresponding SRB/DRB is being setup; otherwise the field is optionally present, need ON. |
| SetupM | The field is mandatory present upon setup of an MCG or split DRB, for EN-DC upon setup of MCG RLC bearer; otherwise the field is optionally present, need ON. |
| SetupS | The field is mandatory present upon setup of an SCG or split DRB, as well as upon change from MCG to split DRB; otherwise the field is optionally present, need ON. |
| SetupS2 | The field is mandatory present upon setup of an SCG or split DRB, as well as upon change from MCG to split or SCG DRB. For an SCG DRB the field is optionally present, need ON. Otherwise the field is not present. |
| SPS | The field is optionally present, need ON, if sps-ConfigUL in sps-Config (without suffix) and sps-ConfigUL-STTI-ToAddModList-r15 and sps-ConfigUL-ToAddModList-r15 in sps-Config-v1530 are not configured, or if sps-ConfigUL-ToAddModList-r14 in sps-Config-v1430 is not configured; otherwise it is not present. |
| SPS2 | The field is optionally present, need ON, if sps-ConfigUL in sps-Config-r12 and sps-ConfigUL-STTI-ToAddModList-r15 and sps-ConfigUL-ToAddModList-r15 in sps-Config-v1530 are not configured, or if sps-ConfigUL-ToAddModList-r14 in sps-Config-v1430 is not configured; otherwise it is not present. |
| SPS3 | The field is optionally present, need ON, if sps-Config (without suffix) is not configured and sps-Config-v1430 is not configured; otherwise it is not present. |
| SPS4 | The field is optionally present, need ON, if sps-Config-r12 is not configured and sps-Config-v1430 is not configured; otherwise it is not present. |

CHANGE END