3GPP TSG-RAN WG2 Meeting #105	R2-1900144

Athens, Greece, 25th February – 1st March 2019

Agenda Item:	12.3.3

Source: 	MediaTek Inc.

Title:  	Signalling Overhead Reduction for Conditional Handover

Document for:	Discussion and decision

Introduction

In RAN2#104 meeting, we made the following agreements about conditional handover (CHO) for handover robustness improvement in LTE.

Agreement 1 RAN2 will consider a conditional handover: This is defined as UE having network configuration for initiating access to a target cell based on configured condition(s). 2 Usage of conditional handover is decided by network. UE evaluates when the condition is valid. => FFS on the exact details of the procedures Agreement 1 Support configuration of one or more candidate cells for conditional handover. => FFS how many candidate cells (UE and network impacts should be clarified).

While CHO improves mobility robustness, one major concern is the signalling overhead due to preparation for candidate cells that are eventually not selected as handover targets. In this contribution, we study the signalling overhead reduction for CHO, with focus on the signalling overhead due to extra candidate cell preparation.

Discussion

CHO signalling overhead

In the preparation stage of legacy handover procedure, UE context is transferred to the target cell, then the target cell sends RRC configuration and random access (RA) information (in a container) to source cell, and then the information is sent to UE via a handover command.

While CHO improves mobility robustness, the robustness comes at the cost of signalling overhead. More specifically, the overhead due to preparation of a CHO candidate cell can be described as follows.

- The preparation message (incl. UE context transfer) from source eNB to every candidate eNB (X2).

- The handover command “container” (carrying configurations) sent from every candidate eNB to source eNB (X2).

- The handover command (configurations) from source eNB to UE every time a candidate is configured (Uu).

- The de-configuration message from source eNB to every unselected candidate eNB, if “explicit de-configuration” method is adopted (X2).

These messages are anyway needed for UE to perform handover. The problem for CHO is that they are sent every time a candidate eNB is configured, even though a candidate may eventually not be chosen as handover target.

Observation 1:	The signalling overhead for CHO preparation includes: (1) the preparation message from source eNB to every candidate eNB (X2), (2) the handover command “container” from candidate eNB to source eNB (X2), (3) the handover command (configurations) from source eNB to UE (Uu), and (4) the de-configuration message from source eNB to every unselected candidate eNB (for explicit de-configuration method) (X2).

Figure 1 illustrates the signalling overhead of conditional handover.

- As shown in Figure 1(a), at t1 UE is served by eNB1, and then at t2 UE detects eNB2, and the network configures eNB2 as a candidate eNB. This involves: (1) UE context transfer from source eNB (eNB1) to candidate eNB (eNB2), (2) candidate eNB prepares the CHO command and send it to source eNB via a container, and (3) source eNB transmits the CHO command to UE.

- Then as shown in Figure 1(b), UE detects eNB3 at t3, and eNB3 is configured as CHO candidate in a similar way.

- Finally, as shown in Figure 1(c), UE executes handover to eNB3 at t4. If explicit de-configuration is adopted, source eNB (eNB1) also sends a CHO De-configuration signal to de-configure eNB2 as a CHO candidate.

Figure 1. Signalling overhead in CHO preparation stage

In this example, the signalling introduced by the configuration and de-configuration of eNB2 is considered as extra signalling overhead, since eNB2 is eventually not chosen as handover target. To reduce signalling overhead, we consider two methods in the following sections.

Implicit de-configuration via validity timer

In CHO, not every configured candidate cell will be chosen as the target for handover. A cell cannot keep UE context and CHO configuration for a UE forever, and it needs to be de-configured at some point of time. As indicated in previous section, de-configuration of unselected candidate cell also contributes to CHO signalling overhead. One simple way to reduce this part of overhead is to configure validity timer(s) for candidate cell(s), and a candidate cell is implicitly de-configured once the associated validity timer expires.

Some numerical results for CHO with different validity timer and explicit de-configuration are provided in Table 1. We consider the handover performance metrics (handover failure rate, ping-pong rate), major Uu signals (measurement report, CHO command) and X2 signals (CHO preparation, validity timer extension, de-configuration), for a single UE moving at 120 km/h speed for one hour in a LTE network. Moreover, for the explicit de-configuration method, we consider a mechanism in which a candidate cell is explicitly de-configured only when the UE executes handover to another candidate cell (we assume that multiple candidate cells can be supported). For the validity timer method, when the validity timer is about to expire, network checks the validity of candidate cells and sends Validity Extension message to candidate cells that are still valid.

Table 1.	CHO with different validity timer and explicit de-configuration

|  | Validity Timer (ms) | 1000 | 5000 | 10000 | 100000 | Exp. de-conf. |
| --- | --- | --- | --- | --- | --- | --- |
| Mobility performance | HO Failure Rate (%) | 2.21 | 2.20 | 2.04 | 2.04 | 2.04 |
|  | Ping-pong Rate (%) | 11.93 | 11.54 | 11.62 | 11.62 | 11.62 |
| Uu signalling | Measurement Report # | 4955 | 4747 | 4714 | 4714 | 4714 |
|  | CHO CMD # | 4115 | 3919 | 3894 | 3894 | 3894 |
| X2 signalling | CHO Preparation # | 4909 | 4710 | 4666 | 4665 | 4665 |
|  | Timer Extension # | 425 | 39 | 2 | 0 | 0 |
|  | De-configuration # | 0 | 0 | 0 | 0 | 2978 |

From the numerical results in Table 1, we have the following observations:

- Each metric converges as validity timer length increases. That is, the length of validity timer has little impact on the mobility performance once the length is above some threshold. Therefore, the network can set the validity timer length as any value above such threshold, simply based on how long an eNB is expected to keep UE context.

- With the validity timer method, the Validity Extension mechanism introduces some X2 signalling overhead when timer is short. When timer is long, there is almost no validity extension, since the candidates are mostly invalid upon validity timer expiry.

- With the explicit de-configuration method, almost 3,000 de-configuration messages are sent from source eNBs to unselected candidate eNBs, which introduce significant X2 signalling overhead.

Observation 2:	Compared to de-configuration with explicit message, implicit de-configuration using validity timer significantly reduces X2 signalling overhead.

Proposal 1:	Validity timer can be associated with CHO candidate cells. When validity timer expires, the candidate cells are implicitly de-configured.

CHO with partial preparation

    - The method

As explained in previous section, CHO incurs significant signalling overhead due to preparation of candidate cells that are eventually not chosen as target. To reduce such overhead, we propose a method for CHO with partial preparation. This method incurs context transfer (in CHO Request) from source cell to for each candidate cell, but RRC configurations are not provided in CHO command.

With the partial preparation method:

- UE context is transferred from source eNB to every candidate eNB. This introduces signalling overhead, but allows candidate eNB to perform admission control.

- No handover command “container” from candidate eNB to source eNB is sent; the candidate eNB only sends a simple ACK. This means reduction of inter-eNB message size.

- CHO command is sent to UE for every candidate, but it carries only the cell ID and thus the size is much smaller.

- Since RRC configuration is not provided in CHO command, UE needs to read SIB before accessing target cell. Also, a CHO Execution Request (similar to RRC Reestablishment Request) message needs to be sent to target cell, and then UE receives further RRC configurations carried by a CHO Execution message from target cell.

The procedure of CHO execution in the proposed method is similar to RRC reestablishment. But the main difference is that UE context transfer and admission control have been done at the preparation stage. This not only avoids context fetch after target cell receives connection request (and thus reduces service interruption time), but also avoids the risk of fallback to RRC setup procedure when target cell fails to retrieve UE context.

    - Message flow

Figure 2 shows the message sequence chart of the partial preparation method. The steps in Figure 2 are described as follows:

Measurement report and preparation for conditional handover

- Network sends measurement configuration, including measurement events, to the UE.

- UE sends CHO measurement report when corresponding configured event is triggered.

- Source eNB decides that a conditional handover should be configured.

- Source eNB sends CHO REQUEST to target eNB, carrying UE context.

- Target eNB performs admission control.

- Target eNB sends CHO ACK to source eNB.

- Source eNB sends the CHO command to the UE, indicating target cell ID and associated conditions for UE to execute handover. But RRC configurations are NOT included.

Handover execution

- When pre-configured conditions are met, UE initiates CHO execution.

- UE sends CHO Indication to source eNB.

- UE acquires SI, performs synchronization and random access towards target eNB.

- UE sends CHO Execution Request message to target eNB.

- Target eNB sends CHO Execution message to UE, carrying RRC configurations.

Handover completion

- UE sends CHO Execution Complete message to target eNB.

- Source eNB sends SN STATUS TRANSFER message to target eNB.

- Target eNB sends a PATH SWITCH REQUEST message to MME to inform that the UE has changed cell.

- The MME sends a MODIFY BEARER REQUEST message to the S-GW.

- The S-GW switches the downlink data path to the target side. The S-GW sends one or more “end marker” packets on the old path to the source eNB.

- The S-GW sends a MODIFY BEARER RESPONSE message to MME.

- The MME confirms the PATH SWITCH REQUEST message with the PATH SWITCH REQUEST ACKNOWLEDGE message.

- By sending the UE CONTEXT RELEASE message, the target eNB informs success of HO to source eNB and triggers the release of resources by the source eNB.

- The source eNB releases radio and C-plane related resources associated to the UE context.

Figure 2. Message sequence chart of CHO with partial preparation

Based on the discussions above, we propose to support CHO with partial preparation, in which UE context is transferred to target cell at preparation stage, but RRC configuration is provided by target cell after UE sends CHO execution request.

Proposal 2:	Support conditional handover with partial preparation, in which UE context is transferred to candidate cell at preparation stage, but RRC configuration is provided by the target cell after UE selects a candidate cell as its target cell.

    - Practical usage

The signalling overhead reduction in proposed “partial preparation” method is achieved at the cost of longer interruption time, compared to a “fully-prepared” CHO which has interruption time similar to that for normal handover procedure. In practice, the CHO partial preparation may coexist with normal handover procedure, and executed only when normal handover is about to fail. Instead of triggering RRC re-establishment, UE initiates CHO and access partially prepared candidate cells. In other words, CHO with partial preparation serves as a fast mobility failure recovery mechanism.

Observation 3:	CHO with partial preparation has longer service interruption time than fully-prepared CHO and normal handover procedures.

Proposal 3:	CHO with partial preparation may coexist with normal handover procedure, and serve as a fast mobility failure recovery mechanism for normal handover.

Conclusion

We have the following observation:

Observation 1:	The signalling overhead for CHO preparation includes: (1) UE context transfer from source eNB to every candidate eNB (X2), (2) the handover command “container” from candidate eNB to source eNB (X2), (3) the handover command (configurations) from source eNB to UE (Uu), and (4) the de-configuration message from source eNB to every unselected candidate eNB (for explicit de-configuration method) (X2).

Observation 2:	Compared to de-configuration with explicit message, implicit de-configuration using validity timer significantly reduces X2 signalling overhead.

Observation 3:	CHO with partial preparation has longer service interruption time than fully-prepared CHO and normal handover procedures.

It is proposed to discuss and decide on the following proposal:

Proposal 1:	Validity timer can be associated with CHO candidate cells. When validity timer expires, the candidate cells are implicitly de-configured.

Proposal 2:	Support conditional handover with partial preparation, in which UE context is transferred to candidate cell at preparation stage, but RRC configuration is provided by the target cell after UE selects a candidate cell as its target cell.

Proposal 3:	CHO with partial preparation may coexist with normal handover procedure, and serve as a fast mobility failure recovery mechanism for normal handover.

Reference

- RP-181433, New WID: NR mobility enhancements,	Intel Cooperation

- R2-1816960, Performance Evaluation and Implication for Conditional HO, MediaTek Inc.