3GPP TSG-RAN WG2 Meeting #105                                                        R2-1900153

Athens, Greece, 25th Feb – 1st Mar 2019

Source:	CATT

Title:	SPS and Configured Grant periodicities

Agenda Item:	11.7.2.2

Document for:	Discussion and Decision

Introduction

It is expected that SPS and Configured Grants (CG) will play a key role in serving the various co-existing traffic types expected in TSN networks [1]. One first consequence is the requirement to support multiple active CG configurations per BWP per Serving Cell agreed in RAN1 [2], which we propose extending to SPS as well in [3]. Another impact is the need to map IEEE 802.1Qbv schedules and cycle times to SPS/CG periodicities [1].

In this contribution we analyze the impact of IIoT traffic requirements on SPS/CG periodicities.

Discussion

Given we support upgrading Rel-15 SPS configurability to the same flexibility as configured grants, we only focus on CG periodicities in this contribution, provided it should equally apply to SPS.

Rel-15 periodicity is defined as 2, 7, n*14 symbols (= n*slot) in RRC specification, thus guaranteeing slot-aligned periods. The lower range of the CG periodicity ≥ 500 µs provides limited granularity: 500, 625, 750, 875, 1000 µs, etc, as shown in Table 1 up to n = 10.

- Note the lower values of Rel-15 (31.25 and 62.5) are for FR2 only.

Table 1: Rel-15 CG periodicities (low range) for the 5 supported numerologies

| µ | df [kHz] | CG periodicity (us) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | symbols |  | slots |  |  |  |  |  |  |  |  |  |
|  |  | 2 | 7 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| 0 | 15 | 142.8571 | 500 | 1000 | 2000 | 3000 | 4000 | 5000 | 6000 | 7000 | 8000 | 9000 | 10000 |
| 1 | 30 | 71.42857 | 250 | 500 | 1000 | 1500 | 2000 | 2500 | 3000 | 3500 | 4000 | 4500 | 5000 |
| 2 | 60 | 35.71429 | 125 | 250 | 500 | 750 | 1000 | 1250 | 1500 | 1750 | 2000 | 2250 | 2500 |
| 3 | 120 | 17.85714 | 62.5 | 125 | 250 | 375 | 500 | 625 | 750 | 875 | 1000 | 1125 | 1250 |
| 4 | 240 | 8.928571 | 31.25 | 62.5 | 125 | 187.5 | 250 | 312.5 | 375 | 437.5 | 500 | 562.5 | 625 |

The question is: is this sufficient for TSN? In order to answer this question, we need to look further into the cycle times and end-to-end latency requirements of IIoT traffic.

22.804 requirements on cycle time and end-to-end latency

Some existing IIoT solutions support very small cycle times, for example EtherCAT and PROFINET IRT support cycle times as low as 12.5 µs and 31.25 µs respectively. TSN 802.1Qbv also supports very low cycle times. So it should be first checked which lower bound should be considered for this SI.

The cycle times for different Factories of the Future applications are expressed in Table 5.3.2.1-1 of Section 5.3.2.1:

Table 5.3.2.1-1: Typical characteristics of motion control systems for three major applications

| Application | # of sensors / actuators | Typical message size | Cycle time Tcycle | Service area |
| --- | --- | --- | --- | --- |
| Printing Machine | > 100 | 20 byte | < 2 ms | 100 m x 100 m x 30 m |
| Machine Tool | ~ 20 | 50 byte | < 0.5 ms | 15 m x 15 m x 3 m |
| Packaging Machine | ~ 50 | 40 byte | < 1 ms | 10 m x 5 m x 3 m |

And then converted into requirements in Section 5.3.2.6:

| Reference number | Requirement text | Application / transport | Comment |
| --- | --- | --- | --- |
| Factories of the Future 2.1 | The 5G system shall support cyclic traffic with cycle times in the order of 1 ms for a communication group of about 50 UEs and payload sizes of about 40 byte. | T |  |
| Factories of the Future 2.2 | The 5G system shall support cyclic traffic with cycle times in the order of 0,5 ms for a communication group of about 20 Ues and payload sizes of about 50 byte. | T |  |
| Factories of the Future 2.3 | The 5G system shall support cyclic traffic with cycle times in the order of 2 ms for a communication group of about 100 Ues and payload sizes of about 20 byte. | T |  |

From the above, a first observation can be derived as follows:

Observation 1: The lower end cycle time values of EtherCAT, PROFINET IRT or 802.1Qbv below 500 µs are not in the scope of this SI.

Now looking at the end-to-end latency requirements in the merged requirement section 8.1.2, we can see that the above cycle times for the most stringent IIoT applications constitute an upper bound for the end-to-end latency:

| Characteristic parameter (KPI) |  |  | Influence quantity |  |  |  |  |  | Requirement | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Communication service availability | End-to-end latency: target value | End-to-end latency: jitter (note) | Message size [byte] | Transfer interval: target value | Survival time | UE speed | # of UEs | Service area |  |  |
| 99,9999% to 99,999999% | < transfer interval |  | 20 to 50 | 0,5 ms to 2 ms | Transfer interval | ≤ 20 m/s | ≤ 100 |  | Factories of the Future 2.1, 2.2, 2.3, 2.8, 2.10 | Motion control and control-to-control use cases |
| NOTE 1: The jitter interval is symmetric. However, only late arrivals count as communication error. |  |  |  |  |  |  |  |  |  |  |

Another example of cycle time is given in Section 5.6.6.1 (Smart Grid)

| Reference number | Requirement text |
| --- | --- |
| Electric Power Distribution 5.1 | The 5G system shall support a peer-to-peer data communication service with cycle times in the order of 0,8 ms. |

From further description in the body text, it is clarified that this requirement comes with an end-to-end latency of 15ms.

From the above examples/requirements we can derive the following observation:

Observation 2: Depending on the IIoT application, the end-to-end latency can be smaller or larger than the cycle time.

Impact on CG periodicities

As discussed in Section 2.1, we need to consider two cases: the end-to-end latency is larger or smaller than the cycle time.

Case 1: the end-to-end latency is larger than the cycle time (Figure 1)

In such case consecutive messages can be grouped into one CG/SPS and de-jittered at the 5GS boundary (e.g. in the translators), as shown in Figure 1. As a consequence, CG/SPS periodicity does not need to meet the cycle time as long as it meets the end-to-end latency, which is not expected to be stringent in that case, and therefore can be addressed by Rel-15 CG periodicities.

Figure 1: The end-to-end latency is larger than the cycle time

Observation 3: Rel-15 CG periodicities are sufficient to address the case when the end-to-end latency is larger than the cycle time.

Case 2: the end-to-end latency is smaller or equal to the cycle time (Figure 2)

This is the case of the most stringent end-to-end latency requirements (< 0.5ms). In such case, the solution consists in oversampling the cycle time with SPS/CGs resources and to use only one SPS/CG out of N. The (oversampling) SPS/CG periodicity is chosen to meet the end-to-end latency requirement, rather than the cycle time.

Let’s take an example: cycle time = 0.6ms; end-to-end latency = 0.5ms. As shown in Figure 2, such usecase can be addressed with the following periodicities for the different numerology cases:

- @ 15kHz numerology, SPS/CG periodicity = 2 symbols ≈ 143 µs, N = 4;

- @ 30kHz numerology, SPS/CG periodicity = 7 symbols = 250 µs, N = 2;

- @ 60kHz numerology, SPS/CG periodicity = 7 symbols = 125 µs, N = 5;

Then the question is: what to do with unused SPS/CG resources to avoid spectral waste?

Without changing the legacy CG configuration, the unused CGs will be skipped. Then their resource can be re-used by other CGs or by dynamic scheduling (see intra-UE multiplexing and prioritization solutions).

Another option is to upgrade the SPS/CG configuration by indicating the “modulo N” to the UE so it only considers one SPS/CG resource every N. This is already supported by the current specification for slot periods, but not for 2-symbol or 7-symbol periods. Hence, all needs to be upgraded is the support of periodicities expressed as N*(2-symbols) and N*(7-symbols).

Proposal: Rel-16 SPS/CG periodicity is defined as n*2, n*7, n*14 symbols.

Figure 2: The end-to-end latency is smaller than the cycle time

Conclusion

This contribution discusses the need to upgrade the existing SPS/CG periodicities in support of the flexible TSN traffic cycle times and end-to-end latency. The resulting observations and proposals are as follows:

Observation 1: The lower end cycle time values of EtherCAT, PROFINET IRT or 802.1Qbv below 500 µs are not in the scope of this SI.

Observation 2: Depending on the IIoT application, the end-to-end latency can be smaller or larger than the cycle time.

Observation 3: Rel-15 CG periodicities are sufficient to address the case when the end-to-end latency is larger than the cycle time.

Proposal: Rel-16 SPS/CG periodicity is defined as n*2, n*7, n*14 symbols.

Reference

- R2-1900151, Impact of supporting IEEE 802.1Qbv on 5GS RAN, CATT

- R2-1818991, LS on multiple active configured grant configurations, 3GPP RAN2#104, 3GPP TSG RAN WG2#104, 12 - 16 November, 2018;

- R2-1900152, Multiple active SPS and Configured Grant Configurations, CATT