3GPP TSG-RAN WG2 Meeting #105                                                 	R2-1900226

Athens, Greece, 25th Feb – 1st Mar 2019

Source:	CATT

Title:	Discussion on MDT

Agenda Item:	11.12.2

Document for:	Discussion and Decision

Introduction

We started to discuss the topic of RAN-centric data collection and utilization for LTE and NR from RAN3#101bis meeting, and a running TR37.816 [1][2] was agreed. Currently, the TR mainly includes the use cases name of this study item, but the other parts like the content of use cases or architecture are still empty and should be discussed further.

In this contribution, we intend to discuss the MDT architecture, classification, procedures and measurement quantities in NR, and analyze the differences between NR and LTE.

Discussion

  - Architecture of Performing MDT

LTE MDT is based on the LTE Trace architecture and Uu interface configuration. LTE MDT could be divided into signalling based MDT and management based MDT. Signalling based MDT could be used to configure a specific UE and the configuration signalling is delivered via several network nodes such as HSS, MME and eNB while management base MDT could be used to configure a number of UEs and the configuration is delivered from network manager to eNB directly.

In NR SA5 has already added support for 5G Trace in the specifications of TS32.421~TS32.423 in which include use cases, requirements, activation/deactivation mechanisms, control and configuration parameters, and the definition of trace record. All of the above are similar with LTEand could be simply reused for NR MDT.

Observation 1: SA5 has already added support for 5G Trace in the specifications of TS32.421~TS32.423.

Proposal 1: Reuse SA5 Trace architecture to perform NR MDT, similar as LTE does.

  - Immediate MDT and Logged MDT

Information about radio coverage is essential for network planning. MDT WID is starting in LTE from Release10which includes immediate MDT and logged MDT, for coverage optimization.

Immediate MDT

Immediate MDT in LTE is almost based on the RRM measurementand could add the location information for assistance. Since the design of LTE RRM measurement is reused in NR, the design of immediate MDT could also be reused as baseline, with some revisions:

- For configuration:

  - RRCReconfiguration message is used to send the MDT configuration to UE;

  - The measurement events A1-A6, B1 and B2 could all be utilized;

  - Measurement quantities of RSRP, RSRQ and SINR could all be utilized;

- For result collection:

  - MeasurementReport message is used to send the MDT result to the network;

  - Related information (e.g. Location Info) can be reported together with the MDT result;

In NR, the network may transmit in multiple beams with different SSB, and the beam related result could be added in the measurement report. NR RRM measurement could also be performed based on CSI-RS, and the concerned result could be considered for MDT case either.

Observation 2: SSB and CSI-RS could both be used for immediate MDT.

Proposal 2: Reuse NR RRM measurement to perform NR immediate MDT, similar as LTE does.

Proposal 3: Current RRM measurement result of beam level (based on SSB/CSI-RS) could also be used for immediate MDT in NR.

Logged MDT

In LTE, Logged MDT can be regarded as the supplement of CONNECTED mode RRM measurementand is only performed in IDLE mode. Logged MDT collection is base on IDLE mode measurement which is originally used for cell selection/re-selection.

In NR, the logged MDT could also be performed in either INACTIVE mode or IDLE mode for coverage detection. IDLE/INACTIVE mode measurement originally used for cell selection/re-selection could be utilized for NR logged MDT, and it doesn’t need any additional measurement for MDT purpose.

Proposal 4: Record NR IDLE/INACTIVE mode measurement result (original used for cell selection/re-selection) for NR logged MDT, similar as LTE does.

Since the design of LTE RRM(MDT) measurement is reused in NR, the design of logged MDT in LTE could also be reused as baseline, with some revisions:

- For configuration:

  - Introduce LoggedMeasurementConfiguration message to configure logged MDT, which includes:

  - Trace related informations e.g. trace identity (Whether the TCE identity for collecting all the NR MDT result from gNB is send to UE, or other collection node is used in NR could be further studied) ;

  - Parameters to perform logging e.g. length of logging time, logging area, duration and interval;

  - All the measurement quantities of RRM measurement could be utilized;

  - Since CSI-RS is only send to UE in CONNECTED mode, Logged MDT measurement could only be performed base on the reference signal of SSB;

- For result collection:

  - The messages for indicating available of logged measurement result to the network could be: RRCReconfigurationComplete, RRCReestablishmentComplete, RRCSetupComplete and RRCResumeComplete.

  - Introduce UEInformationResquest and UEInformationResponse messages to acquire the logged MDT result from UE;

Observation 3: RRCResumeComplete message could also be used for indicate available of logged result in NR.

Proposal 5: Introduce signalings similar as LTE logged MDT for configuration and result reporting of NR logged MDT.

  - RLF Report and Accessibility Measurement

RLF Report

RLF report is discussed in LTE MDT scope. Since the HOF/RLF has tight relationship with SON, it is suggested to discuss this issue in NR SON, e.g. Coverage and Capacity Optimizations use case. And the signalling in air-interface could be added after discussion of RAN3.

Proposal 6: Procedure and content of RLF report could be further studied after discussion of RAN3.

Accessibility Measurement

Accessibility measurement is used to collect the parameters when RRC setup failure occurred, and the network could optimize UL coverage and RACH parameters from multiple results reported from lots of UE.

The design of accessibility measurement in LTE could be reused as baseline, with some revisions:

- The accessibility measurement does not need network configuration;

- For result collection:

  - UE could log the accessibility problem when failed to setup or resume an RRC connection:

  - The logged content may includes: the identity and measurement result of failed cell, the identity and measurement result of neighbor cells, preamble send numbers, if it is contention based, and anything helpful for the network to spot the problem;

  - Certainly, the accessibility measurement may be used for resume request failure, which also needs UL synchronization between UE and network.

  - The available indication and the result report procedures could be similar as logged MDT reporting.

Observation 4: Accessibility measurement could be performed when RRC setup failure, or when RRC resume failure.

Proposal 7: Introduce accessibility measurement in NR for MDT use case, the available indication and the result report procedure could be performed similar as logged MDT reporting.

Proposal 8: Accessibility measurement could be both performed for RRC resume failureand RRC setup failurein NR.

Conclusion

According to the analysis in section 2, we have:

Observation 1: SA5 has already added support for 5G Trace in the specifications of TS32.421~TS32.423.

Observation 2: SSB and CSI-RS could both be used for immediate MDT.

Observation 3: RRCResumeComplete message could also be used for indicate available of logged result in NR.

Observation 4: Accessibility measurement could be performed when RRC setup failure, or when RRC resume failure.

And we propose:

Proposal 1: Reuse SA5 Trace architecture to perform NR MDT, similar as LTE does.

Proposal 2: Reuse NR RRM measurement to perform NR immediate MDT, similar as LTE does.

Proposal 3: Current RRM measurement result of beam level (based on SSB/CSI-RS) could also be used for immediate MDT in NR.

Proposal 4: Record NR IDLE/INACTIVE mode measurement result (original used for cell selection/re-selection) for NR logged MDT, similar as LTE does.

Proposal 5: Introduce signalings similar as LTE logged MDT for configuration and result reporting of NR logged MDT.

Proposal 6: Procedure and content of RLF report could be further studied after discussion of RAN3.

Proposal 7: Introduce accessibility measurement in NR for MDT use case, the available indication and the result report procedure could be performed similar as logged MDT reporting.

Proposal 8: Accessibility measurement could be both performed for RRC resume failureand RRC setup failurein NR.

A TP is attached in Annex.

Reference

- R3-186226 Skeleton for TR 37.816 v0.0.1

- R3-186227 TP of agreed use cases for TR 37.816

- Email discussion for RAN-centricDCU with summary v4.docx

Annex

The draft TP is based on TR 37.816[1] and Email discussion for RAN-centricDCU with summary v4.docx [3].

<<<<<<<<<<<<<<<<<<<<Start of change >>>>>>>>>>>>>>>>>>>>

- 5.7	MDT

- 5.7.1	Use case description

Minimization of drive test functionality helps operators to reduce the requirement for time consuming drive testing by collecting measurements from the subscribers and network elements.

There are two modes for MDT: logged MDT and immediate MDT. Logged MDT is to collect measurements from RRC idle mode UEs whereas immediate MDT is performed only for RRC connected UEs. Logged MDT and immediate MDT and accessibility report should be supported for NR MDT. Logged MDT should also be supported for RRC_INACTIVE mode UEs.

Management based and signalling based trace procedures in LTE should be taken as the baseline. In addition to that, MDT in NR should study the configuration and measurement collection procedures which are needed for CU-DU split, CP-UP separation architecture.

LTE MDT measurements could be the baseline, new MDT measurement for NR, e.g., beam, UL/SUL accessibility report, UE sensor information should be studied.

- 5.7.2	Solution description

Immediate MDT

The design of immediate MDT in LTE could be reused as baseline in NR, with some revisions:

- For configuration:

  - RRCReconfiguration message is used to send the MDT configuration to UE;

  - The measurement events A1-A6, B1 and B2 could all be utilized;

  - Measurement quantities of RSRP, RSRQ and SINR could all be utilized;

  - Measurement could be performed base on either reference signal of SSB/CSI-RS.

- For result collection:

  - MeasurementReport message is used to send the MDT result to the network;

  - Related information (e.g. Location Info) could be report together with MDT result;

Logged MDT

NR logged MDT could be performed in either INACTIVE mode or IDLE mode. IDLE/INACTIVE mode measurement original used for cell selection/re-selection could be utilized for NR logged MDT.

The design of logged MDT in LTE could be reused as baseline in NR, with some revisions:

- For configuration:

  - Introduce LoggedMeasurementConfiguration message to configure logged MDT, which includes:

  - Trace related information e.g. trace identity (Whether the TCE identity for collecting all the NR MDT result from gNB is send to UE, or other collection node is used in NR could be further studied) ;

  - Parameters to perform logging e.g. length of logging time, logging area, duration and interval;

  - All the measurement quantities of RRM measurement could be utilized;

  - Logged MDT measurement could only be performed base on the reference signal of SSB;

- For result collection:

  - The messages for indicating available of logged measurement result to the network could be: RRCReconfigurationComplete, RRCReestablishmentComplete, RRCSetupComplete and RRCResumeComplete.

  - Introduce UEInformationResquest and UEInformationResponse messages to acquire the logged MDT result from UE;

Accessibility Measurement

Accessibility measurement is used to collect the parameters when RRC setup/resume failure occurred, and the network could optimize UL coverage and RACH parameters from multiple results reported from lots of UE.

The design of accessibility measurement in LTE could be reused as baseline, with some revisions:

- The accessibility measurement does not need network configuration;

- For result collection:

  - UE logs the accessibility problem when failed to setup/resume an RRC connection:

  - The logged content may includes: the identity and measurement result of failed cell, the identity and measurement result of neighbor cells, preamble send numbers, if it is contention based, and anything helpful for the network to spot the problem;

  - The available indication and the result report procedures could be similar as logged MDT reporting.

<<<<<<<<<<<<<<<<<<<<End of change >>>>>>>>>>>>>>>>>>>>