3GPP TSG-RAN WG2 Meeting #105                               R2-1900228

Athens, Greece, 25th February - 1st March 2019

Source:	CATT

Title:	Mobility Issue for Connected Mode UE in NTN System

Agenda Item:	11.6.4.1

Document for:	Discussion and Decision

Introduction

Handover is a main issue addressed for mobility in NTN system, the objective of this issue is given below:

- Handover: Study and identify mobility requirements and necessary measurements that may be needed for handovers between some non-terrestrial space-borne vehicles (such as Non Geo stationary satellites) that move at much higher speed but over predictable paths [RAN2, RAN1]

In RAN2#104 meeting, some basic agreements are made for NTN mobility [1]:

Agreements:

1.	Satellite beams, satellites or satellite cells are not considered to be visible from UE perspective in NTN SI.  This does not preclude differentiating at the PLMN level the type of network (e.g. NTN vs. terrestrial).  This is up to SA2.

2.   Revise the current definition of satellite cell in TR 38.821 and refer to a satellite beam.  Definition of satellite beam can be discussed during email discussion.

3.	Add text in TR 38.821 stating that association between NR PCI and NR SSBs is left for implementation (i.e. it will not be specified)

4.	Consider Rel-15 definitions as a baseline for NTN

5.	Both option a and b can be considered in NTN SI with one or multiple SSBs per PCI.  The TR will capture a figure for both option.

Agreements

1	The details of the satellite ephemeris can be captured in the TR.

In this document, we’d like to discuss solutions for UE mobility in connected mode.

Discussion

In TN system, cell is fixed once deployed by operator, but the UE is moving in an unpredictable way.  In order to control the mobility for UE in connected mode efficiently, the network needs UE measurement report to assist the serving cell to choose a feasible target cell for the UE.

Observation 1: In TN system, the mobility for UE in connected mode is based on UE measurement report.

As for NTN system, the situation is quite different. All the satellites apart from GEO have speed relative to ground and the satellite cells are moving by time but in a predictable way.

Observation 2: In NTN system, all the satellites apart from GEO are moving by time relative to ground but in a predictable way.

According to the latest TP in TR38.821 [2], six basic scenarios are considered as depicted in Table 1:

Table 1: Reference scenarios

|  | Transparent satellite | Regenerative satellite |
| --- | --- | --- |
| GEO based non-terrestrial access network | Scenario A | Scenario B |
| LEO based non-terrestrial access network: steerable beams | Scenario C1 | Scenario D1 |
| LEO based non-terrestrial access network: the beams move with the satellite | Scenario C2 | Scenario D2 |

To make the analysis for UE mobility easy, we list all the possible handover cases in Table 2:

|  | Handover Cases | Serving cell | Target cell |
| --- | --- | --- | --- |
| Inter-satellite | Case1 | GEO | GEO |
|  | Case2 | GEO | LEO |
|  | Case3 | LEO | GEO |
|  | Case4 | LEO | LEO |
| Intra-satellite | Case5 | GEO | GEO |
|  | Case6 | LEO | LEO |

We don’t think there is necessary to discuss the cases above one by one. We can talk about some common baseline to cover all the cases. In RAN2#104 meeting, we agree to capture the details of the satellite ephemeris into TR38.821, so it’s natural to consider satellite ephemeris based handover mechanism in NTN system. But in TR 38.821, we also assume not all UEs are capable of positioning, so we can’t assume that the network can always get UE location info to assist the satellite ephemeris based handover mechanism. If some UEs are not capable of positioning, we think NR R15 handover mechanism can be as a baseline.

Proposal 1: For UEs not capable of positioning, NR R15 handover mechanism can work as a baseline.

If UEs are capable of positioning, the location info of UEs can be used to assist the satellite ephemeris based handover mechanism.

Proposal 2: For UEs capable of positioning, the location info of UEs can be used to assist the satellite ephemeris based handover mechanism.

As for intra-satellite handover case, some adjustments are needed. If the satellite can get the UE location information, the satellite can know which neighbor cell is more feasible for intra-satellite handover based on UE location info and the cell distribution info within the satellite. But how the satellite knows its cell distribution info within the satellite is more like an implementation issue.

Proposal 3: For intra-satellite handover case, how the satellite knows its cell distribution info within the satellite is up to implementation.

If there is only one cell within a satellite, UE mobility within a satellite is more like a beam management issue, RAN1 will handle this part.

Proposal 4: RAN1 will lead beam management based UE mobility.

Conclusion

In conclusion, we propose the followings:

Observation 1: In TN system, the mobility for UE in connected mode is based on UE measurement report.

Observation 2: In NTN system, all the satellites apart from GEO are moving by time relative to ground but in a predictable way.

Proposal 1: For UEs not capable of positioning, NR R15 handover mechanism can work as a baseline.

Proposal 2: For UEs capable of positioning, the location info of UEs can be used to assist the satellite ephemeris based handover mechanism.

Proposal 3: For intra-satellite handover case, how the satellite knows its cell distribution info within the satellite is up to implementation.

Proposal 4: RAN1 will lead beam management based UE mobility.

References

[1] RAN2#104 Chairman Notes

[2] 3GPP TR 38.821