3GPP TSG-RAN WG2 Meeting #105												R2-1900238

Athens, Greece, 25th Feb. - 01st Mar. 2019

Source: 	vivo

Title:  	Paging Enhancements for NR-U

Agenda Item:	11.2.2.1

Document for:	Discussion and Decision

Introduction

According to the NR-U TR 38.889 [1], some identified enhancements/agreements regarding paging are quoted as follows:

For paging, it may be beneficial to introduce more opportunities per DRX cycle for the UE to receive the page. The additional locations can be provided in time domain by configuring an extended paging occasion (i.e. a paging window) or configuring multiple paging occasions to a UE. In any specified solution(s) based on additional paging opportunities, the UE power consumption should also be taken into account; to this end, it is beneficial that the paging occasions are transmitted in close time to or overlap with the reference signals.

In this contribution, we discuss more detailed paging enhancements for NR-U, regarding the multi-beam and the power saving.

Discussion

Extended paging occasion or multiple paging occasions

According to the TR 38.889, we could have the following the time domain solutions:

- Option 1: One paging occasion with a paging window

- Option 2: Multiple paging occasions

From our understandings, both options can provide more opportunities per paging DRX cycle for the UE. Compared with Option 1, Option 2 would need to modify the calculation formula of the paging occasion, which causes more complexity in the UE and the specification. And there could be lots of ways to provide multiple paging occasions, based on different calculation formulas. Thus, we slightly prefer Option 1.

Proposal 1: A paging window (e.g. in number of slots) starts at the first monitoring occasion of the PO.

Proposal 2: The PO calculation as specified in the current specification is reused to determine the first monitoring occasion of the PO.

More transmission opportunities for multi-beam paging

Figure 1: Multiple paging transmission

According to the NR IDLE UE behaviours, “the length of one PO is one period of beam sweeping”. However the IDLE UE may only be able to receive the paging from one beam. Then for each beam, in order to increase the transmission opportunities, the paging window should be able to include more than one period of beam sweeping.

Proposal 3: One paging window can include more than beam sweeping period.

Consideration on the power consumption for paging reception

Although increasing the transmission opportunities per DRX cycle can improve the transmission for the NR-U paging, the main drawback would be the increased UE power consumption for monitoring the paging transmission. Here we consider that if the paging message is received by the UE and the paging message does not include the UE ID in the paging window, the UE can stop the subsequent paging monitoring in the paging window.

Proposal 4: The UE can stop the paging monitoring within the paging window if a paging message is received by the UE and the paging message does not include the UE ID.

Conclusions

Based on the analysis given above, we have the following Observations and Proposals：

Proposal 1: A paging window (e.g. in number of slots) starts at the first monitoring occasion of the PO.

Proposal 2: The PO calculation as specified in the current specification is reused to determine the first monitoring occasion of the PO.

Proposal 3: One paging window can include more than beam sweeping period.

Proposal 4: The UE can stop the paging monitoring within the paging window if a paging message is received by the UE and the paging message does not include the UE ID.

Reference

[1] 3GPP TR 38.889, “Study on NR-based access to unlicensed spectrum”.