3GPP TSG-RAN WG2 Meeting #105       							      R2-1900241

Athens, Greece, 25th Feb. - 01st Mar. 2019                  (Revision of R2-1818270)

Source: 	vivo

Title:  	RAN2 Impacts of RLM for NR-U

Agenda Item:	11.2.2.2

Document for:	Discussion and Decision

Introduction

In the previous RAN#82 Plenary, a new WID on NR-based access to unlicensed spectrum has been approved [1]. According to the detailed objectives of the WI, the following enhancements to RLM/RRM for NR-U should be specified:

- 	RLM/RRM extensions for NR-U operation due to uncertain and reduced transmission opportunities for DL signals and channels due to LBT failure in line with agreements during the study phase (NR-U TR section 7.2.1.3.2), including configuring different DRS Measurement Time Configuration (DMTCs) for RRM and RLM respectively, identifying the set of RLM-RSs to measure, which set(s) are used for in-sync, out-of-sync evaluations, potential definition of a metric to accurately identify unsuccessful detection of RLM-RS. Support RSSI reporting. Define a metric to measure channel occupancy or medium contention and its corresponding reporting. (RAN1/RAN2)

In this contribution, we will discuss RAN2 impacts of potential enhancements to RLM for NR-U based on the objectives of the WI.

Discussion

RLM in NR

In NR, the UE estimates the downlink radio link quality and compare it to the thresholds Qout and Qin for the purpose of monitoring downlink radio link quality of the cell. The threshold Qout is defined as the level at which the downlink radio link cannot be reliably received and shall correspond to the out-of-sync block error rate. The threshold Qin is defined as the level at which the downlink radio link quality can be significantly more reliably received than at Qout and shall correspond to the in-sync block error rate [2].

The PHY layer in the UE shall, in frames where the radio link quality is assessed, indicate out-of-sync (OOS) to higher layer when the radio link quality is worse than the threshold Qout for all resources in the set of resources for radio link monitoring. When the radio link quality is better than the threshold Qin for any resource in the set of resources for radio link monitoring, the physical layer in the UE shall, in frames where the radio link quality is assessed, indicate in-sync (IS) to higher layer [3].

If the higher layer in the UE receives N310 consecutive OOS indications, timer T310 will start. While T310 is running, if the higher layer receives N311 consecutive IS indications, T310 will stop. Otherwise, the UE declares RLF upon T310 expiry. After RLF declaration, the UE performs RRC connection re-establishment and starts timer T311. If T311 expires, the UE will go to RRC_IDLE [4].

An example of RLM procedure in NR is shown in the following figure.

Figure 1. An example of RLM procedure in NR

Enhancements to RLM

In the unlicensed spectrum, a gNB has to perform CCA before transmitting the configured RLM-RS. Due to the uncertainty of LBT, it is possible that RLM-RS cannot be transmitted by the gNB. As a result, the UE may declare RLF even if the downlink radio link quality is good enough.

To mitigate the LBT impacts on RLM in NR-U, we need to concentrate on avoiding the “false” OOS indication when the RLM-RS cannot be transmitted due to LBT failure. In another words, the indication mechanism of RLM should be enhanced to accurately represent the downlink radio link quality. For example, if a configured RLM-RS cannot be detected by the UE due to DL LBT failure, the PHY layer in the UE should not indicate OOS to higher layer.

More specifically, if the SINR of all configured RLM-RS resources are worse than Qout while the SINR of a predefined number of configured RLM-RS resources are worse than another configured threshold (e.g. Threshold_1 given in the Figure 2), the PHY layer in the UE shall indicate a new state indication (e.g. LBT failure indication) to the higher layer, instead of OOS. Thus, the “false” OOS indication due to DL LBT failure can be avoided. Obviously, this kind of indication mechanism has extra impacts on RAN2 specifications. At least the corresponding UE behavior upon receiving the new state indication needs to be further studied in RAN2. Last but not least, the feasibility and the reliability of this kind of indication mechanism are discussed with more details in [5].

Figure 2. A new criterion for radio link quality evaluation

Alternatively, LBT failure can be considered as a form of bad radio link quality since the downlink radio link between the gNB and the UE is not operational. Without introducing a new state indication, we consider that, if the UE determines that all configured RLM-RS in the set of RLM-RS resources cannot be detected, the PHY layer in the UE shall indicate OOS to the higher layer, as shown in the Figure 3. Although this kind of indication mechanism has no additional impact on RAN2 specifications, both the “false” OOS indication due to the LBT failure and not detecting physical layer problems in timely are still inevitable in some cases.

Figure 3. A new criterion for IS/OOS indication

Proposal 1: Considering that the UE may not detect RLM-RS due to LBT, radio link quality indication mechanism should be enhanced via the following alternatives：

- A new state indicated to higher layer is suggested to be defined based on the number of undetected RLM-RS.

- The criterion of in-sync and out-of-sync indication should be properly enhanced with taking the number of undetected RLM-RS into account.

RAN2 impacts of RLM enhancements

According to the potential options of avoiding the “false” OOS indication given above, it seems necessary to analyze and discuss the impacts on RAN2.

The basic intention of the RLM/RLF procedure is to recovery the UE from a broken downlink radio link between the gNB and the UE. As far as we are concerned, LBT failure can be regarded as a sign of bad radio link quality because the UE becomes unreachable from the perspective of the gNB. Persistent NW LBT failures should be considered as physical layer problems. Then the UE shall detect these problems, starting timer T310 or declaring RLF. Thus, we propose that, if the high layer in the UE receives N320 consecutive new state indications, timer T310 will be started or RLF will be triggered.

Due to the measurement inaccuracy, the measured SINR of the configured RLM-RS may fluctuate around the Threshold_1 given in the Figure 2. Consequently, it is possible that the PHY layer in the UE indicates OOS to high layer in an indication period while it will indicate the new state indication to high layer in the next indication period, even though the channel state is not changed for a while. Based on this fact, as depicted in the Figure 4, the evaluation method of the number of consecutive IS or OOS or new state indications needs to be revisited by RAN2.

Figure 4. Impact on the evaluation of the number of consecutive IS or OOS or new state indications

Proposal 2: RAN2 is kindly requested to enhance the mechanism to start timer T310 or declare RLF when the UE is unable to detect RLM-RS continually.

Conclusions

Based on the given above, we have the following Proposals：

Proposal 1: Considering that the UE may not detect RLM-RS due to LBT, radio link quality indication mechanism should be enhanced via the following alternatives：

- A new state indicated to higher layer is suggested to be defined based on the number of undetected RLM-RS.

- The criterion of in-sync and out-of-sync indication should be properly enhanced with taking the number of undetected RLM-RS into account.

Proposal 2: RAN2 is kindly requested to enhance the mechanism to start timer T310 or declare RLF when the UE is unable to detect RLM-RS continually.

Reference

[1] 3GPP RP-182878, “New WID on NR-based Access to Unlicensed Spectrum”, Qualcomm, RAN#82, December, 2018.

[2] 3GPP TS 38.133, “Requirements for support of radio resource management”.

[3] 3GPP TS 38.213, “Physical layer procedures for control”.

[4] 3GPP TS 38.331, “Radio Resource Control (RRC) protocol specification”.

[5] 3GPP R1-1901676, “Discussion on enhancements to initial access procedure”, vivo, RAN1#96, February, 2019.