3GPP TSG-RAN WG2 Meeting #105	R2-1900440

Athens, Greece, 25 Feb - 01 Mar 2019

Agenda item:	11.2.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Considerations on mobility and measurement gap enhancements in NR-U

WID/SID:	NR-U - Release 16

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we discuss one of the proposed enhancements for increasing robustness in connected mode mobility. We also address the need for measurement gap modifications in unsynchronized scenarios.

# 2	Discussion

## Robustness for Mobility Procedures

The NR-U WID [1] the Mobility procedures to be designed for NR-U are described as follows:

The technical report to the study item [TR 38.889], detailed the solutions to be studied for mobility as:

While not mentioned by name, the above highlighted text points to conditional handover (CHO).

Our view is that the applicability and possible NR-U specific enhancements of CHO can only be investigated after the concepts and the procedures for CHO have been agreed in NR mobility enhancements work item to avoid parallel work.

Proposal 1: Address condition handover in NR-U towards the end of the WI, to allow to re-use the decisions and agreements concerning CHO made in NR mobility enhancements work item.

## Measurement Gaps

In order for a UE to perform RRM measurements of other cells on other frequencies, measurement gaps can be configured in which the UE does not need to receive from or transmit to its own cell.

The possible gap patterns in NR (as defined in 38.133 table 9.2.2-1) have a maximum length of 6 msec, and minimum repetition period of 20 msec and are designed to allow capturing the SSB(s) of a neighbouring cell.

In NR-U neighbouring cells may not be synchronized. Our view is that some enhancements in the measurement gap configuration (e.g. increasing the maximum measurement gap length) to increase the probability that the UE is able to capture the DRS of neighbouring cells may be beneficial.

Proposal 2: Study what changes of the measurement gap configuration are needed to support reliable measurements of unsynchronized neighbouring NR-U cells.

# 2	Conclusion

In this paper we have the following proposals:

Proposal 1: Address condition handover in NR-U towards the end of the WI, to allow to re-use the decisions and agreements concerning CHO made in NR mobility enhancements work item.

Proposal 2: Study what changes of the measurement gap configuration are needed to support reliable measurements of unsynchronized neighbouring NR-U cells.

# 3	References

- RP-182878, New WID on NR-based Access to Unlicensed Spectrum, Qualcomm Inc.