3GPP TSG-RAN WG2 Meeting #105	 R2-1900442

Athens, Greece, 25 Feb - 1 Mar 2019

Agenda Item:	11.10.4

Source:	Mediatek Inc.

Title:	Fast SCell Activation for Data Transmission

Document for:	Discussion, Decision

# Introduction

In RAN#80 meeting, the Rel-16 work item of ‘DC and CA enhancements’ was approved. One of the main objectives is:

Efficient and low latency serving cell configuration/activation/setup: Minimizing signalling overhead and latency needed for initial cell setup, additional cell setup and additional cell activation for data transmission. [RAN2, RAN1, RAN4, RAN3] - This objective applies to MR-DC, NR-NR DC and CA - The objective should consider enhancements when starting from IDLE, INACTIVE mode and CONNECTED mode The work on objective 2 and 3 should consider UEs in all states (i.e. IDLE, INACTIVE and CONNECTED).

In this contribution, we analyse the latency for SCell activation in NR and discuss the applicability of the solutions for LTE Rel-15 euCA to enable fast SCell activation for data transmission.

# Discussion

## 2.1 SCell Activation Latency

The overall procedures for setting up MR-DC and CA is illustrated in our accompanied contribution [1]. The SCell activation timeline is illustrated in Figure 1. The procedures from IDLE/INACTIVE to CONNECTED transition to the RRC reconfiguration procedure with measurement configuration are not shown. Figure 1 show the delay starting from initial MR-DC/CA setup.

Figure 1 SCell Activation Latency

Usually, SCG or SCells are configured by the network based on the measurement reports provided by the UE through RRC Reconfiguration procedure.  After UE SCell is configured, the SCells will be activated through MAC-CE. Typically, the measurement reports for the SCells are required to help the network make decide to activate the SCells. The overall latency comprises of:

- RRC processing delay for the RRCReconfiguration message;

- The time period waiting for SCell activation MAC CE

- SCell activation delay as specified in TS38.133 [2], which starts from receiving MAC CE for activation and until UE transmits valid CSI report and applies actions related to the activated SCell.

In NR, the RRC processing delay for RRCReconfiguration with SCell addition/release is 16ms.

The SCell activation delay includes following elements:

- delay for MAC CE decoding (THARQ),

- SCell activation delay (Tactivation_time): It consists of the time duration for RF preparation, ACG tuning, cell search, fine time tracking as well as the extra margin for post-processing.

- CSI reporting delay (TCSI_reporting).

In NR, the SCell activation delay is mainly determined by N times of SMTC periodicity. For FR1, N equals to 1 or 2 for the cases that the SCell is known and unknown respectively. For FR2, N has not been completely decided for all cases. CSI reporting delay mainly consider the time duration for CSI measurement, processing and waiting for the first available UL resource.

In LTE the SCell activation delay is fixed, which is 24ms or 34ms depending on whether the prior SCell measurements are not available or not. In NR, the SCell activation delay is variable, determined by SMTC periodicity, FR1 or FR2, as well as the different scenarios, which can be much longer than the delay in LTE. Taking FR1 with SMTC periodicity of 20ms as example, if the SCell is known, the SCell activation delay can be as long as 85ms.

Observation 1: In NR, the RRC processing delay for RRCReconfiguration with SCell addition/release is 16ms.

Observation 2:  In NR, the SCell activation delay is variable and can be much longer than the delay in LTE.

## 2.2 SCell Activation at SCell Configuration

In LTE euCA, configuring SCell directly in activated/deactivated state is supported, which allows direct SCell activation in any occasion when deactivated SCell can be (re)configured. In NR, the SCell is considered to be in deactivated when it is configured. Direct SCell activation in SCell configuration can help to speed up the activation for data transmission. So the latency of waiting for MAC CE and the HARQ processing delay for the MAC CE can be saved. When a SCell is configured, the activated/deactivated state can be indicated.

Observation 3: Direct SCell activation in SCell configuration can help to reduce the overall SCell activation latency.

Proposal 1: Indicate SCell directly in activated/deactivated state when a SCell is (re)configured.

If SCell can be activated directly in SCell configuration, the timing when UE is able to use the activated SCell should be clarified. It relates to both the RRC processing delay and the RAN4 requirement for SCell activation delay. In LTE euCA, if the RRC configures the SCell with activation state in TTI n, UE considers the SCell is in activated state in n+20, i.e. after RRC processing delay. In NR, RAN4 performance requirements for “configuring activated SCell” may be needed. Since the HARQ processing for Activation MAC CE is omitted, THARQ in Scell activation delay can be replaced by TRRCprocessingDelay. This should be confirmed by RAN4.

Proposal 2: Send LS to RAN4 to confirm that SCell activation delay by RRC configuration message is TRRCprocessingDelay+ Tactivation_time + TCSI_Reporting.

## 2.3 BWP without PDCCH Monitoring

In LTE euCA, the concept of ‘dormancy’ is introduced for SCell, i.e. a new ‘Dormant SCell state’ is supported in addition to ‘activated’ and ‘deactivated’ state.  For the SCell in dormant state, UE doesn’t monitor PDCCH and only perform period CQI report sparsely.

Without dormant SCell state, the SCell should be deactivated when there is no data to be scheduled. Then the SCell will be activated again by MAC CE later when additional radio resources are required. The SCell activation delay from deactivated to activated state is long, during which UE initiates CSI measurement and sends CSI reports. The network can start scheduling on the SCell only after the initial CSI report is received. The frequent transition between activated state and deactivated state is not efficient to offload the bursty traffic.

If the Scell is kept in dormant state, the SCell activation delay can be reduced to 8ms, since the delay for initial CSI measurement and reporting is reduced by continuous CSI report.  Compared with the deactivated state, more UE power consumption is expected due to the continuous CSI measurement and report. So the resources for CSI report is configured sparsely to balance the UE power consumption. In NR, the concept of ‘dormancy’ is still useful to reduce the SCell activation latency, especially considering the uncertainty in time duration to acquire the first available UL resource for CSI reporting.

Observation 4: The dormant state can help to reduce the SCell activation delay by continuous CSI measurement and reporting. UE power consumption can be balanced by sparse CSI resource allocation.

Proposal 3: The ‘dormancy’ behaviour, that UE doesn’t monitor PDCCH and only performs CSI measurement and reporting, is supported in Rel-16 to reduce the SCell activation delay.

In NR, BWP operation is supported. Data transmission/reception are performed per BWP on top of the activated serving cells.  The BWP switching for a serving Cell is used to activate an inactive BWP and deactivate an active BWP. So all the physical channels including PDCCH, PDSCH, PUCCH, PUSCH, PRACH, etc are configured per BWP instead of per cell. The BWP switching is controlled by DCI, by the bwp-InactivityTimer, by RRC signalling, or by the MAC entity itself upon initiation of Random Access procedure.

Observation 5: In NR, BWP operation is introduced and data transmission/reception are performed per BWP on top of the activated serving cells.

If the concept of ‘Dormancy’ is introduced in NR, how to realize it and make it compatible with BWP operation should be considered.

One option is to define the ‘dormancy’ on the level of serving cell, same as LTE. In NR, the first active BWP is defined, which is the BWP UE perform data transmission/reception upon SCell activation. If the SCell is in dormant state, UE only needs to perform CSI measurement and reporting for the first active BWP.  In this option, the state transitions among activated, dormant and deactivated should be supported. It will requires new MAC CEs and new procedures in MAC layer, which is very complicated.

Observation 6: If dormant state is introduced on the level of serving cells (e.g. dormant SCell), new MAC CEs and new procedures in MAC layer to support state transitions among activated, dormant and deactivated should be specified.

The other option is to realize the ‘dormancy’ behaviours on the level of BWP.  Actually, suspending PDCCH monitoring doesn’t necessarily need to create a new SCell state in NR. A particular BWP without PDCCH/PDSCH configuration can be configured to effectively realize the same behaviour as in dormant state. When the network intends to make UE only perform CSI measurement and reporting without PDCCH monitoring, BWP switching to the particular BWP can be performed.

Basically, only a particular BWP without PDCCH/PDSCH is configured by RRC. BWP switching with DCI can be used to send UE to the particular BWP without PDCCH monitoring. No new signalling and procedures are needed in both PHY and MAC layer. Furthermore, compared with the SCell activation delay from dormant state to activated state, the activation latency can be further reduced since the BWP switching delay is shorter compared with SCell state transition, which is only 2-3 ms.

Observation 7: If dormancy behaviour is introduced on the level of BWPs (i.e. configuring a particular BWP without PDCCH/PDSCH), no new signalling and procedures are needed on MAC layer to support different SCell state transition. BWP switching can further reduce the activation latency.

Proposal 4: A particular BWP without PDCCH/PDSCH can be configured.

Proposal 5: BWP switching to the particular BWP without PDCCH/PDSCH is used to realize ‘dormancy’.

# Conclusion

In this contribution, we analyse the latency for SCell activation in NR and discuss the applicability of the solutions for LTE Rel-15 euCA.

We have following observations compared with LTE CA enhancement.

Observation 1: In NR, the RRC processing delay for RRCReconfiguration with SCell addition/release is 16ms.

Observation 2:  In NR, the SCell activation delay is variable and can be much longer than the delay in LTE.

Observation 3: Direct SCell activation in SCell configuration can help to reduce the overall SCell activation latency.

Observation 4: The dormant state can help to reduce the SCell activation delay by continuous CSI measurement and reporting. UE power consumption can be balanced by sparse CSI resource allocation.

Observation 5: In NR, BWP operation is introduced and data transmission/reception are performed per BWP on top of the activated serving cells.

Observation 6: If dormant state is introduced on the level of serving cells (e.g. dormant SCell), new MAC CEs and new procedures in MAC layer to support state transitions among activated, dormant and deactivated should be specified.

Observation 7: If dormancy behaviour is introduced on the level of BWPs (i.e. configuring a particular BWP without PDCCH/PDSCH), no new signalling and procedures are needed on MAC layer to support different SCell state transition. BWP switching can further reduce the activation latency.

Based on the above observations, we propose:

Proposal 1: Indicate SCell directly in activated/deactivated state when a SCell is (re)configured.

Proposal 2: Send LS to RAN4 to confirm that SCell activation delay by RRC configuration message is TRRCprocessingDelay+ Tactivation_time + TCSI_Reporting.

Proposal 3: The ‘dormancy’ behaviour, that UE doesn’t monitor PDCCH and only performs CSI measurement and reporting, is supported in Rel-16 to reduce the SCell activation delay.

Proposal 4: A particular BWP without PDCCH/PDSCH can be configured.

Proposal 5: BWP switching to the particular BWP without PDCCH/PDSCH is used to realize ‘dormancy’.

# Reference

[1] R2-1900437, Early measurement reporting from IDLE/INACTIVE to CONNECTED, Mediatek Inc.

[2] TS38.133 v15.4.0