3GPP TSG-RAN WG2 #105	Tdoc R2-1900549

Athens, Greece, 25th February - 1st March 2019

Agenda Item:	11.6.4.2

Source:	Ericsson

Title:	RRM for NTN

Document for:	Discussion

# Introduction

In RAN#80, a new SI “Solutions for NR to support Non-Terrestrial Network” was agreed [1]. It is a continuation of the preceding SI “NR to support Non-Terrestrial Networks” (RP-171450), where the objective was to study the channel model for the non-terrestrial networks, to define deployment scenarios, parameters and identify the key potential impacts on NR. The new study item that had update SID [3] has the objective at evaluating potential solutions addressing the minimum necessary identified key impact areas from the previous activity and to study impact on RAN protocols/architecture. The objectives for layer 2 and above are:

- Study the following aspects and identify related solutions if needed: Propagation delay: Identify timing requirements and solutions on layer 2 aspects, MAC, RLC, RRC, to support non-terrestrial network propagation delays considering FDD and TDD duplexing mode. This includes radio link management. [RAN2] - Handover: Study and identify mobility requirements and necessary measurements that may be needed for handovers between some non-terrestrial space-borne vehicles (such as Non Geo stationary satellites) that move at much higher speed but over predictable paths [RAN2, RAN1] - Dual connectivity [RAN3 aspects] involving - NTN-based NG-RAN (Transparent GEO or LEO satellites) and terrestrial based NG-RAN access: Xn terminated on the ground - or two NTN-based NG-RAN access (between Regenerative LEO satellites): Xn over ISL - Architecture: Identify needs for the 5G’s Radio Access Network architecture to support non-terrestrial networks (e.g. handling of network identities) [RAN3] - Paging: procedure adaptations in case of moving satellite foot prints or cells Note: - This new study item does not address regulatory issues.

In RAN2#103bis, it is agreed to study the following UP and CP aspects:

UP Impacts to study

- DRX

- HARQ

- Random access response

- RLC/PDCP reordering (e.g. timers and SN space)

- SDAP => no impact

Impacts to study for CP

- Mobility

- TA management and update

In RAN2#104, the following is agreed for mobility:

Agreements:

1.	Satellite beams, satellites or satellite cells are not considered to be visible from UE perspective in NTN SI.  This does not preclude differentiating at the PLMN level the type of network (e.g. NTN vs. terrestrial).  This is up to SA2.

2.   Revise the current definition of satellite cell in TR 38.821 and refer to a satellite beam.  Definition of satellite beam can be discussed during email discussion.

3.	Add text in TR 38.821 stating that association between NR PCI and NR SSBs is left for implementation (i.e. it will not be specified)

4.	Consider Rel-15 definitions as a baseline for NTN

5.	Both option a and b can be considered in NTN SI with one or multiple SSBs per PCI.  The TR will capture a figure for both option.

[104#54][NR - NTN] Mobility  (Ericsson)

Intended outcome:

-	Stage 1: Identify key issues to address for mobility

-	Stage 2: Capture solutions identified in contributions for each of the issues

Deadline:  Thursday 2019-02-16

The email discussion 104#54 on mobility had specific questions for measurement aspects but the replies reflected that more discussion and study are needed before actual solutions can be captured. In this paper, we present our views for the RRM aspects for NTN systems.

# Discussion

The measurement configuration includes the following parameters as specified in TS 38.331:

1.	Measurement objects (MO): A list of objects on which the UE shall perform the measurements.

-	For intra-frequency and inter-frequency measurements a measurement object indicates the frequency/time location and subcarrier spacing of reference signals to be measured. Associated with this measurement object, the network may configure a list of cell specific offsets, a list of 'blacklisted' cells and a list of 'whitelisted' cells. Blacklisted cells are not applicable in event evaluation or measurement reporting. Whitelisted cells are the only ones applicable in event evaluation or measurement reporting.

-	The measObjectId of the MO which corresponds to each serving cell is indicated by servingCellMO within the serving cell configuration.

-	For inter-RAT E-UTRA measurements a measurement object is a single E-UTRA carrier frequency. Associated with this E-UTRA carrier frequency, the network can configure a list of cell specific offsets, a list of 'blacklisted' cells and a list of 'whitelisted' cells. Blacklisted cells are not applicable in event evaluation or measurement reporting. Whitelisted cells are the only ones applicable in event evaluation or measurement reporting.

2.	Reporting configurations: A list of reporting configurations where there can be one or multiple reporting configurations per measurement object. Each reporting configuration consists of the following:

-	Reporting criterion: The criterion that triggers the UE to send a measurement report. This can either be periodical or a single event description.

-	Reference Signal (RS) type: The RS that the UE uses for beam and cell measurement results (e.g. SS/PBCH block or CSI-RS).

-	Reporting format: The quantities per cell and per beam that the UE includes in the measurement report (e.g. RSRP) and other associated information such as the maximum number of cells and the maximum number beams per cell to report.

3.	Measurement identities: A list of measurement identities where each measurement identity links one measurement object with one reporting configuration. By configuring multiple measurement identities, it is possible to link more than one measurement object to the same reporting configuration, as well as to link more than one reporting configuration to the same measurement object. The measurement identity is also included in the measurement report that triggered the reporting, serving as a reference to the network.

4.	Quantity configurations: The quantity configuration defines the measurement filtering configuration used for all event evaluation and related reporting, and for periodical reporting of that measurement. For NR measurements, the network may configure up to 2 quantity configurations with a reference in the NR measurement object to the configuration that is to be used. In each configuration, different filter coefficients can be configured for different measurement quantities, for different RS types, and for measurements per cell and per beam.

5.	Measurement gaps: Periods that the UE may use to perform measurements.

A UE in RRC_CONNECTED maintains a measurement object list, a reporting configuration list, and a measurement identities list according to signalling and procedures in this specification. The measurement object list possibly includes NR measurement object(s) and inter-RAT objects. Similarly, the reporting configuration list includes NR and inter-RAT reporting configurations.

# Measurements assuming UE is able to determine its location

As the cell size in NTN, especially for GEO, is relatively large, the location based RRM reporting or location reporting has been discussed. In this SI, both UEs with and without GNSS capability are considered and these location discussions assume that UE is capable of determining its location. First obvious discussion point is whether the LTE location reporting could be assumed to be in place in NR. In LTE, the UE includes the location in RSRP/RSRQ measurement reports if the location info is available at the UE and if the reporting configurations have set the flag of including location to on. That is, if NR NTN supports piggybacking location information in RSRP/RSRQ based reporting, the network would get to know UE location when those measurements are triggered, whether event based or periodically. If the network is primarily interested in UE location, sending always also RSRP/RSRQ measurements e.g. in periodical reports increases the UL traffic amount.

- If NR NTN supports piggybacking location information in RSRP/RSRQ based reporting, the network would get to know UE location when those measurements are triggered.

- If the network is primarily interested in UE location, sending always also RSRP/RSRQ measurements e.g. in periodical reports increases the UL traffic amount.

In order for the network to receive UE location for example when UE approaches the presumed cell edge, the triggering of the measurement report needs to consider the UE location. The triggering could depend only on UE location versus a reference location, or it could be an event that considers a combination of RSRP/RSRQ and a location. This also limits the sending of RSRP reports in UL only when the UE is really on cell edge. This could be achieved also by conditioning a reporting configuration to a reference location, e.g distance from the cell center.

RAN2 to study the benefits of conditioning the triggering of measurement results on UE location.

# Location accuracy for NTN

Longitude

Latitude

Ref. location 1

Ref. location 2

Ref. location 3

Ref. location 4

A periodic report of location data estimated with GNSS or a non-GNSS method can cause heavy signaling load to the network, and the expected low uplink data rates in NTN make a design with frequent high-resolution location reporting unfeasible. In particular, uplink reporting for NTN is challenging since UEs are more power limited in comparison to the satellite transmit power. In order to limit the reporting overhead due to fine resolution of location expression, one alternative is to define a set of reference locations. For example, for LEO, these could be derived using the ephemeris data. One coarse example is illustrated in the figure below, where the UE reports its closest location (1-4). The closest location report can be interpreted visually as that the network knows what hexagon the UE is inside.

- Frequent location reporting with fine resolution can potentially cause a lot of UL overhead.

RAN2 to study coarse location reporting.

# Conclusion

We made the following observations:

Observation 1	If NR NTN supports piggybacking location information in RSRP/RSRQ based reporting, the network would get to know UE location when those measurements are triggered.

Observation 2	If the network is primarily interested in UE location, sending always also RSRP/RSRQ measurements e.g. in periodical reports increases the UL traffic amount.

Observation 3	Frequent location reporting with fine resolution can potentially cause a lot of UL overhead.

We propose the following:

Proposal 1	RAN2 to study the benefits of conditioning the triggering of measurement results on UE location.

Proposal 2	RAN2 to study coarse location reporting.

# References

RP-181370, New SID on Solutions for NR to support Non Terrestrial Network, Thales, RAN#80, La Jolla

R3-186269, TR 38.821, Ver 020

RP-182880, Study on solutions for NR to support non-terrestrial networks (NTN)