3GPP TSG-RAN WG2 #105	R2- 1900553

# Athens, Greece, 25 Feb – 1 March 2019	                           Revision of R2-1817248

Agenda Item:	11.7.2.2

Source:	OPPO, vivo

Title:	Scheduling enhancements for TSN network

Document for:	Discussion, Decision

Introduction

In RAN #81, a revised SID on Study on NR Industrial Internet of Things (IoT) is approved. The potential enhancement for TSN network on scheduling enhancement by using TSN traffic patterns are discussed and captured in this study item, as shown in the following [1]:

2)	Time Sensitive Networking related enhancements:

b)	Enhancements (e.g. for scheduling) to satisfy QoS for wireless Ethernet when using TSN traffic patterns as specified in TR 22.804 (RAN2/RAN1). Note: RAN2 to start the work, RAN1 to take action based on RAN2 progress.

And one question is provided in RAN2 latest meeting, which is shown as below:

- FFS if we use DL SPS

In this paper, we will discuss the issue about scheduling enhancement for TSN traffic and give our proposals.

Discussion

As mentioned in TR 22.804 clause 8.1 and the LS sent out on TSN requirement evaluation, RAN2 will focus on the following use cases and KPIs which reaches to the most stringent requirement:

| Case | #UE | Communications service availability | Transmit period | Allowed E2E latency | Survival time | Packet size | Service area | Traffic periodicity | Use case |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I | 20 | 99,9999% to 99,999999% | 0.5 ms | ≤ Transmit period | Transmit period | 50 bytes | 15 m x 15 m x 3 m | Periodic | Motion control and control-to-control use cases |
| II | 50 | 99,9999% to 99,999999% | 1 ms | ≤ Transmit period | Transmit period | 40 bytes | 10 m x 5 m x 3 m | Periodic | Motion control and control-to-control use cases |
| III | 100 | 99,9999% to 99,999999% | 2 ms | ≤ Transmit period | Transmit period | 20 bytes | 100 m x 100 m x 30 m | Periodic | Motion control and control-to-control use cases |
| IV | N/A | 99.9999% | N/A | < 1 ms | N/A | N/A, but service bit rate from 150 kbit/s to 4.61 Mbits/s | N/A | Aperiodic | Audio streaming for live performance |

Table1: TSN use cases with the stringent requirement

From Table 1, it is observed that all traffics are belong to deterministic traffic pattern, and most of them are with periodic characteristic. By saying deterministic traffic pattern, it means that the delay of message delivery is bounded, i.e. message should be arriving at 5GS in the fixed duration (i.e. ingress window), and message should be leaving 5GS in a fixed duration (i.e. egress window). In addition, TSN traffic exists both in uplink direction and in downlink direction according to the description in TR 22.804. Thus, by considering the low latency tolerance requirement, e.g. 0.5ms, and the minimum period of 2 symbol granularity supported in current NR spec, using CG is the most straightforward method for such TSN traffic in UL direction to assure the required QoS.

- TSN traffic targets at 0.5ms latency requirement.

CG scheduling is supported for TSN traffic in UL direction to meet TSN traffic QoS requirement.

In DL direction, it is undetermined to adopt SPS scheduling or dynamic scheduling for TSN traffic. The pros and cons for the two potential solutions are listed in the following table:

|  | Pros | Cons |
| --- | --- | --- |
| SPS scheduling is adopted | - Fulfill latency requirement - Fast processing due to no PDCCH detection. - Lessen PDCCH monitoring overhead to match long period service or aperiodic service | - Time-frequency resource reserved, and sparse resource left for other traffics. |
| Dynamic scheduling is adopted | - Fulfill latency requirement as gNB knows the arrival time of DL packets | - Large PDCCH overhead - Reliability requirement on PDCCH decoding. |

Table2: the analysis on SPS scheduling and dynamic scheduling for DL TSN traffic

From the table above, it can be clearly seen that more advantage can be achieved if SPS scheduling is supported for DL TSN traffic. Thus, we prefer to configure SPS for DL TSN traffic transmission. Since the minimum SPS period is 10ms, and the straightest TSN traffic latency is 0.5ms, shorter SPS period should be introduced for DL TSN traffic.

- Current SPS periodicity configuration cannot fully match TSN traffic requirement.

Introduce shorter SPS periodicities for DL TSN traffic.

SPS scheduling with shorter periodicity should be supported for TSN traffic in DL direction to meet TSN traffic QoS requirement.

According to RAN1 LS, RAN1 has already agreed to support multiple CG configurations: “Multiple active configured grant configurations for a given BWP of a serving cell should be supported at least for different services/traffic types and/or for enhancing reliability and reducing latency”. Since TSN traffic are symmetric-like traffic at the most case, it can be concluded that multiple active SPS configurations may also be supported on DL direction. Considering this feature are not previously specified in LTE or NR, details to support multiple SPS configuration should be considered by RAN1 and RAN2.

- Multiple SPS configuration is applicable at least for the different services/traffic types.

Multiple SPS configuration should be supported for TSN traffic.

FFS how RAN1/RAN2 support multiple SPS configuration.

Furthermore, one scenario on TSN message periodicities with non-integer multiple of NR supported periodicities is also mentioned in NR IIoT. We think this misalignment could happen in some cases which is one of the contributors for latency. But we also doubt how much problematic it is, and it can be resolved by SPS/CG configuration reconfiguration.

- The misalignment between TSN periodicity and CG/SPS periodicity is evitable and it seems to be solved by existing SPS/CG reconfiguration.

However, if SPS/CG configuration reconfiguration is proved insufficient for such mismatches eventually, some potential solutions can be considered:

- Multiple SPS/CG configuration.

- Finer granularity CG/SPS configuration

- Flexible SPS/CG configuration

E.g., Specific time-frequency resource pattern is provided to UE from the network, which is used to indicate each transmission point.

E.g. UE calculates each transmission point based on specific information, i.e. traffic period, which is indicated from the network.

If needed, RAN2 consider solutions for misalignment issue between TSN periodicity and CG/SPS periodicity, including flexible SPS/CG configuration, finer granularity CG/SPS configuration, Multiple SPS/CG configuration, etc.

The knowledge of TSN traffic pattern could be provided to the gNB either from the UE or from the Core Network. Both the UE and the Core network can obtain the information from CNC or other bridges on the same TSN link, according to 802.1qcc. Since similar information is already provided as part of UE Assistance Information message in LTE R15, it can be easily extend for TSN pattern reporting. In addition, recommended configured grant configuration can also be included in UE assistant information for more preferred/proper time-frequency resource reservation for such TSN traffic. For example, UE can report some traffic related information, e.g. UL traffic pattern, recommended configured grant configuration, to help more proper resource configuration.

- TSN traffic pattern can be provided to gNB from the UE and the network.

UE Assistance information is recommended for more proper resource configuration for TSN traffic, including TSN traffic pattern, recommended configured grant configuration, etc.

Conclusion

Based on the discussion above, we made the following observations:

Observation 1	TSN traffic targets at 0.5ms latency requirement.

Observation 2	Current SPS periodicity configuration cannot fully match TSN traffic requirement.

Observation 3	Multiple SPS configuration is applicable at least for the different services/traffic types.

Observation 4	The misalignment between TSN periodicity and CG/SPS periodicity is evitable and it seems to be solved by existing SPS/CG reconfiguration.

Observation 5	TSN traffic pattern can be provided to gNB from the UE and the network.

And propose the following:

Proposal 1	CG scheduling is supported for TSN traffic in UL direction to meet TSN traffic QoS requirement.

Proposal 2	Introduce shorter SPS periodicities for DL TSN traffic.

Proposal 3	SPS scheduling with shorter periodicity should be supported for TSN traffic in DL direction to meet TSN traffic QoS requirement.

Proposal 4	Multiple SPS configuration should be supported for TSN traffic.

Proposal 5	FFS how RAN1/RAN2 support multiple SPS configuration.

Proposal 6	If needed, RAN2 consider solutions for misalignment issue between TSN periodicity and CG/SPS periodicity, including flexible SPS/CG configuration, finer granularity CG/SPS configuration, Multiple SPS/CG configuration, etc.

Proposal 7	UE Assistance information is recommended for more proper resource configuration for TSN traffic, including TSN traffic pattern, recommended configured grant configuration, etc.

References

RP-182090, Revised SID: Study on NR Industrial Internet of Things (IoT), 3GPP RAN#81, Gold Coast, Australia, September 10-13, 2018

TR 22.804, Study on Communication for Automation in Vertical Domains

R2-1816043, LS on TSN requirements evaluation, contact: Nokia

R2-1818991, LS on multiple active configured grant configurations, Source: RAN1