3GPP TSG-RAN WG2 #105	Tdoc R2-1900655

Athens, Greece, 25 Feb – 1 Mar 2019

Agenda Item:	11.12.2

Source:	Ericsson

Title:	MDT for RRC_INACTIVE state

Document for:	Discussion, Decision

# 1	Introduction

In the study item, “Study on RAN-centric data collection and utilization for LTE and NR (FS_LTE_NR_data_collect)”, Minimization of Drive Tests (MDT) is one of the use cases to be studied for NR and LTE enhancement. MDT for NR has not been specified in the involved standards.

# 2	Background

MDT was firstly studied in TR 36.805 [1] in Rel-9 driven by RAN2 with the purpose to minimize the actual drive tests. MDT has been introduced since Rel-10 in LTE. MDT has not been specified for NR in the involved standards in RAN2, RAN3 and SA5 groups.

In the TS 37.320 [2], the following measurements are specified for MDT in LTE.

| MDT mode | RRC states | Measurement quantities |
| --- | --- | --- |
| Logged MDT | RRC_IDLE | RSRP and RSRQ of the serving cell and available UE measurements for intra-frequency/inter-frequency/inter-RAT, time stamp and location information. |
| Immediate MDT | RRC_CONNECTED | M1: RSRP and RSRQ measurement by UE. M2: Power Headroom measurement by UE. M3: Received Interference Power measurement by eNB. M4: Data Volume measurement separately for DL and UL, per QCI per UE, by eNB. M5: Scheduled IP Throughput for MDT measurement separately for DL and UL, per RAB per UE and per UE for the DL, per UE for the UL, by eNB. M6: Packet Delay measurement, separately for DL and UL, per QCI per UE, see UL PDCP Delay, by the UE, and Packet Delay in the DL per QCI, by the eNB. M7: Packet Loss rate measurement, separately for DL and UL per QCI per UE, by the eNB. M8: RSSI measurement by UE. M9: RTT measurement by UE. |
| Logged MBSFN MDT | RRC_IDLE, RRC_CONNECTED | RSRP, RSRQ, MBSFN RSRP, MBSFN RSRQ, BLER for signalling and BLER for data per MCH. |
| Enhancement to RLF | RRC_CONNECTED | RLF report with detailed location information (e.g., GNSS) if available; RLF reports may also include available WLAN measurement results and/or Bluetooth measurement results for calculating UE location. |

# 3	MDT for RRC_INACTIVE UE

The RRC_INACTIVE state has been introduced in Rel-15 in NR to reduce the latency and the number of signals of the connection establishment. A UE in RRC_INACTIVE shall obtain normal service when camping on a suitable cell and operator service when camping on a reserved cell. The UE in RRC_INACTIVE shall perform cell reselection and the required measurements on the serving cell and neighbor cells, similar to that for the UE in RRC_IDLE state.

The DL pilot strength measurement (i.e., RSRP/RSRQ) of Logged MDT is collected based on the existing measurements required for cell reselection purpose, without imposing UE to perform additional measurements. The logged measurement report consists of measurement results for the serving cell, available UE measurements performed in idle for intra-frequency/inter-frequency/inter-RAT, time stamp and location information.

The measurement logging in Logged MDT for UE in RRC_IDLE is based on the measurements required by the cell reselection purpose. For a UE in RRC_INACTIVE, the measurement requirements for cell reselection purpose are same as in RRC_IDLE.

For immediate MDT, the configuration is based on the existing RRC measurement procedures of RRC_CONNECTED state for configuration and reporting with some extensions for location information.

The existing procedures of configuration and reporting for immediate MDT are not applicable to RRC_INACTIVE state.

The table below shows a summary on the comparation between RRC_IDLE and RRC_INACTIVE.

TABLE 1. A summary on the comparation between RRC_IDLE state and RRC_INATIVE state

|  | RRC_IDLE | RRC_INACTIVE |
| --- | --- | --- |
| Supported Service | Limited service; normal service; operator service | normal service; operator service |
| UE behaviors when UE is in Camped Normally state, i.e., camping on a suitable cell and obtain the normal service (common for both) | When camped normally, the UE shall perform the following tasks defined in TS38.304 [3]: - monitor the paging channel of the cell as specified in clause 7 according to information broadcast in SIB1; - monitor Short Messages transmitted with P-RNTI over DCI as specified in clause 6.5 in TS 38.331; - monitor relevant System Information as specified in TS 38.331; - perform necessary measurements for the cell reselection evaluation procedure; - execute the cell reselection evaluation process. |  |
| Requirements on UE measurements | According to TS 38.133, the requirements below are common for both RRC_IDLE and RRC_INACTIVE during the cell reselection procedure. - UE measurement capability - Measurement and evaluation of serving cell - Measurement of intra-frequency NR cells - Measurement inter-RAT E-UTRAN cells |  |
| UE behaviors when UE is in Any Cell Selection state (common for both) | UE shall attempt to find a suitable cell by following the cell selection process. If failed after a complete scan of all RATs and all frequency bands supported by the UE, it shall attempt to find an acceptable cell of any PLMN to camp on. |  |
| UE behaviors when UE is in Camped on Any Cell state | UE is camped on an acceptable cell to obtain the limited service. | Not applicable (UE shall go to Idle mode when it finds an acceptable cell.) |
| RNA update | Not applicable | In RRC_INACTIVE state, if the new cell does not belong to the configured RNA during the cell reselection procedure, an RNA update procedure is performed. |

Based on the discussions and observations, we have the following proposals.

Apply the Logged MDT configuration, logged measurements and reporting procedures to RRC_INACTIVE state.

For the Logged MDT, the UE receives the MDT configurations including loggingduration and logginginterval, as discussed in Section 3. In LTE, the loggingduration defines a timer activated at the moment of configuration, that continues independent of state changes, RAT or RPLMN change. The logginginterval should be configured in seconds in multiples of the applied IDLE mode Discontinuous Reception (DRX). The loggingduration and logginginterval configured by Element Manager and the default DRX in LTE are shown below.

LoggingDuration-r10 ::= ENUMERATED { min10, min20, min40, min60, min90, min120, spare2, spare1}

LoggingInterval-r10 ::= ENUMERATED { ms1280, ms2560, ms5120, ms10240, ms20480, ms30720, ms40960, ms61440}

defaultPagingCycle ENUMERATED { rf32, rf64, rf128, rf256},

Rel-15 introduces a RAN-based notification area (RNA) managed by RAN for RRC_INACTIVE state. A UE in RRC_INACTIVE can be configured with DRX for RAN paging by RAN (ran-PagingCycle).

For UE in Idle mode, the DRX which the measurement requirement and MDT logging are based on is determined by the shortest of the UE specific DRX value, if configured by RRC or upper layers, and a default DRX value broadcast in system information. If UE specific DRX is not configured by RRC or by upper layers, the default value is applied.

For UE in RRC_INACTIVE state, a shorter ran-PagingCycle in the dedicated signal than defaultPagingCycle in system information requires UE to perform measurement requirement more frequently based on ran-PagingCycle. But the management system which configures the LoggingInterval may not be able to know the ran-PagingCycle determined by RAN node. In this case, the LoggingInterval is not possible to be configured as small as the ran-PagingCycle. However, we don’t see this can be the reason that requires to separate the MDT configurations for RRC_IDLE and RRC_INACTIVE.

The details of the configuration message, LoggedMeasurementConfiguration, for Logged MDT defined in TS 36.331 [4] in LTE are shown below.

-- ASN1START

LoggedMeasurementConfiguration-r10 ::=	SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

loggedMeasurementConfiguration-r10		LoggedMeasurementConfiguration-r10-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture				SEQUENCE {}

}

}

LoggedMeasurementConfiguration-r10-IEs ::= SEQUENCE {

traceReference-r10				TraceReference-r10,

traceRecordingSessionRef-r10	OCTET STRING (SIZE (2)),

tce-Id-r10						OCTET STRING (SIZE (1)),

absoluteTimeInfo-r10			AbsoluteTimeInfo-r10,

areaConfiguration-r10			AreaConfiguration-r10		OPTIONAL,	-- Need OR

loggingDuration-r10				LoggingDuration-r10,

loggingInterval-r10				LoggingInterval-r10,

nonCriticalExtension			LoggedMeasurementConfiguration-v1080-IEs	OPTIONAL

}

LoggedMeasurementConfiguration-v1080-IEs ::= SEQUENCE {

lateNonCriticalExtension-r10	OCTET STRING						OPTIONAL,

nonCriticalExtension			LoggedMeasurementConfiguration-v1130-IEs	OPTIONAL

}

LoggedMeasurementConfiguration-v1130-IEs ::= SEQUENCE {

plmn-IdentityList-r11			PLMN-IdentityList3-r11		OPTIONAL,	-- Need OR

areaConfiguration-v1130			AreaConfiguration-v1130		OPTIONAL,	-- Need OR

nonCriticalExtension			LoggedMeasurementConfiguration-v1250-IEs	OPTIONAL

}

LoggedMeasurementConfiguration-v1250-IEs ::= SEQUENCE {

targetMBSFN-AreaList-r12	TargetMBSFN-AreaList-r12		OPTIONAL,	-- Need OP

nonCriticalExtension			LoggedMeasurementConfiguration-v1530-IEs					OPTIONAL

}

LoggedMeasurementConfiguration-v1530-IEs ::= SEQUENCE {

bt-NameList-r15					BT-NameList-r15					OPTIONAL, 	--Need OR

wlan-NameList-r15				WLAN-NameList-r15				OPTIONAL, 	--Need OR

nonCriticalExtension			SEQUENCE {}						OPTIONAL

}

TargetMBSFN-AreaList-r12 ::=			SEQUENCE (SIZE (0..maxMBSFN-Area)) OF TargetMBSFN-Area-r12

TargetMBSFN-Area-r12 ::=				SEQUENCE {

mbsfn-AreaId-r12					MBSFN-AreaId-r12		OPTIONAL,	-- Need OR

carrierFreq-r12						ARFCN-ValueEUTRA-r9,

...

}

-- ASN1STOP

Therefore, we have the following proposal.

RAN2 to define a common configuration message which is applicable to both RRC_IDLE and RRC_INACTIVE for Logged MDT. The parameters in the message such as loggingInterval, loggingduration and areaConfiguration are common for RRC_INACTIVE and RRC_IDLE.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The measurement logging for Logged MDT for UE in RRC_IDLE is based on the measurements required by the cell reselection purpose. For a UE in RRC_INACTIVE, the measurement requirements for cell reselection purpose are same as in RRC_IDLE.

Observation 2	The existing procedures of configuration and reporting for immediate MDT are not applicable to RRC_INACTIVE state.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Apply the Logged MDT configuration, logged measurements and reporting procedures to RRC_INACTIVE state.

Proposal 2	RAN2 to define a common configuration message which is applicable to both RRC_IDLE and RRC_INACTIVE for Logged MDT. The parameters in the message such as loggingInterval, loggingduration and areaConfiguration are common for RRC_INACTIVE and RRC_IDLE.

# References

TS 36.805, Evolved Universal Terrestrial Radio Access (E-UTRA); Study on minimization of drive-tests in next generation networks, 3GPP, V9.0.0, 2010-01

TS 37.320, Radio measurement collection for Minimization of Drive Tests (MDT), 3GPP, V15.0.0, 2018-07

TS 38.304, NR; User Equipment (UE) procedures in idle mode and in RRC Inactive state, 3GPP, v15.2.0, 2019-01

TS 36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification, 3GPP, v15.4.0, 2019-01