3GPP TSG-RAN WG2 #105	Tdoc R2-1900658

Athens, Greece, 25 Feb – 1 Mar 2019

Agenda Item:	11.12.4 Other use cases

Source:	Ericsson

Title:	RACH optimization in NR

Document for:	Discussion, Decision

# 1	Introduction

The new Rel-16 SI “Study on RAN-centric data collection and utilization for LTE and NR” was approved at the TSG RAN #80 meeting. Among the objectives, are the study of the use cases and benefits of RAN centric Data utilization, e.g. SON features including RACH optimization.

In this paper we discuss potential assistance information reported to the network to support RACH optimization in LTE and potential areas to be explored in NR considering new aspects such as RACH resource selection based on beam selection.

# 2	Discussion

RACH optimization has been introduced in LTE since Rel-9. The importance of optimizing RACH parameters that comes from the fact that depending on parameter settings and conditions when the procedure is performed, the UE may require multiple random-access attempts before it succeeds, which may be a source of latency that may affect user experience in procedures such transitions from IDLE to CONNECTED, which occur very often and handovers.

One of the components of the LTE solution is the usage of UE assistance information in the form of a RACH report, indicating to the network events that occurred during a successful RACH procedure or state variables known at the UE that would not have been visible to the network otherwise. In the following we discuss the LTE reporting mechanism and the exact information included in the RACH report.

## 2.1	Reporting mechanism

The existing mechanism to obtain a RACH report in LTE is the UE Information procedure in RRC (section 5.6.5 of TS 36.331), in the case where a RACH procedure was successful. UE may receive an UEInformationRequest containing a flag called rach-ReportReq set to ‘true’. Upon that the UE sets the content of the information to be included in the RACH report, to be later transmitted to the network in the UEInformationResponse message, as shown below:

Figure 1 UE Information procedure

The relevant part of that procedure for RACH reporting is summarized below, as described in RRC specifications:

***************************************************************************************************************************

#### 5.6.5.3	Reception of the UEInformationRequest message

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

1>	if rach-ReportReq is set to true, set the contents of the rach-Report in the UEInformationResponse message as follows:

2>	set the numberOfPreamblesSent to indicate the number of preambles sent by MAC for the last successfully completed random access procedure;

2>	if contention resolution was not successful as specified in TS 36.321 [6] for at least one of the transmitted preambles for the last successfully completed random access procedure:

3>	set the contentionDetected to true;

2>	else:

3>	set the contentionDetected to false;

. . .

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1;

***************************************************************************************************************************

In our view, a similar procedure is also needed in NR so that the UE only transmits a RACH report upon request from the network with dedicated signalling. The network also needs to know if a specific UE is capable of providing RACH reports. And, this procedure should also be used for other types of report assisting SON function as in LTE e.g. RLF report, etc. Hence, it seems reasonable to assume a request/response procedure like the RRC UE information procedure to enable the network to request a RACH report so that upon the reception of that message the UE transmits a RACH report, at least as baseline.

As in LTE, the UE Information procedure (e.g. UE Information Request / UE Information Response) is introduced in NR for RACH reporting.

One aspect that could perhaps be further discussed is that in NR, with a CU/DU split architecture, some of these parameters could possibly be optimized by the DU, instead of the CU (where RRC is terminated). However, there are alternatives to solve that with network signalling without creating complications to the UE in case these reports are transmitted in a different protocol terminated in the DU, which may be unreliable and not support same RRC functionalities.

## 2.1	Content of the RACH report in LTE

The exact content of the RACH report in LTE is reproduced below:

rach-Report-r9							SEQUENCE {

numberOfPreamblesSent-r9				NumberOfPreamblesSent-r11,

contentionDetected-r9					BOOLEAN

}																OPTIONAL,

As it is shown, it consists of the numberOfPreamblesSet (which is basically an integer) and the contentionDetected flag. These are related to two mechanisms in the random-access procedure where the procedure only succeeds after multiple attempts and are described in the following.

Number of preambles sent

This corresponds to the variable PREAMBLE_TRANSMISSION_COUNTER in the MAC specifications (TS 36.321).

In random access procedure in LTE, the UE sends a preamble and waits for a random-access response (RAR) during a pre-configured time window (RAR window). If the RAR does not come within that time, the UE adjusts some preamble transmission parameters (e.g. transmission power) and transmit it again (in what is called power ramping adjustment). If the procedure is successful, at the N-th transmission the preamble will be responded. The number N is what would be provided in the RACH report, so the network knows how many times the UE needed to ramp the power before the procedure was successful. As described in RRC, this field indicates the number of preambles sent by MAC for the last successfully completed random access procedure. Notice that larger this number is, potentially worse is the impact in user experience as it would take longer to e.g. access a target cell during a handover or it takes longer from the time an application request the data transfer until data may be transmitted/Received. This is shown in the figure below:

Unsuccessful contention resolution

This corresponds to the detection of contention in Contention-Based Random-Access (CBRA).

In a CBRA in LTE, a RACH preamble is randomly chosen by the UE, which may result in more than one UE simultaneously transmitting the same signature, leading to a need for a subsequent contention resolution process. If multiple UEs select the same preamble, the network possibly sends a similar Random-Access Response (RAR) message to both UEs indicating the same UL grant for MSG.3 transmission and the same temporary C-RNTI (TC-CRNTI).

Upon transmission of MSG.3, a UE without a C-RNTI allocated by target (E.g. a UE coming from RRC_IDLE) would transmit MSG.3 in the UL resources indicated in RAR and addressing the provided TC-RNTI by RAR. Upon that transmission the UE starts a contention resolution timer. In case of preamble collision that may again lead to interference and eventually colliding UEs would reach a maximum number of allowed HARQ retransmissions until they try again to perform random access. However, for some UEs the network may succeed in decoding MSG.3 and, to solve potential contention, the network sends a MSG.4 addressing the TC-RNTI and including in the MAC payload the UE identity transmitted in MSG.3 (e.g. a resume identifier). If the UE detects MSG.4 addressing its TC-RNTI and with the UE identifier matching the one transmitted in MSG.3, the UE considers the contention successful. Otherwise, if the contention resolution timer expires or if the UE identity in MSG.4 does not match the UE identity transmitted in MSG.3, the UE considers contention not successful.

A UE with a C-RNTI allocated by target (E.g. a UE being handed over) would transmit MSG.3 in the UL resources indicated in RAR and addressing the allocated C-RNTI. Upon that transmission the UE also starts a contention resolution timer. If for that UE MSG.4 addresses the allocated C-RNTI, the contention resolution is considered successful. If the contention resolution timer expires before that, contention is considered unsuccessful.

Hence, in summary, contention is detected as not successful at the UE in the following cases:

- Expiry of the contention resolution timer;

- After transmitting a MSG.3 using a C-RNTI assigned by target cell (e.g. in handover or when UE is in RRC_CONNECTED), UE detects a MSG.4 not addressing its C-RNTI;

- After transmitting a MSG.3 using a TC-RNTI assigned to it in the RAR, UE detects a MSG.4 addressing the same TC-RNTI but the UE Identity in the MSG.4 payload does not match the UE’s identity transmitted on MSG.3;

As in the case of expiry of the RAR time window, upon the detection of a non-successful contention resolution the UE shall perform random access once more i.e. RACH resource selection, preamble retransmission, etc.

Notice also that the field “numberOfPreamblesSent” included in the RACH report includes both retransmissions with power ramping (e.g. due to non-received RAR) and retransmissions due to non-successful contention resolution.

The field numberOfPreamblesSent includes both retransmissions with power ramping (e.g. due to non-received RAR) and retransmissions due to non-successful contention resolution.

## 2.1	Content of the RACH report in NR

In our view, the discussion on the content of the RACH report in NR should have as the starting point the content of the RACH report in LTE. Hence, one should start by analyzing the scenarios how preamble power retransmission and non-successful contention resolution are handled in NR.

Preamble retransmissions in NR (coverage)

In NR, the UE shall select a preamble and transmit it in a selected PRACH resource, as in LTE.

However, as in NR a cell may transmit reference signals and sync signals like Synchronization Signal and PBCH Block (SSBs) and CSI-RSs in multiple downlink beams that are measured by the UE, the preamble transmission step in NR during random access is preceeded by a beam selection step where the UE first needs to select a beam of the target cell (i.e. the cell the UE is performing random-access) and, thanks to a mapping provided to the UE (either in SIB1 or in dedicated signaling e.g. in handovers), know which preambles it may chose and which PRACH resources shall be used.

If a given transmission is not responded (i.e. UE does not receive a RAR within configured time window) there are two alternatives:

- Select the same beam, mapped to the same RACH resource(s), perform power ramping and retransmit a preamble;

- Select a different beam, mapped to the same RACH resource(s), perform power ramping and retransmit a preamble;

One example is illustrated below, where in the first round the UE in t0 selects the beam indicated by SSB-63, transmits the preamble in the mapped RACH resource for that beam and it does not receive a RAR. In the second round the UE selects again the beam indicated by SSB-63, ramps the power according to configuration, transmits the preamble in the mapped RACH resource for that beam and it does not receive a RAR. In the third round, the UE selects a new beam indicated by SSB-64, remains with the previous tranmission power, transmits the preamble in the mapped RACH resource for that beam and finally receives a RAR.

Preamble retransmissions in NR (unsuccessful contention resolution)

Preamble retransmissions may also be triggered in the case of unsuccessfull contention resolution. The detection of unsuccessfull contention resolution is similar to LTE i.e. at every preamble transmission (or retransmission), regardless of wchih beam has been selected, the UE considers contention resolution non-successful if upon transmitting MSG.3 and starting the contention resolution timer, the timer expires before the UE is able to decode a MSG.4, or, upon receiving MSG.4 the UE detects that the UE identity is not the one matching the one transmitted in MSG.4.

As in the case of preamble retransmission due to a non-received RAR, each time the UE detects an unresolve contention the UE may either select the same beam or a different beam.

An example is illustrated below, where in the first round the UE in t0 selects the beam indicated by SSB-63, transmits the preamble in the mapped RACH resource for that beam, receives a RAR, transmits MSG.3 and, upon reception of MSG.4 detects non-successful contention. Then, in the next transmission the UE selects selects a new beam indicated by SSB-64 to continue random-access.

RACH report considers beam selection at every preamble transmission attempt.

Fallback of contention-free and contention-based random-access

In LTE, the UE may be configured with contention-free random access in some use cases, e.g., in handovers, in an RRCReconfiguration message containing a reconfigurationWithSync. Upon the reception of that message the UE starts a failure timer (T304), performs CFRA towards the indicated target cell and, if that fails e.g. RAR is not received within the RAR time window, the UE can continue with preamble power ramping until it suceeds or reach the maximum number of attempts. If that finally suceeds, the network is aware this was a CFRA as the preamble has been allocated to that particular UE.

In NR, where beam selection is needed before preamble transmission, the UE needs to be configured with a set of CFRA resources that maps to a set of downlink beams that the network is transmitted. That set does not have to be the whole set of beams, to avoid an inneficient use of resources. Hence, network could provide CFRA resource only for a subset of beams (e.g. the beams provided to source in measurement reports with good quality and forwared to target during HO preparation). Hence, before accessing the target with random access the UE needs to select a beam. If the UE selects a beam with CFRA resources it sends the preamble and, in case the UE does not receive a RAR, the UE performs beam selection once more. And, differently from LTE, the UE has the alternative to select a beam without CFRA resources associated and, in that case, the UE is allowed to use CBRA. If that also fails, the UE may again select a beam with CFRA resources, as long as the failure timer is still running.

After suceeding, that UE would finally be able to transmit a handover complete message. At that point the network is aware this UE was provided a CFRA resource, but has possibly suceeded with CBRA, or, the UE may finally suceed using a CFRA even though it was not the first time.

Since the network partitions RACH resources per beam, both for CFRA and CBRA, it is relevant to discuss the content of RACH report taking into account this fallback procedure where the UE is allowed to select a beam without CFRA resource and perform CBRA in case it does not receive a RAR.

RACH report considers the fallback between CFRA and CBRA upon beam selection at every preamble transmission attempt.

# Conclusion

Based on the discussion in the previous sections we propose the following:

- As in LTE, the UE Information procedure (e.g. UE Information Request / UE Information Response) is introduced in NR for RACH reporting.

RACH report considers beam selection at every preamble transmission attempt.

RACH report considers the fallback between CFRA and CBRA upon beam selection at every preamble transmission attempt.