3GPP TSG-RAN WG2 Meeting #105 	                              R2-1900667

Athens, Greece, 25th Feb– 1st Mar 2019

Source: 			ZTE Corporation, Sanechips

Title: 	Clarification on intra-NR handover capabilities

Agenda item:		10.4.4.2

Document for: 	Discussion and Decision

- Introduction

After checking the latest TS38.331, we found that all intra-NR handover related capabilities are defined in UE-NR-Capability, not in UE-MRDC-Capability, for PSCell change in EN-DC, it is not clear which capabilities should be used, in this contribution, we discussed this issue and provided potential solutions.

- Discussion

In the latest version of 38.331, for intra-NR handover, the related handover and measurement capabilities are defined under MeasAndMobParameters element, the relevant capabilities are highlighted as below:

MeasAndMobParameters information element

-- ASN1START

-- TAG-MEASANDMOBPARAMETERS-START

MeasAndMobParameters ::=                    SEQUENCE {

measAndMobParametersCommon              MeasAndMobParametersCommon              OPTIONAL,

measAndMobParametersXDD-Diff                MeasAndMobParametersXDD-Diff        OPTIONAL,

measAndMobParametersFRX-Diff                MeasAndMobParametersFRX-Diff        OPTIONAL

}

MeasAndMobParametersCommon ::=          SEQUENCE {

supportedGapPattern                 BIT STRING (SIZE (22))          OPTIONAL,

ssb-RLM                             ENUMERATED {supported}          OPTIONAL,

ssb-AndCSI-RS-RLM                   ENUMERATED {supported}          OPTIONAL,

...,

[[

eventB-MeasAndReport        ENUMERATED {supported}      OPTIONAL,

handoverFDD-TDD             ENUMERATED {supported}      OPTIONAL,

eutra-CGI-Reporting         ENUMERATED {supported}      OPTIONAL,

nr-CGI-Reporting            ENUMERATED {supported}      OPTIONAL

]],

[[

independentGapConfig        ENUMERATED {supported}                  OPTIONAL,

periodicEUTRA-MeasAndReport ENUMERATED {supported}                  OPTIONAL,

handoverFR1-FR2             ENUMERATED {supported}                  OPTIONAL,

maxNumberCSI-RS-RRM-RS-SINR ENUMERATED {n4, n8, n16, n32, n64, n96} OPTIONAL

]]

}

MeasAndMobParametersXDD-Diff ::=            SEQUENCE {

intraAndInterF-MeasAndReport        ENUMERATED {supported}  OPTIONAL,

eventA-MeasAndReport                ENUMERATED {supported}  OPTIONAL,

...,

[[

handoverInterF              ENUMERATED {supported}  OPTIONAL,

handoverLTE             ENUMERATED {supported}  OPTIONAL,

handover-eLTE               ENUMERATED {supported}  OPTIONAL

]]

}

MeasAndMobParametersFRX-Diff ::=            SEQUENCE {

ss-SINR-Meas                        ENUMERATED {supported}      OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithSSB        ENUMERATED {supported}      OPTIONAL,

csi-RSRP-AndRSRQ-MeasWithoutSSB     ENUMERATED {supported}      OPTIONAL,

csi-SINR-Meas                       ENUMERATED {supported}      OPTIONAL,

csi-RS-RLM                          ENUMERATED {supported}      OPTIONAL,

...,

[[

handoverInterF              ENUMERATED {supported}  OPTIONAL,

handoverLTE                 ENUMERATED {supported}  OPTIONAL,

handover-eLTE               ENUMERATED {supported}  OPTIONAL

]],

[[

maxNumberResource-CSI-RS-RLM    ENUMERATED {n2, n4, n6, n8}     OPTIONAL

]]

}

-- TAG-MEASANDMOBPARAMETERS-STOP

-- ASN1STOP

However, these fields are only indicated in UE-NR-Capability branch, not in UE-MRDC-Capability.

Observation 1 	So far, the intra-NR handover related capabilities are only defined in UE-NR-Capability branch, not in UE-MRDC-Capability branch.

In our understanding, for sure these fields can be used to indicate the measurement and handover capabilities for NR SA operations, however, for EN-DC UE, the PSCell change is also a kind of intra-NR handover, and SN can configure intra-freq or inter-freq measurements to UE for triggering PSCell change. So from network perspective, it is unclear whether these capabilities are also applicable to PSCell change when UE is configured with EN-DC.

Observation 2 	From network perspective, it is unclear whether those capabilities are also applicable to PCell change in case of EN-DC.

In our opinion, there are two alternative solutions to solve this issue:

- Solution1: Clarify in spec that these fields are applicable to both NR SA and PSCell change in EN-DC;

- Solution2: Clarify in spec that these fields are only applicable to NR SA, and introduce new fields in UE-MRDC-Capability for EN-DC.

Compare above two solutions, solution2 is more or less considered as a functional NBC change. Since EN-DC is already frozen, we slightly prefer to adopt solution1 in RAN2, and the corresponding CR to 38.306 is provided in [1].

Proposal 1:  The current intra-NR measurement and handover related capabilities are applicable to both NR SA handover and PSCell change in EN-DC. And adopt the CR in [1].

- Conclusion and proposals

RAN2 is kindly asked to discuss and adopt the following proposal:

Observation 1 	So far, the intra-NR handover related capabilities are only defined in UE-NR-Capability branch, not in UE-MRDC-Capability branch.

Observation 2 	From network perspective, it is unclear whether those capabilities are also applicable to PCell change in case of EN-DC.

Proposal 1:  The current intra-NR measurement and handover related capabilities are applicable to both NR SA handover and PSCell change in EN-DC. And adopt the CR in [1].

- Reference

[1] R2-1900668 CR to clarify intra-NR handover capabilities.  ZTE Corporation, Sanechips.  CR  TS38.331 f.4.0