3GPP TSG-RAN WG2#105	R2-1900697

Athens, Greece, 25th February – 1st March 2019

Agenda Item:	11.10.4

Source:	Ericsson

Title:	Configuring or resuming SCG in RRCConnectionResume in (NG)EN-DC

Document for:	Discussion, Decision

# Introduction

The Rel-16 WI LTE and NR Carrier aggregation and dual connectivity enhancements include the following objective:

- Efficient and low latency serving cell configuration/activation/setup: Minimizing signalling overhead and latency needed for initial cell setup, additional cell setup and additional cell activation for data transmission. [RAN2, RAN1, RAN4, RAN3]

  - This objective applies to MR-DC, NR-NR DC and CA

  - The objective should consider enhancements when starting from IDLE, INACTIVE mode and CONNECTED mode

This contribution discusses the required modifications to the specifications in order to support resuming a suspended SCG configuration in (NG)EN-DC.

# Discussion

In E-UTRA Rel-15, if a UE is configured with (NG)EN-DC when it is suspended to RRC_INACTIVE or RRC_IDLE with suspended RRC connection, the UE will:

Upon entering RRC_INACTIVE, the UE shall:

…

2>	in the UE Inactive AS Context, store the received rrc-InactiveConfig, all current parameters configured with RRCConnectionReconfiguration or RRCConnectionResume, the current KeNB and KRRCint keys, the ROHC state, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell;

1>	if leaving RRC_CONNECTED was triggered by suspension of the RRC:

…

2>	store the UE AS Context including the current RRC configuration, the current security context, the PDCP state including ROHC state, C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell;

Although this procedure text doesn’t explicitly mention the SCG configurations, it also doesn’t exclude them.

However, for EN-DC, if a UE initiates the resumption of a suspended RRC connection, it will:

Except for NB-IoT, upon initiating the procedure, if connected to EPC or 5GC, the UE shall:

1>	if the UE is resuming an RRC connection from a suspended RRC connection or from RRC_INACTIVE:

2>	if the UE is resuming an RRC connection from a suspended RRC connection:

3>	if the UE was configured with EN-DC:

4>	perform EN-DC release, as specified in TS 38.331 [82, 5.3.5.10];

Since NGEN-DC will be based on EN-DC, it can be assumed that the UE will release any SCG and/or configuration when it initiates resumption from RRC_INACTIVE as it was for EN-DC.

If the UE is configured with an SCG and/or SN terminated bearers when it is suspended to RRC_INACTIVE, the UE will store these configurations while it remains in RRC_INACTIVE

During the email discussion [104#64][NR] Running 38.331 CR for Late drop (Ericsson), the consensus was that SCG configuration during RRC Resume in NR should be postponed to Rel-16. Similarly for (NG)EN-DC, there has not been any agreement on how to treat (NG)EN-DC configurations during resume.

However, there are certain scenarios where a UE would benefit from configurations of the (NG)EN-DC already in RRC Connection Resume. Given that NR carriers may provide significantly larger bandwidth than LTE carriers, a UE would benefit from resuming the SCG as quickly as possible However, if the UE cannot be configured with the SCG configurations in RRC Resume, it will have to wait until entering RRC_CONNECTED before receiving the SCG configurations.

If the UE has frequent transitions from RRC_INACTIVE or RRC_IDLE with suspended RRC connection to RRC_CONNECTED, the network would have to wait for the UE to enter RRC_CONNECTED before it can configure the UE to use NR capacity.

Enable configuration of NR SCG in for (NG)EN-DC in RRCConnectionResume

We have provided a draft CR showing the implementation of configuration of SCG in RRCConnectionResume [2].

In addition, in case the UE is configured in RRCConnectionResume with the same SCG configurations as it used before leaving RRC_CONNECTED, the signalling could be greatly reduced if delta signalling is used. However, since the SCG configurations are released during initiation of resume, the network needs to be instructed to refrain from this step.

Enable delta signalling of NR SCG for (NG)EN-DC in RRCConnectionResume

# Conclusion

In section 2 we made the following observations:

Observation 1	If the UE is configured with an SCG and/or SN terminated bearers when it is suspended to RRC_INACTIVE, the UE will store these configurations while it remains in RRC_INACTIVE

Observation 2	If the UE has frequent transitions from RRC_INACTIVE or RRC_IDLE with suspended RRC connection to RRC_CONNECTED, the network would have to wait for the UE to enter RRC_CONNECTED before it can configure the UE to use NR capacity.

Based on the discussion in section 2 we propose the following:

Proposal 1	Enable configuration of NR SCG in for (NG)EN-DC in RRCConnectionResume

Proposal 2	Enable delta signalling of NR SCG for (NG)EN-DC in RRCConnectionResume

# References

RP-182076, WID on Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE_NR_DC_CA_enh-Core), Ericsson, Nokia, Nokia Shanghai Bell, Huawei, 3GPP TSG RAN#81, 10-13 September 2018, Gold Coast, Australia

R2-1900698, Configuration of SCG during RRC Resume in NGEN-DC, Ericsson, 3GPP WG2#105, Athens, Greece, 25th February – 1st March 2019