3GPP TSG-RAN2 Meeting #105	R2-1900816

# Athens, Greece, 25th Feb 2019 - 1st Mar 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.305 | CR | 0008 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.305 on use of positioning measurement gaps for subframe and slot timing detection towards E-UTRA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN4 concluded that the positioning measurement gap request procedure can be used without measPRS-Offset to support detection of subframe and slot timing of the target E-UTRA system. The current specification of the request procedure does not allow for this use. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the description of the Location Measurement Indication procedure, add the use case for subframe and slot timing acquisition. In the description of the procedural steps, references to “location measurements” are replaced with “location procedures” since the timing acquisition is not a measurement. Impact analysis Impacted functionality: Measurement gap request for positioning Inter-operability: - If the network is implemented according to the CR and the UE is not, the UE may be unable to take RSTD measurements due to not having the timing of the target E-UTRA system. - If the UE is implemented according to the CR and the network is not, the network may not provide the needed measurement gaps for timing acquisition, causing the OTDOA positioning operation to fail. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE cannot request measurement gaps to acquire E-UTRA system timing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.331 CR 0868 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 7.4.1.1	Location Measurement Indication

The location measurement indication procedure is used by the UE to request measurement gaps for OTDOA RSTD measurements, or for subframe and slot timing detection towards E-UTRA preparatory to requesting measurement gaps for OTDOA RSTD measurements.

Figure 7.4.1.1-1: Location measurement indication procedure

Precondition:	The UE served by an gNB has received a LPP message from an LMF requesting inter-RAT RSTD measurements for OTDOA positioning.

1.	If the UE requires measurement gaps for performing the requested location measurements while measurement gaps are either not configured or not sufficient, or if the UE needs gaps to acquire the subframe and slot timing of the target E-UTRA system before requesting measurement gaps for the inter-RAT RSTD measurements, the UE sends an RRC Location Measurement Indication message to the serving gNB. The message indicates that the UE is going to start location measurements, or that the UE is going to acquire subframe and slot timing of the target system, and includes information required for the gNB to configure the appropriate measurement gaps. When the gNB has configured the required measurement gaps the UE performs the location procedures (measurements or timing acquisition).

2.	When the UE has completed the location procedures which required measurement gaps, the UE sends another RRC Location Measurement Indication message to the serving gNB. The message indicates that the UE has completed the location procedures.