3GPP TSG-RAN WG2 Meeting #105	R2-1900823

Athens, Greece, 25 February-1 March 2019	(resubmission of R2-1816824)

Agenda Item:	11.5.4

Source: 	MediaTek Inc.

Title:  	Analysis of UE capability size

Document for:	Discussion and decision

# Introduction

This document analyses the size of the NR UE capability structure.

# Discussion

## General overview of the capability structure

The UE-NR-Capability structure is as shown below:

UE-NR-Capability ::= SEQUENCE {

accessStratumRelease			AccessStratumRelease,

pdcp-Parameters					PDCP-Parameters,

rlc-Parameters					RLC-Parameters						OPTIONAL,

mac-Parameters					MAC-Parameters						OPTIONAL,

phy-Parameters					Phy-Parameters,

rf-Parameters					RF-Parameters,

measAndMobParameters				MeasAndMobParameters						OPTIONAL,

fdd-Add-UE-NR-Capabilities		UE-NR-CapabilityAddXDD-Mode			OPTIONAL,

tdd-Add-UE-NR-Capabilities		UE-NR-CapabilityAddXDD-Mode			OPTIONAL,

fr1-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFRX-Mode			OPTIONAL,

fr2-Add-UE-NR-Capabilities		UE-NR-CapabilityAddFRX-Mode			OPTIONAL,

featureSets						FeatureSets 						OPTIONAL,

featureSetCombinations			SEQUENCE (SIZE (1..maxFeatureSetCombinations)) OF FeatureSetCombination			OPTIONAL,

lateNonCriticalExtension		OCTET STRING						OPTIONAL,

nonCriticalExtension			UE-NR-Capability-15xy							OPTIONAL

}

The size of this structure is dominated by two fields: rf-Parameters (including the band combination list) and featureSetCombinations (the list of feature set combinations per band).  The other fields have essentially flat structures or only a single layer of lists.

Observation 1: The size of the capability is dominated by rf-Parameters and featureSetCombinations.

In the subsequent sections, we investigate the size of these two fields.

## Size of rf-Parameters

In the RF-Parameters structure, the size is dominated by the band combination list:

RF-Parameters ::= SEQUENCE {

supportedBandListNR				SEQUENCE (SIZE (1..maxBands)) OF BandNR,

supportedBandCombinationList		BandCombinationList						OPTIONAL,

appliedFreqBandListFilter			FreqBandList							OPTIONAL

}

BandCombinationList ::=			SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination

BandCombination ::= 				SEQUENCE {

bandList								SEQUENCE (SIZE (1..maxSimultaneousBands)) OF BandParameters,

featureSetCombination				FeatureSetCombinationId,

ca-ParametersEUTRA					CA-ParametersEUTRA					OPTIONAL,

ca-ParametersNR						CA-ParametersNR						OPTIONAL,

mrdc-Parameters						MRDC-Parameters						OPTIONAL,

supportedBandwidthCombinationSet		BIT STRING (SIZE (1..32))			OPTIONAL,

powerClass-v15xy					ENUMERATED {pc2}					OPTIONAL

}

BandParameters ::=					CHOICE {

eutra							SEQUENCE {

bandEUTRA						FreqBandIndicatorEUTRA,

ca-BandwidthClassDL-EUTRA		CA-BandwidthClassEUTRA				OPTIONAL,

ca-BandwidthClassUL-EUTRA		CA-BandwidthClassEUTRA				OPTIONAL

},

nr								SEQUENCE {

bandNR							FreqBandIndicatorNR,

ca-BandwidthClassDL-NR			CA-BandwidthClassNR					OPTIONAL,

ca-BandwidthClassUL-NR			CA-BandwidthClassNR					OPTIONAL

}

}

Focussing only on the BandCombinationList and on the bandList fields inside it, the size is:

(#_of_band_combinations) * (Length_indicator + (#_of_bands * (Choice_bit + 2*Optionality_bit + Band_indicator + 2*Bandwidth_class)))

The length indicator (1..maxSimultaneousBands) is 5 bits; the band indicator is 10 bits and the NR bandwidth class indicators are 6 bits each (5 bits plus the extension flag; the LTE bandwidth class indicators are 4 bits).  In a hypothetical deployment with 1024 band combinations and up to 4 bands per band combination, then, the size of this list would be (1024*(5+(4*(6+10+8)))) = 101 Kbits = 12.625 KB if all bands were NR, or (1024*(5+(4*(4+10+8)))) = 93 kbits = 11.625 KB if all bands were LTE (in both cases assuming the bandwidth class indicator is present for all bands), so the size for a deployment of mixed LTE and NR bands would fall somewhere in this range.  This calculation neglects the contribution of the fields featureSetCombination, ca-ParametersEUTRA/NR, mrdc-Parameters, supportedBandwidthCombinationSet, and powerClass-v15xy, which would add several kilobits.

Observation 2: The size of the BandCombinationList structure alone can exceed the 9 KB PDCP limit.

## Size of featureSetCombinations

The feature set combination list is now decoupled from the number of band combinations; instead, its size depends on the total number of feature set combinations and the number of feature sets per band.  The structure is as follows:

featureSetCombinations			SEQUENCE (SIZE (1..maxFeatureSetCombinations)) OF FeatureSetCombination			OPTIONAL,

FeatureSetCombination ::=	SEQUENCE (SIZE (1..maxSimultaneousBands)) OF FeatureSetsPerBand

FeatureSetsPerBand ::= 		SEQUENCE (SIZE (1..maxFeatureSetsPerBand)) OF FeatureSet

FeatureSet ::=					CHOICE {

eutra							SEQUENCE {

downlinkSetEUTRA				FeatureSetEUTRA-DownlinkId,

uplinkSetEUTRA					FeatureSetEUTRA-UplinkId

},

nr								SEQUENCE {

downlinkSetNR					FeatureSetDownlinkId,

uplinkSetNR						FeatureSetUplinkId

}

}

The total size of the structure is

(#_of_FSCs) * (Length_indicator_1 + #_of_bands * (Length_indicator_2 + #_of_FSs_per_band * (Choice_bit + 2*Feature_set_ID)))

Here the first length indicator is (1..maxSimultaneousBands) with a size of 5 bits, and the second is (1..maxFeatureSetsPerBand) with a size of 7 bits (maxFeatureSetsPerBand=128).  Each feature set ID is 10 bits in NR or 8 bits in LTE.

There could be high variation in the number of feature sets.  In some cases feature sets might be relatively consistent across different band combinations, leading to a relatively small list of FSCs; in others there might be a large amount of variation between band combinations leading to a large list.  The theoretical maximum of the structure would have 1024 FSCs and 128 FSs per band, so with 4 bands per combination, the total size would be 1024 * (5 + 4 * (7 + 128 * (1 + 2*10))) = 10.53 Mbits = 1.32 MB.  We assume this is not a reasonable scenario, however; more modest assumptions cause large variation in the total size, as shown in Table 1.  (Four bands per band combination is assumed in all cases.)

Table 1: Size of feature set combination list

| Number of FSCs | Number of FSs per band | Total size |
| --- | --- | --- |
| 64 | 2 | 1.57 KB |
| 256 | 8 | 22 KB |
| 1024 | 128 | 1.32 MB |

Observation 3: The size of the feature set combination list is highly variable depending on assumptions, but in some cases can far exceed the size of the RF-Parameters structure.

Observation 4: The initial capability size estimate of “several tens of kilo-octets” given in [2] appears to be valid.

# Conclusion

This document made the following observations regarding the size of the NR UE capability:

Observation 1: The size of the capability is dominated by rf-Parameters and featureSetCombinations.

Observation 2: The size of the BandCombinationList structure alone can exceed the 9 KB PDCP limit.

Observation 3: The size of the feature set combination list is highly variable depending on assumptions, but in some cases can far exceed the size of the RF-Parameters structure.

Observation 4: The initial capability size estimate of “several tens of kilo-octets” given in [2] appears to be valid.

It is proposed to capture the conclusions in the TR.

# References

- RP-181146, “UE Capability ID Motivation”, MediaTek Inc., RAN#80

- R2-1810966, “Reply LS on optimisation of UE capability signalling”, LS from RAN2 to SA2, RAN2 ad hoc #1807

- RP-181459, “Study on optimisations on UE radio capability signalling – NR/E-UTRA Aspects”, MediaTek Inc. et al., RAN#80