3GPP TSG-RAN WG2 Meeting #105                                	R2-1900825

Athens, GREECE, 25th February – 1st March 2019

Source: 	Asia Pacific Telecom, MTI

Title: 	Discussion on the BFR timer in NTN

Agenda item:	11.6.3.1

Document for:	Discussion and Decision

# Introduction

In RAN2#103bis, the following agreement has been agreed.

Agreements in RAN2#103bis: UP Impacts to study - DRX - HARQ - Random access response - RLC/PDCP reordering (e.g. timers and SN space) - SDAP => no impact Agreements in RAN2#104: - [103bis#xx][NR – NTN ] - Impacts on user plane timers (Nomor) - Intended outcome: - Identify timers impacted by RTD and how to handle them - Outcome: TP capturing the timers - Deadline: Thursday 2019-2-15

In this contribution, we discuss the impact of round-trip time (RTT) on the BFR timer for NTN.

# Discussion

In TS 38.811, RTT for Geostationary Earth Orbit (GEO) satellites is given in the following table.

Table 5.3.2.1-1: Propagation delays for GEO satellite at 35786 km

|  | GEO at 35786 km |  |  |
| --- | --- | --- | --- |
| Elevation angle | Path | D (km) | Time (ms) |
| UE :10° | satellite - UE | 40586 | 135.286 |
| GW : 5° | satellite - gateway | 41126.6 | 137.088 |
| 90° | satellite - UE | 35786 | 119.286 |
| Bent Pipe satellite |  |  |  |
| One way delay | Gateway-satellite_UE | 81712.6 | 272.375 |
| Round trip Time | Twice | 163425.3 | 544.751 |
| Regenerative Satellite |  |  |  |
| One way delay | Satellite -UE | 40586 | 135.286 |
| Round Trip Time | Satellite-UE-Satellite | 81172 | 270.572 |

As shown, up to 545ms of RTT needs to be supported by NR NTN. As a result, many timers have been reviewed and discussed by companies among previous meetings. However, one discussion on the BFR timer is missing.

In TS 38.331, details on the BFR timer can be found in the BFR configuration IE:

BeamFailureRecoveryConfig ::=       SEQUENCE {

[…]

beamFailureRecoveryTimer            ENUMERATED {ms10, ms20, ms40, ms60, ms80, ms100, ms150, ms200}          OPTIONAL,   -- Need M

[…]

}

and its functional description can be found as below:

beamFailureRecoveryTimer Timer for beam failure recovery timer. Upon expiration of the timer the UE does not use CFRA for BFR. Value in ms. ms10 corresponds to 10ms, ms20 to 20ms, and so on.

In Rel. 15, we have found that the maximum value of the BFR timer is 200ms, which is less than the RTT for GEO satellites as 545ms. As a result, the current length of the BFR timer is insufficient for NR NTN, and this leads to a malfunction on RA procedure, i.e., the BFR timer will always expire before UE receives any response from gNB and upon expiration of the timer the UE cannot use CFRA for BFR. An illustration is given below.

Fig.1 an illustration of CBRA for the BFR timer in NTN

Current value range of RRC parameter beamFailureRecoveryTimer is shorter than RTT time for NTN application.

Thus, we then propose

Extend the BFR timer to cover RTT in NTN.

# Conclusion

In this contribution, we have the following observation

Observation 1	A modification of the BFR timer is required to support NTN.

Based on the observation, we then propose

Proposal 1	Extend the BFR timer to cover RTT in NTN.